RTL Interview Handbook

Simple English  •  RTL vocabulary  •  D → P → W → E  •  Tested code

For 3+ years RTL design interviews

220 questions. Each question appears once.

Contents

Part 0 — HOW TO USE THIS HANDBOOK

Part 1 — RTL VOCABULARY

Part 2 — CORE RTL FUNDAMENTALS

Part 3 — READY/VALID HANDSHAKE AND FSM

Part 4 — FIFO, PIPELINE, PERFORMANCE

Part 5 — AXI-STREAM AND AXI

Part 6 — AVALON-ST PROJECT (MASTER → FIFO → SLAVE)

Part 7 — CLOCK DOMAIN CROSSING (CDC) AND RESET CROSSING

Part 8 — RTL LINT

Part 9 — STATIC TIMING ANALYSIS (STA)

Part 10 — XDC CONSTRAINTS (FPGA / VIVADO)

Part 11 — VERIFICATION, TESTBENCH, ASSERTIONS AND DEBUG

Part 12 — RTL CODING PRACTICE (TESTED SOLUTIONS)

Part 13 — DEBUG SCENARIOS (“WHAT WOULD YOU DO IF…?”)

Part 14 — RTL DESIGN THINKING

Part 15 — REVISION PAGES

Part 16 — PROGRESS CHECKLIST

# PART 0 — HOW TO USE THIS HANDBOOK

This handbook is for RTL design interviews at the 3+ years level. It combines your two earlier documents into one format. Every question appears only once. If two topics overlap, the second one points to the first (you will see “Answered in Q…”).

The style is simple: short sentences, easy grammar, and the correct RTL words. You should be able to remember each topic and say it naturally.

## The D → P → W → E method

| Letter | Meaning | What you say |
| --- | --- | --- |
| D | Definition | One simple sentence. What is it? |
| P | Purpose | Why do we use it? What problem does it solve? |
| W | Working | How does it work? Step by step, with RTL words. |
| E | Example | One small signal example, code, or a real design use. |

## What each question contains

- Question number (for example Q7.12) so you can track your progress in the checklist at the end.

- D, P, W, E — the four-step answer.

- Code — small Verilog / SystemVerilog examples where they help.

- Interview line (orange box) — one sentence you can say out loud.

- Follow-up questions to expect — what a 3+ years interviewer usually asks next.

- MUST KNOW tag — ask yourself these first. ADVANCED tag — for strong 3+ years answers.

- Coloured boxes: Note, Common mistake, Interview trap, Tip.

## Rules for speaking in the interview

- Explain first, code later. Say the architecture and behavior. Show code only if asked.

- Short first, deep later. Give the 20-second answer. Go deeper only if the interviewer wants more.

- Use the RTL words. Say “clock domain”, not “different clock area”. Say “backpressure”, not “stop signal”.

- Say the reason. Not only what you did, but why you did it.

- Say the risk. Good engineers mention what can go wrong: metastability, overflow, latch, timing violation.

- If you do not know, think aloud. Say: “I have not used this directly, but I would approach it like this…”

- Use your own project. Every concept sounds stronger when you attach it to a real signal or test from your design.

## 14-day study plan

| Day | Study | Practice |
| --- | --- | --- |
| 1 | Part 1 vocabulary, Part 2 fundamentals | Say 10 terms aloud without reading |
| 2 | Part 3 handshake and FSM | Draw a valid/ready waveform; write an FSM from memory |
| 3 | Part 4 FIFO, pipeline, performance | Size a FIFO depth using the worked example |
| 4 | Part 5 AXI-Stream and AXI | Explain TVALID / TREADY / TLAST in 60 seconds |
| 5 | Part 6 your Avalon-ST project | Give the one-minute project pitch |
| 6–7 | Part 7 CDC | Write the 2-FF, toggle and async FIFO from memory |
| 8 | Part 8 lint | Fix 5 mistake → corrected examples |
| 9 | Part 9 STA | Solve the setup and hold numeric examples |
| 10 | Part 10 XDC | Write a small constraint file |
| 11 | Part 11 verification and debug | Write one assertion and one scoreboard |
| 12–13 | Part 12 coding practice | Code 6 problems on paper without looking |
| 14 | Parts 13, 14, 15 | Mock interview using the follow-up bank |

## Interview round map — what to expect

| Round | Usually asked | Best parts to revise |
| --- | --- | --- |
| Manager / project | Explain your project, your role, a bug you fixed, trade-offs | Parts 6, 13, 14 |
| Technical 1 | Fundamentals, FSM, FIFO, handshake, small coding problem | Parts 2, 3, 4, 12 |
| Technical 2 | CDC, async FIFO, timing (STA), constraints, lint | Parts 7, 8, 9, 10 |
| Design / whiteboard | Design a block: arbiter, FIFO, width converter, packet engine | Parts 12, 14 |
| Final / lead | Debug scenarios, verification thinking, ownership | Parts 11, 13, 14 |

Note:  Code in Part 12 (and the FSM and skid-buffer examples elsewhere) was compiled and run in Icarus Verilog with small self-checking testbenches. Snippets in other parts are teaching examples: use them as patterns, then run them in your own flow. Lint rule numbers (W336, W18, …) are SpyGlass-style IDs; other tools use different IDs for the same ideas.

# PART 1 — RTL VOCABULARY

Learn these words first. Each one has a simple meaning. Use them in your answers.

| Term | Simple meaning |
| --- | --- |
| RTL | Register Transfer Level. Describes how data moves between registers and what logic sits in between. |
| Combinational logic | Output depends only on current inputs. No storage. Example: mux, adder. |
| Sequential logic | Output depends on current inputs and stored state. Updates on a clock edge. Example: flip-flop, counter. |
| Clock domain | All the logic driven by one clock. |
| State / next state | State is the FSM’s current condition. Next state is what it becomes on the next clock edge. |
| Handshake | Two sides agree before data moves. Example: VALID and READY. |
| Backpressure | The receiver tells the sender to wait because it cannot accept data now. |
| Beat / packet | A beat is one transfer on the bus. A packet is one or more beats. |
| Latency | How many cycles from input to output. |
| Throughput | How much data moves per cycle or per second. |
| Pipeline | Split long logic into stages with registers between them. |
| Fmax | The highest clock frequency at which the design still meets timing. |
| Metastability | A flip-flop output is unstable for a short time because its input changed close to the clock edge. |
| Synchronizer | Flip-flops that bring a single-bit signal safely into another clock domain. |
| CDC / RDC | Clock Domain Crossing / Reset Domain Crossing. |
| Pointer | A value that points to a memory location. Example: FIFO write pointer. |
| Gray code | A counting code where only one bit changes per step. |
| Setup time | Data must be stable this long before the clock edge. |
| Hold time | Data must stay stable this long after the clock edge. |
| Slack | Timing margin. Positive means it meets timing. Negative means it fails. |
| Clock skew | Difference in clock arrival time between two flip-flops. |
| Jitter | The clock edge moves a little from its ideal position. |
| Timing closure | Meeting setup and hold on every path. |
| Critical path | The path with the worst slack. It limits Fmax. |
| Recovery / removal | Setup-like and hold-like checks for the release of an asynchronous reset. |
| Synthesis | The tool converts RTL into gates, registers, and muxes. |
| Inference | Writing RTL so the tool understands the hardware you want (RAM, register, mux). |
| Elaboration | The tool expands parameters and generate blocks into the real design structure. |
| Lint | Static check of RTL for bad coding style and hidden bugs. No simulation needed. |
| STA | Static Timing Analysis. Checks timing on all constrained paths without test vectors. |
| XDC / SDC | Constraint file formats (Vivado / industry standard). They give clocks, I/O timing and exceptions to the tools. |
| Scoreboard | A checker that compares what went in with what came out. |
| Assertion | A rule about behavior that is checked automatically. |
| PPA | Power, Performance, Area. The three design trade-offs. |

# PART 2 — CORE RTL FUNDAMENTALS

## Q2.1  What is RTL design?   MUST KNOW

D  RTL means Register Transfer Level. It describes how data moves between registers and what logic operates on the data.

P  We use RTL to describe digital hardware before synthesis.

W  Identify registers, combinational logic, control logic, clocks, and data paths. Write them in Verilog or SystemVerilog. The synthesis tool turns them into real hardware.

E  A counter, a FIFO controller, or an AXI-Stream block can all be written in RTL.

Interview line:  “RTL describes the data path and the control path at the register-transfer level.”

Follow-up questions to expect

- RTL vs behavioral model? — RTL is written so it can be synthesized. A behavioral model may use delays and constructs with no hardware meaning.

- RTL vs gate level? — Gate level is the netlist after synthesis, with real cells.

## Q2.2  What is combinational logic?   MUST KNOW

D  Combinational logic is logic where the output depends only on the current inputs.

P  We use it for decisions and calculations that need no memory: mux, decoder, adder, comparator.

W  The output is re-evaluated when any input changes. There is no clock and no stored value. In RTL we write it with assign or always_comb. Every output must get a value on every path, or a latch is created.

E  A 2:1 mux: the output follows a or b based on sel.

always_comb begin

y = 1'b0;            // default value avoids a latch

if (sel) y = a;

else     y = b;

end

Interview line:  “Combinational logic has no memory. I always give a default value so synthesis does not infer a latch.”

Follow-up questions to expect

- What causes a latch? — Not assigning an output in every branch of if or case.

- Which coding style do you use? — always_comb for combinational logic.

## Q2.3  What is sequential logic?   MUST KNOW

D  Sequential logic is logic whose behavior depends on the current inputs and stored past state.

P  We use it when the design must remember something: counters, FSM state, data registers.

W  A flip-flop captures its input on the active clock edge and holds the value until the next edge. We write it with always_ff @(posedge clk) and non-blocking assignments.

E  A counter that adds 1 on every clock cycle. Flip-flops, registers, counters, and FSMs are all sequential.

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) cnt <= '0;

else        cnt <= cnt + 1'b1;

end

Interview line:  “Sequential logic stores state and updates it on the active clock edge, so I use non-blocking assignments.”

## Q2.4  What is a flip-flop, and what is a register?

D  A flip-flop stores one bit on a clock edge. A register is a group of flip-flops that share a clock (and usually an enable and reset).

P  Flip-flops give clean, predictable timing. All synchronous design is built on them.

W  The flip-flop samples D at the clock edge and shows it on Q after clock-to-Q delay. Inputs must meet setup and hold around the edge. Between edges the output does not change.

E  A 32-bit data register with clock enable is 32 flip-flops sharing clk and en.

Interview line:  “A register is a bank of flip-flops. I think about its setup, hold, and clock-to-Q whenever I look at a path.”

Follow-up questions to expect

- What is clock enable? — A control that lets the flop load only when en is high. It saves a mux in the D path on FPGAs that have enable pins.

- D flip-flop vs T flip-flop? — D loads data. T toggles. In RTL we normally write D behavior.

## Q2.5  What is a latch, and why is an unintended latch a problem?   MUST KNOW

D  A latch is a level-sensitive storage element: it is transparent while enable is active and holds its value otherwise.

P  We must know this because an unintended latch is a very common RTL bug. It causes timing trouble and simulation mismatch.

W  If combinational code does not assign an output on every path, synthesis keeps the old value, which needs storage. That storage is a latch. The fix is a default assignment or a complete if/else and case with default.

E  A missing else in an always @(*) block creates an unwanted latch.

// BAD: latch, because y is not assigned when en = 0

always_comb if (en) y = a;

// GOOD

always_comb begin

y = 1'b0;

if (en) y = a;

end

Interview line:  “An unintended latch usually means an incomplete combinational assignment. I fix it with a default value at the top of the block.”

Follow-up questions to expect

- Are latches always bad? — No. Clock gating cells use them on purpose. In normal RTL they are a bug.

- Latch vs flip-flop? — Latch is level-sensitive. Flip-flop is edge-triggered.

## Q2.6  Blocking versus non-blocking assignment?   MUST KNOW

D  Blocking (=) executes in order, one statement after another. Non-blocking (<=) schedules the update and applies it at the end of the time step.

P  The correct style makes simulation match the real hardware.

W  Rule of thumb: `<=` in clocked blocks, `=` in combinational blocks. Do not mix both styles on the same signal.

E  A 2-stage shift register with <= gives two flip-flops. With = both variables get the same value in the same step, and only one flop remains.

// Correct: two flip-flops

always_ff @(posedge clk) begin

q1 <= d;

q2 <= q1;

end

Interview line:  “For clocked RTL I use non-blocking assignments so all registers update together at the clock edge.”

Follow-up questions to expect

- What does lint report for this? — Blocking assignment in a sequential block (W336 style).

- Why does the wrong style cause races? — Two blocks triggered by the same edge may read a signal before or after it is updated, depending on simulator order.

## Q2.7  What is the difference between wire, reg, and logic, and between always, always_ff, always_comb, and always_latch?

D  wire is a net driven by continuous assignment. reg is a variable assigned in a procedural block (it does not always mean a register). logic is the SystemVerilog type that works for both. always_ff, always_comb, always_latch state the intent of the block.

P  Intent-based blocks let the tool check your coding: always_ff must be clocked, always_comb must not create a latch.

W  always_comb runs at time 0 and is sensitive to all inputs automatically. always_ff requires an edge event. always_latch is for intentional latches. Use assign for simple combinational expressions.

E  logic [7:0] data; can be driven from always_ff or assign, but by only one driver.

Interview line:  “In SystemVerilog I use logic with always_ff and always_comb, so the tool checks my intent.”

Follow-up questions to expect

- Does reg always become a flip-flop? — No. It becomes a flop only if assigned in a clocked block.

- Can a logic signal have two drivers? — Not from two procedural blocks. That is a multiple-driver error.

## Q2.8  What are synchronous and asynchronous resets?   MUST KNOW

D  Reset puts registers into a known state. Synchronous reset acts on the clock edge. Asynchronous reset acts immediately, without a clock.

P  A known start state avoids unknown (x) behavior and gives a repeatable startup.

W  Synchronous reset is part of the D logic and needs an active clock. Asynchronous reset works without a clock, but its release is asynchronous, so it needs a reset synchronizer (assert async, release sync). Data-path registers often need no reset. Control registers do.

E  A reset synchronizer for a clock domain: two flops, D tied high, async clear.

always_ff @(posedge clk or negedge arst_n) begin

if (!arst_n) {rst_sync_n, rst_meta} <= 2'b00;

else         {rst_sync_n, rst_meta} <= {rst_meta, 1'b1};

end

Interview line:  “I assert reset asynchronously but release it synchronously, so recovery and removal timing are safe.”

Follow-up questions to expect

- Do all registers need reset? — No. Reset every control and state register. Data registers can be left without reset to save area and routing.

- What is the drawback of a synchronous reset? — It needs the clock running, and it adds logic on the D input.

- What is the drawback of an asynchronous reset? — Release timing (recovery/removal) and a glitchy reset can reset the design by mistake.

## Q2.9  What is a parameter, and what is the difference between parameter and localparam?

D  A parameter is a constant that can be changed when the module is instantiated. A localparam cannot be changed from outside.

P  It makes the module reusable: change width or depth without editing the code.

W  Use parameter for configurable values (DATA_W, DEPTH). Use localparam for values calculated from them (ADDR_W = $clog2(DEPTH)). Override at instantiation with #(.DATA_W(64)).

E  A FIFO with parameters DATA_W and DEPTH.

module fifo #(parameter int DATA_W = 32,

parameter int DEPTH  = 16) (...);

localparam int ADDR_W = $clog2(DEPTH);

endmodule

Interview line:  “I parameterize width and depth, and derive dependent values with localparam so nothing is hard-coded.”

Follow-up questions to expect

- What does $clog2 do? — Ceiling of log2. Gives the number of bits needed to address N items.

- What is a defparam? — An old way to override a parameter. Avoid it. Use #( ) at instantiation.

## Q2.10  What is a generate block?

D  Generate constructs create repeated or conditional hardware at elaboration time.

P  They keep RTL scalable and structured when the hardware depends on a parameter.

W  The elaborator expands for-generate and if-generate into real instances or logic before simulation and synthesis. The loop variable is a genvar.

E  A parameterized ripple-carry adder that generates N full-adder instances.

genvar i;

generate

for (i = 0; i < N; i++) begin : g_fa

full_adder u_fa (.a(a[i]), .b(b[i]), .cin(c[i]), .s(s[i]), .cout(c[i+1]));

end

endgenerate

Interview line:  “I use generate when the structure of the hardware depends on a parameter.”

Follow-up questions to expect

- Generate vs a normal for loop in always? — A normal loop describes behavior inside one block. Generate creates separate hardware instances.

## Q2.11  Why are signal widths important?   MUST KNOW

D  Width is the number of bits a signal has.

P  Correct width prevents overflow, truncation, and unexpected arithmetic results.

W  During an operation, operands are extended or truncated depending on width and signedness. An 8-bit result placed into a 4-bit signal loses the upper 4 bits. A counter that is too narrow wraps around silently.

E  Adding two 8-bit values needs a 9-bit result to keep the carry.

logic [7:0] a, b;

logic [8:0] sum;

assign sum = {1'b0, a} + {1'b0, b};   // explicit extra bit keeps the carry

Interview line:  “I check widths at arithmetic and interface boundaries, because a width mismatch changes the hardware behavior silently.”

Follow-up questions to expect

- What does lint say? — Width mismatch and carry-loss warnings (W164, W484 style).

## Q2.12  Signed versus unsigned data?

D  Unsigned values are non-negative binary numbers. Signed values normally use two’s-complement.

P  The correct type matters for comparisons, shifts, and arithmetic.

W  The same bit pattern is read differently. If any operand in an expression is unsigned, the whole expression is treated as unsigned. Signed values use >>> for arithmetic shift with sign extension.

E  4-bit 1111 is 15 unsigned but −1 signed.

Interview trap:  Mixing a signed and an unsigned operand in one comparison makes the comparison unsigned. A negative number then looks like a very large positive one.

Interview line:  “Before an arithmetic operation or a comparison I confirm both the width and the signedness of each operand.”

## Q2.13  case, casez, casex, unique case, and priority case — what is the difference?

D  case matches exactly. casez treats z / ? as don’t-care. casex treats x and z as don’t-care (avoid). unique and priority add checks and synthesis intent.

P  The right form avoids latches, wrong matches, and simulation/synthesis differences.

W  unique case: exactly one item must match; no overlap. priority case: the first match wins. Always add a default. Do not use casex in RTL: it can hide x bugs.

E  A priority encoder is naturally casez or a priority if/else if chain.

always_comb begin

idx = 2'd0;

casez (req)

4'b???1: idx = 2'd0;

4'b??10: idx = 2'd1;

4'b?100: idx = 2'd2;

4'b1000: idx = 2'd3;

default: idx = 2'd0;

endcase

end

Interview line:  “I use unique case when the items cannot overlap and always keep a default, so the intent is checked and no latch is created.”

Follow-up questions to expect

- if-else vs case in hardware? — if/else chain gives priority; case gives parallel selection. Tools often optimize both, but the intent should be clear.

## Q2.14  What is synthesis?   MUST KNOW

D  Synthesis converts RTL into a hardware representation such as registers, gates, and multiplexers.

P  It connects the RTL description to real hardware implementation.

W  The tool analyzes the RTL, infers hardware structures, optimizes logic, and maps it to the target cells or FPGA primitives, using your constraints.

E  A Verilog counter becomes flip-flops plus an incrementer.

Interview line:  “I write synthesizable RTL, and I check synthesis reports to see that the tool inferred the hardware I intended.”

Follow-up questions to expect

- Name non-synthesizable constructs. — #delay, initial blocks for logic, $display, file I/O, wait.

- What are synthesis and implementation in Vivado? — Synthesis makes the netlist. Implementation does place and route.

## Q2.15  What is simulation versus synthesis mismatch?   MUST KNOW

D  It is when simulation behavior differs from the synthesized hardware behavior.

P  The mismatch can hide real hardware bugs.

W  Common causes: incomplete assignments (latches), wrong sensitivity list, blocking/non-blocking misuse, x handling, reset assumptions, simulation-only constructs, and unsupported synthesis constructs.

E  An incomplete combinational assignment simulates with a retained value and infers a latch in hardware.

Interview line:  “Passing simulation is not enough. I also run lint and read the synthesis report for latches, width mismatches, and timing.”

Follow-up questions to expect

- How do you catch it? — Lint, synthesis warnings, gate-level simulation, and formal equivalence checking.

## Q2.16  What is a race condition in RTL simulation?

D  A race is when the result depends on the order in which the simulator runs two events in the same time step.

P  Races give results that change between tools or runs, so they are hard to debug.

W  Typical causes: blocking assignments in clocked blocks, testbench driving inputs exactly at the clock edge, and two blocks triggered by the same signal. Fixes: non-blocking assignments in clocked logic; drive testbench inputs away from the active edge (for example on the negedge, or with a clocking block).

E  A testbench that changes rst_n at the same posedge clk where the DUT samples it can see either old or new value.

Tip:  In a testbench, releasing reset on the same clock edge as the first write can give a false mismatch. Release reset on the falling edge and the race disappears.

Interview line:  “I use non-blocking assignments in the design and drive the testbench away from the active clock edge to avoid races.”

Follow-up questions to expect

- What is a clocking block? — A SystemVerilog construct that defines when testbench signals are sampled and driven relative to the clock.

## Q2.17  What is X propagation, and how do you handle unknown values?   ADVANCED

D  x means unknown in simulation. X propagation is when an unknown value moves through logic and spreads to outputs.

P  It shows uninitialized registers, missing resets, and undriven signals.

W  Registers without reset start as x. An if (x) in simulation takes the else branch, which may hide a bug (X-optimism). Some expressions may make x spread more than real hardware (X-pessimism). Reset the control registers, check for x on interface signals, and add assertions such as !$isunknown(valid).

E  A FSM without reset stays in x and never leaves it.

Interview line:  “I reset the control state, and I assert that control signals are never unknown after reset.”

Follow-up questions to expect

- X-optimism vs X-pessimism? — Optimism: sim shows a clean value where hardware is unknown. Pessimism: sim shows x where hardware has a known value.

## Q2.18  What is inference, and how do you infer RAM, DSP, and registers?   ADVANCED

D  Inference is writing RTL in a style that the synthesis tool recognizes as a specific hardware block.

P  It gives efficient hardware (block RAM, DSP slice) without instantiating vendor primitives.

W  Block RAM: array with synchronous read (and write in a clocked block). Distributed RAM / LUT RAM: array with asynchronous read. DSP: multiplier with registers around it. Shift register (SRL): shift chain with no reset. Reading a RAM in a combinational way, or adding reset to every RAM word, can block inference.

E  A simple dual-port RAM with synchronous read infers block RAM. See the code in Part 12.

Interview line:  “I write RAM with a synchronous read and no reset on the array, so the tool infers block RAM instead of building it from flip-flops.”

Follow-up questions to expect

- Why does block RAM give one-cycle read latency? — The read address or the output is registered inside the RAM primitive.

- Can a RAM have reset? — Only the output register can be reset. Not the whole array.

## Q2.19  What is a combinational loop?   ADVANCED

D  It is a signal path that feeds back to itself through only combinational logic, with no register in the loop.

P  It is a design error: the value can oscillate or hold unpredictably, and STA cannot time it properly.

W  Typical accident: ready depends on valid, and valid depends on ready. Break the loop with a register, or change the protocol rule so only one side depends on the other.

E  Two blocks that each derive their handshake signal from the other’s combinational output.

Interview line:  “Every feedback loop in my design has a register in it. I check for combinational loops in lint and STA reports.”

Follow-up questions to expect

- How do tools detect them? — Lint flags them; STA reports “combinational loop” and breaks the loop for analysis.

## Q2.20  What are glitches and hazards?   ADVANCED

D  A glitch is a short unwanted pulse on a signal. A hazard is the condition in combinational logic that can cause it when inputs change close together.

P  Glitches are harmless if the signal is only sampled by a clock edge later. They are dangerous on clocks, resets, and enables of asynchronous circuits.

W  Different path delays make a signal change in steps. Never use a combinational output directly as a clock or asynchronous reset. Register the signal first, or use a proper clock-gating cell.

E  A decoded address used directly as an asynchronous reset can glitch and reset the design.

Interview line:  “Combinational outputs may glitch. So I register anything that drives a clock, a reset, or an asynchronous control.”

Follow-up questions to expect

- Static-1 hazard? — Output should stay 1 but briefly drops to 0 when one input changes.

## Q2.21  What is clock gating, and how is it done safely?   ADVANCED

D  Clock gating stops the clock to a block when it is idle.

P  It saves dynamic power, because idle flip-flops do not toggle.

W  A safe gate uses an integrated clock gating (ICG) cell: a latch that holds the enable while the clock is high, plus an AND. The latch prevents glitches on the gated clock. In FPGAs, prefer clock-enable on flip-flops or the vendor clock-gating primitive (for example BUFGCE).

E  A latch-based clock gate concept:

// Concept only. Use the library ICG cell in real designs.

always_latch if (!clk) en_l = en;

assign gclk = clk & en_l;

Interview line:  “I do not gate a clock with a plain AND. I use an ICG cell, or a clock enable on FPGAs, and I check the clock-gating timing.”

Follow-up questions to expect

- Why not gate with plain AND? — The enable may change while the clock is high and create a glitch or a runt pulse.

- FPGA alternative? — Clock-enable pin on the flip-flop, or a clock-buffer enable primitive.

# PART 3 — READY/VALID HANDSHAKE AND FSM

## Q3.1  What is the ready/valid handshake?   MUST KNOW

D  Ready/valid is a synchronous handshake between a producer and a consumer.

P  It moves data safely and lets the receiver apply backpressure.

W  The producer drives VALID when data is available. The consumer drives READY when it can accept data.

•  A transfer happens on the clock edge where VALID = 1 and READY = 1.

•  Once VALID is high, the producer must keep VALID and the data stable until the transfer happens.

E  AXI-Stream (TVALID/TREADY) and Avalon-ST (valid/ready) both use this.

wire fire = valid && ready;   // transfer happens here

always_ff @(posedge clk)

if (fire) data_q <= data;   // capture only on a real transfer

Interview line:  “The actual transfer occurs on the clock edge when both VALID and READY are high.”

Follow-up questions to expect

- Can VALID wait for READY? — No. The producer must not wait for READY before raising VALID (AXI rule). Otherwise both sides can wait forever.

- Can READY depend on VALID? — Yes. The consumer may wait for VALID before raising READY.

- VALID=1, READY=0? — No transfer. Producer holds data and VALID.

- VALID=0, READY=1? — No transfer. Consumer is waiting.

- VALID=0, READY=0? — Idle. No transfer.

## Q3.2  What is backpressure?   MUST KNOW

D  Backpressure is a flow-control method where a slow receiver tells the sender to wait.

P  It prevents overflow and data loss when the receiver cannot accept data.

W  The receiver lowers READY. If VALID is high at that time, no transfer happens, and the sender keeps the current beat stable. When a FIFO sits in the middle, its almost-full flag lowers READY early, before the FIFO is really full, so beats already in flight still have room.

E  A full FIFO makes the downstream READY go low, and the Master stalls.

Interview line:  “Backpressure controls the flow from the receiver side without losing the current transaction.”

Follow-up questions to expect

- Why almost-full and not full? — Because of the latency between READY and the last accepted word.

- What happens to a stalled sender’s data? — It stays stable with VALID high until READY returns.

## Q3.3  How does backpressure travel through a pipeline, and what is a skid buffer?   ADVANCED

D  In a long pipeline, READY moves backward from the last stage to the first. A skid buffer (register slice) is a small buffer that lets READY be registered without losing throughput.

P  A long combinational READY chain limits Fmax. A registered READY breaks that chain, but it creates one late beat that needs a place to go. The skid buffer holds that beat.

W  There are two storage slots: the main output register and one skid register. Normally data goes straight to the output. If the output stalls, the next beat is stored in the skid register and READY drops (from a flop). When the output moves again, the skid data goes forward.

E  A tested valid/ready skid buffer with full throughput. Full code and test result are in Part 12.

Note:  Full code is in Q12.12. It was simulated with random VALID/READY and checked for data order, stable output while stalled, and one beat per cycle when READY stays high.

Interview line:  “I use a skid buffer to register READY. It breaks the long backpressure path and still keeps full throughput.”

Follow-up questions to expect

- What is the cost? — One extra data register and a little control logic.

- Register slice vs skid buffer? — A forward register slice registers VALID/DATA only. A skid buffer registers the READY path as well.

## Q3.4  How do you avoid deadlock in a handshake design?   ADVANCED

D  Deadlock is when both sides wait for each other forever.

P  A deadlock stops the whole data path and is hard to find in a long simulation.

W  Follow the protocol: VALID must never wait for READY. READY may wait for VALID. Do not create a loop between two blocks where each waits for the other’s output. Give FSMs a timeout or a clear exit. Check FIFO almost-full thresholds so READY can always come back.

E  A master that waits for READY before raising VALID, and a slave that waits for VALID before raising READY: nothing ever moves.

Interview line:  “To avoid deadlock I make sure only one side of each handshake waits for the other, and that every FSM state has a defined exit.”

Follow-up questions to expect

- How do you find it in verification? — Watchdog timers, assertions that “VALID is followed by READY within N cycles”, and end-of-test checks that all FIFOs are empty.

## Q3.5  What is an FSM (Finite State Machine)?   MUST KNOW

D  An FSM is a sequential control block that moves between a fixed set of states.

P  It controls multi-step operations in a clear, predictable way.

W  The FSM has three parts: state register, next-state logic, and output logic.

•  The current state and inputs decide the next state. The state register updates on the clock edge.

E  A packet sender with IDLE, SEND, and DONE states.

module packet_fsm (

input  logic clk, rst_n,

input  logic start, fire, last,

output logic busy

);

typedef enum logic [1:0] {IDLE, SEND, DONE} state_t;

state_t state, next;

// 1) state register

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) state <= IDLE;

else        state <= next;

// 2) next-state logic (default first, so no latch)

always_comb begin

next = state;

case (state)

IDLE:    if (start)         next = SEND;

SEND:    if (fire && last)  next = DONE;

DONE:                       next = IDLE;

default:                    next = IDLE;   // illegal-state recovery

endcase

end

// 3) output logic (Moore)

assign busy = (state == SEND);

endmodule

Interview line:  “I separate the state register, next-state logic, and output logic, and I always give default values so no latch is created.”

Follow-up questions to expect

- How many always blocks? — Two or three. State register in always_ff; next state in always_comb; outputs assigned or in another always_comb.

## Q3.6  Moore versus Mealy FSM?   MUST KNOW

D  In a Moore FSM, outputs depend only on the current state. In a Mealy FSM, outputs depend on the current state and the inputs.

P  The choice affects output timing, number of states, and glitch behavior.

W  Moore outputs change only after a state change, so they are stable and glitch-free, but they arrive one cycle later. Mealy outputs react as soon as an input changes, so they use fewer states, but they can glitch and can create long input-to-output combinational paths.

E  A "1011" detector: as Moore it needs a state for "1011" found. As Mealy the output can go high on the edge where the last bit arrives.

Interview line:  “I choose Moore for stable, glitch-free outputs and Mealy when I need faster response or fewer states.”

Follow-up questions to expect

- Which one do you prefer for outputs that drive other blocks? — Moore, or a registered Mealy output.

- How do you make a Mealy output safe? — Register it, at the cost of one cycle of latency.

## Q3.7  How do you choose FSM state encoding — binary, one-hot, or Gray?

D  Encoding is how state names are mapped to bit patterns.

P  It affects speed, area, and power.

W  Binary: fewest flops, more decode logic. One-hot: one flop per state, very fast and simple decode, more flops (good for FPGAs). Gray: one bit changes per transition, low power and glitch-safe outputs. Many tools re-encode FSMs automatically.

E  A 10-state FSM: binary uses 4 flops, one-hot uses 10.

Interview line:  “On FPGAs I usually let the tool use one-hot for speed. For low power or glitch-sensitive outputs I consider Gray.”

Follow-up questions to expect

- How do you stop the tool from re-encoding? — Use a synthesis attribute such as fsm_encoding = "none" when you need a fixed encoding (for safety).

## Q3.8  Why use default assignments in FSM combinational logic?   MUST KNOW

D  Default assignments give every combinational output a value on every path.

P  They prevent inferred latches and make the code shorter.

W  Assign next_state = state (or a safe state) and all outputs to their idle value at the top of the block. Then override them inside the case items.

E  Set next to state before the case statement.

Interview line:  “I use default assignments and cover all cases to keep the next-state logic purely combinational.”

## Q3.9  What is illegal-state recovery, and why include a default state?

D  A default branch handles unexpected or illegal state encodings.

P  Noise, a bad reset, or a single-event upset can put the FSM in a state that does not exist. Without recovery it may lock up.

W  In the default branch, send the FSM to a known safe state, usually IDLE. One-hot FSMs can add a check that exactly one bit is set.

E  After an illegal state, the controller returns to IDLE.

Interview line:  “The default branch gives a defined recovery path for illegal or unexpected states.”

Follow-up questions to expect

- Is default needed if all states are listed? — With enum encoded in 3 bits and 5 states, three patterns are unused. So yes, keep default for safety and to satisfy lint.

## Q3.10  Should FSM outputs be registered?   ADVANCED

D  A registered output is computed one cycle earlier and stored in a flip-flop.

P  It removes glitches, shortens the output path, and improves timing.

W  Compute the output from next state (or from state and input) and store it. The output is now clean and starts right at the clock edge. The trade-off is that the design must account for the earlier calculation.

E  A registered valid in a packet sender avoids a long path from state decode to the bus.

Interview line:  “For outputs that leave the block, I register them. It gives clean timing and avoids glitches.”

Follow-up questions to expect

- Any downside? — Slight extra logic, and the output timing is one state ahead, so the code needs care.

Other FSM questions that appear in interviews are answered in Part 12 (sequence detector, packet generator) and Part 6 (Master and Slave FSMs).

# PART 4 — FIFO, PIPELINE, PERFORMANCE

## Q4.1  What is a FIFO?   MUST KNOW

D  A FIFO is a buffer where the first data written is the first data read.

P  It connects two blocks that work at different rates, and it absorbs short bursts.

W  A write pointer shows where the next data goes. A read pointer shows where the next data comes from.

•  Write is allowed when the FIFO is not full. Read is allowed when it is not empty.

•  Full and empty come from comparing the pointers. An extra pointer bit tells full from empty.

E  A packet buffer between a producer and a consumer. Tested code is in Part 12.

wire wr_fire = wr_en && !full;

wire rd_fire = rd_en && !empty;

assign empty = (wptr == rptr);

assign full  = (wptr[AW-1:0] == rptr[AW-1:0]) && (wptr[AW] != rptr[AW]);

Interview line:  “I use one extra pointer bit. If all bits match the FIFO is empty. If only the top bit differs it is full.”

Follow-up questions to expect

- Why not keep a counter? — A counter works in a sync FIFO but is hard to use across two clocks.

## Q4.2  What is the difference between synchronous and asynchronous FIFO?   MUST KNOW

D  A synchronous FIFO uses one clock. An asynchronous FIFO uses separate write and read clocks.

P  The asynchronous FIFO safely moves multi-bit data between clock domains.

W  In a sync FIFO, pointers are compared directly. In an async FIFO, pointers are converted to Gray code and synchronized into the opposite domain before comparison. This costs extra flops and gives conservative full/empty flags.

E  Sync: packet buffer inside one block. Async: between a 250 MHz block and a 156 MHz block. Details are in Part 7.

Interview line:  “If the clocks are the same, a sync FIFO is enough. If the clocks are asynchronous, I use an async FIFO with Gray-coded pointers.”

Follow-up questions to expect

- Is a FIFO with two related clocks (2:1) async? — Treat it as async unless the phase relationship is guaranteed and constrained.

## Q4.3  What is FIFO depth, and how do you calculate it?   MUST KNOW

D  Depth is the number of words the FIFO can store.

P  The FIFO must be deep enough to absorb the worst-case burst without overflow, and no deeper than needed.

W  Worst case: the writer writes at full rate during a burst, while the reader is slower or delayed.

•  depth ≥ burst_length × (1 − read_rate / write_rate) + latency margin.

•  Add margin for READY latency, pointer synchronization delay (async FIFO), and any words in flight.

•  Round up to the next practical size (power of two for Gray-pointer async FIFOs).

E  Worked example. Burst = 100 words. Write rate = 1 word/cycle. Read rate = 0.5 word/cycle (READY is low half of the time). During the burst, 100 words go in and 50 come out. Minimum depth = 100 × (1 − 0.5) = 50. Add 8 words margin for latency → 58 → choose 64.

Interview line:  “I size depth from the worst-case burst and the rate difference, then add margin for latency and round up to a power of two.”

Follow-up questions to expect

- What if the burst is back-to-back? — Then use the longest burst plus the gap needed to drain, or apply backpressure.

- Why almost-full at depth − N? — N = words still in flight after READY drops.

## Q4.4  What is FIFO width?

D  Width is the number of bits in each FIFO word.

P  It must carry the data plus all side information that belongs to the same beat.

W  Width = data bits + sideband bits (SOP, EOP, EMPTY, error, user bits). Keeping sideband in the same word keeps it aligned with its data.

E  In the Avalon-ST project the word is 512 + 1 + 1 + 1 + 6 = 521 bits.

Interview line:  “I put the sideband bits in the same FIFO word as the data, so control and payload can never get out of step.”

## Q4.5  What are FIFO overflow and underflow, and how do you prevent them?   MUST KNOW

D  Overflow is writing into a full FIFO. Underflow is reading from an empty FIFO.

P  Both lose or corrupt data.

W  Gate the write with !full and the read with !empty. In a valid/ready design, drive READY low (from almost-full) so the source never writes into a full FIFO. Add assertions on wr_en && full and rd_en && empty.

E  An assertion that fires if the writer pushes while the FIFO is full.

assert property (@(posedge clk) disable iff (!rst_n) !(wr_en && full));

assert property (@(posedge clk) disable iff (!rst_n) !(rd_en && empty));

Interview line:  “I block writes when full and reads when empty in the RTL, and I also put assertions on both, so a protocol bug is caught immediately.”

## Q4.6  What are almost-full and almost-empty?

D  They are early-warning flags set before the FIFO is really full or empty.

P  They give the other side time to react.

W  Almost-full lowers READY early, so beats that are already in flight still fit. Almost-empty warns the reader that data is running out. Thresholds are programmable or fixed at depth − N and N.

E  FIFO almost full → READY low → Master stalls.

Interview line:  “I drive backpressure from almost-full, not from full, so nothing is lost while READY takes effect.”

## Q4.7  What is a FWFT FIFO versus a standard FIFO, and what is read latency?   ADVANCED

D  In a FWFT (first-word fall-through) FIFO the next word is already on the output when the FIFO is not empty. In a standard FIFO the data appears one cycle after rd_en.

P  The choice changes how the read-side logic is designed.

W  Standard: assert rd_en at cycle N, use data at N+1. The read FSM must handle this one-cycle read latency and track a "data valid" flag. FWFT: check !empty, use the data now, and pulse rd_en to move to the next word.

E  A read FSM with IDLE → CHECK → READ → SEND handles the one-cycle delay of a standard FIFO.

Interview line:  “With a standard FIFO I account for the one-cycle read latency in the read FSM, so I never use stale data.”

Follow-up questions to expect

- Which is easier for downstream logic? — FWFT, because valid data is visible early.

- Which does block RAM give? — Registered read, so standard (non-FWFT) unless a wrapper adds FWFT logic.

## Q4.8  What is pipelining?   MUST KNOW

D  Pipelining divides a long combinational path into stages with registers between them.

P  It shortens the longest path, so the clock can run faster. This improves timing closure and throughput.

W  Each stage adds one cycle of latency, but a new item can enter every cycle. When the receiver stalls, the stages need enable or valid/ready control. The valid bit travels with the data.

E  A 2-stage multiply-add pipeline: latency 2 cycles, throughput 1 result per cycle. Tested code is in Part 12.

always_ff @(posedge clk) begin

prod <= a * b;          // stage 1

y    <= prod + c_d;     // stage 2

end

Interview line:  “Pipelining trades latency for frequency. Throughput stays one word per cycle unless backpressure stalls the pipeline.”

Follow-up questions to expect

- What is the cost? — More flops, more area, more latency, and more control logic.

- How do you pipeline with backpressure? — Valid/ready per stage, a global stall enable, or skid buffers.

## Q4.9  Latency versus throughput?   MUST KNOW

D  Latency is the time from input to the matching output. Throughput is the rate at which results are produced.

P  Both are performance metrics, and they trade off differently.

W  A pipeline can have high latency and still high throughput, because many items are in flight at once. Stalls (backpressure) reduce throughput. Bus width × clock frequency sets the peak throughput.

E  A 4-stage pipeline accepts new data every cycle after it fills: latency 4, throughput 1 per cycle. A 512-bit bus at 250 MHz moves 64 B × 250 M = 16 GB/s.

Interview line:  “Latency and throughput are different. A design can have higher latency but still achieve high throughput.”

Follow-up questions to expect

- How do you compute throughput with stalls? — Throughput = beats accepted / total cycles. If READY is high 80% of the time, throughput is 80% of peak.

## Q4.10  How do you calculate counter width and bytes per beat?   MUST KNOW

D  Counter width is the number of bits needed for the largest value the counter reaches.

P  A counter that is too narrow wraps around silently. A counter that is too wide wastes area.

W  Width = $clog2(MAX + 1) bits for values 0 to MAX.

•  Bytes per beat = bus width / 8. Beats per packet = ceil(packet bytes / bytes per beat).

•  Watch for rollover, and for signed/unsigned in comparisons.

E  350 needs 9 bits (2⁹ = 512 > 350). A 256-byte packet on a 512-bit bus: 64 B/beat → 4 beats.

localparam int MAX_CNT = 350;

localparam int CW = $clog2(MAX_CNT + 1);   // 9 bits

logic [CW-1:0] delay_cnt;

Interview line:  “I calculate counter width from the maximum value and check that no add or compare loses a carry.”

Follow-up questions to expect

- What happens on overflow? — It wraps to 0, and control logic may misbehave.

- A 100-byte packet on a 64-byte bus? — 2 beats. Last beat has 36 valid bytes and EMPTY = 28.

## Q4.11  What is packing and unpacking?

D  Packing joins several fields into one wider word. Unpacking splits them back.

P  A FIFO stores one word. Data and its side information must stay together and aligned.

W  Choose a fixed field order, document it, and use the same order on the write and read side. A mismatch in order is a classic bug.

E  {EMPTY[5:0], EOP, SOP, VALID, DATA[511:0]} = 521 bits.

wire [520:0] fifo_din = {empty, eop, sop, valid, data};

wire [511:0] data_o   = fifo_dout[511:0];

wire         valid_o  = fifo_dout[512];

wire         sop_o    = fifo_dout[513];

wire         eop_o    = fifo_dout[514];

wire [5:0]   empty_o  = fifo_dout[520:515];

Interview line:  “I keep control bits stored with their data beat, so packet boundaries cannot get out of step with the payload.”

## Q4.12  What is PPA?

D  PPA means Power, Performance, Area.

P  RTL choices affect all three, so every design is a trade-off.

W  More registers can improve timing but add area and clock power. More parallel logic can raise throughput but adds area and power. Sharing hardware saves area but may lower throughput.

E  A large parallel datapath versus a smaller, reused datapath is a PPA trade-off.

Interview line:  “RTL optimization is a trade-off between timing, area, and power, so I first ask which one is the priority.”

## Q4.13  How do you improve Fmax, reduce area, and reduce power in RTL?   ADVANCED

D  These are the three common optimization goals.

P  Interviewers ask this to see whether you understand the causes, not only the tool switches.

W  Fmax: pipeline long paths, reduce logic levels, reduce fanout (duplicate registers), register outputs, retime, use one-hot FSMs.

•  Area: share resources, use smaller widths, remove unused logic, use block RAM instead of flops for storage, reduce reset on data.

•  Power: clock gating or clock enables, reduce toggling of idle data, smaller widths, avoid unnecessary resets.

E  A critical path with a 32×32 multiplier and an add is split into multiply and add stages.

Interview line:  “First I read the timing report to find the real critical path. Then I fix that path, not a guess.”

Follow-up questions to expect

- What is retiming? — Moving registers across combinational logic to balance stage delay.

- What is resource sharing? — One adder or multiplier reused in different cycles.

# PART 5 — AXI-STREAM AND AXI

## Q5.1  What is AXI-Stream and how does it work?   MUST KNOW

D  AXI-Stream is a point-to-point streaming interface that moves data from a transmitter to a receiver.

P  It gives simple data transfer with flow control. It has no address, only a stream.

W  The transmitter drives TDATA, TVALID, TLAST (and optional sideband signals).

•  The receiver drives TREADY.

•  A beat moves when TVALID && TREADY are both high on the same rising clock edge.

E  Transmitter → AXI-Stream interface → Receiver.

wire beat_accepted = tvalid && tready;

always_ff @(posedge aclk)

if (beat_accepted)

beat_cnt <= tlast ? '0 : beat_cnt + 1'b1;

Interview line:  “In AXI-Stream the transmitter drives data, valid, and last. The receiver drives ready. A beat moves only when valid and ready are both high.”

Follow-up questions to expect

- Is AXI-Stream address-based? — No. It is only a data stream.

- Which side owns TREADY? — The receiver.

The general handshake questions for AXI-Stream (transfer condition, VALID high with READY low, READY high with VALID low, backpressure and why it matters) are answered once, in Part 3:

| Question (already answered, so not repeated) | Answered in |
| --- | --- |
| When does an AXI-Stream data transfer (handshake) occur? | Q3.1 |
| What happens if TVALID is HIGH but TREADY is LOW? | Q3.1 |
| What happens if TREADY is HIGH but TVALID is LOW? | Q3.1 |
| What is backpressure in AXI-Stream, and why is it important? | Q3.2 |

## Q5.2  What are the AXI-Stream signals: TDATA, TKEEP, TSTRB, TLAST, TUSER, TID, TDEST?

D  They are the payload and sideband signals of a stream. Only TVALID/TREADY control the flow.

P  Sideband signals let the receiver understand byte validity, packet boundaries, and routing.

W  TDATA: payload bytes. TKEEP: marks bytes that are part of the stream (null bytes are removed). TSTRB: marks data bytes versus position bytes.

•  TLAST: last beat of a packet. TUSER: user sideband, for example an error flag. TID / TDEST: stream identifier and routing destination.

•  TKEEP on the last beat plays the same role as EMPTY in Avalon-ST.

E  A 100-byte packet on a 64-byte bus: beat 2 has TLAST = 1 and TKEEP with 36 ones.

Interview line:  “TKEEP tells me which bytes of a beat are real, and I use it on the last beat to know the exact packet length.”

Follow-up questions to expect

- Which signals are mandatory? — TVALID and TREADY (TREADY may be omitted if the slave is always ready). The rest are optional.

## Q5.3  Why do we need TLAST, and what does it mean when TVALID, TREADY, and TLAST are all 1?   MUST KNOW

D  TLAST indicates that the current transfer is the last beat of the packet.

P  It lets the receiver find the end of a packet without counting bytes.

W  TLAST is only meaningful while TVALID is high. When TVALID = 1, TREADY = 1, and TLAST = 1 on the active edge, the beat is transferred and the packet is complete.

E  Handshake + last beat → packet completes; the receiver can update its packet counter.

Interview line:  “TLAST marks the end of a packet. When the handshake happens with TLAST high, that beat is the last beat of the packet.”

Follow-up questions to expect

- Can a packet have only one beat? — Yes. TLAST is high on its first and only beat.

## Q5.4  What is the difference between a data beat and a packet?   MUST KNOW

D  A beat is one data transfer on the interface. A packet is one or more beats.

P  It explains how a packet is split across a bus that is narrower than the packet.

W  The bytes in one beat depend on the data-bus width. The number of beats depends on the packet size. TLAST (or EOP) is asserted on the last beat.

E  On a 512-bit interface one beat carries 64 bytes. A 256-byte packet needs four beats, with TLAST on the fourth.

Interview line:  “I go from bus width to bytes per beat, then to number of beats, and assert TLAST on the last one.”

## Q5.5  Explain the complete data flow in your AXI-Stream design.

D  The design streams packet data from a transmitter to a receiver using AXI-Stream.

P  It gives both data transfer and flow control between the two blocks.

W  The transmitter drives TDATA, TVALID, and TLAST. The receiver drives TREADY. When both TVALID and TREADY are high on the same clock edge, a beat is transferred. TLAST marks the last beat.

E  Transmitter → TDATA / TVALID / TLAST → Receiver, with TREADY coming back.

Interview line:  “The transmitter sends beats. The receiver controls the rate with TREADY. TLAST tells the receiver where each packet ends.”

## Q5.6  What are the five channels of AXI4 (memory-mapped)?   ADVANCED

D  AXI4 has five independent channels: write address (AW), write data (W), write response (B), read address (AR), read data (R).

P  Independent channels let reads and writes run in parallel and allow pipelined, high-speed transfers.

W  Each channel has its own VALID/READY handshake. A write needs AW and W, then a B response. A read needs AR, then R data (with RLAST at the end of a burst).

E  A 16-beat write burst: one AW transfer, 16 W beats (WLAST on the 16th), one B response.

Interview line:  “Each AXI channel has its own valid-ready handshake, so address, data, and response can be pipelined independently.”

Follow-up questions to expect

- Do read and write use the same channels? — No. They are separate, which allows simultaneous read and write.

## Q5.7  AXI4 vs AXI4-Lite vs AXI-Stream — what is the difference?

D  AXI4 is memory-mapped with bursts. AXI4-Lite is a simple memory-mapped version with single transfers. AXI-Stream has no address.

P  The right one depends on the data: bulk memory access, control registers, or a continuous stream.

W  AXI4: address + burst up to 256 beats. AXI4-Lite: no bursts, fixed 32/64-bit, used for register access. AXI-Stream: no address, unlimited packet length, TLAST for boundaries.

E  A DMA reads memory with AXI4, gets configured over AXI4-Lite, and sends packets out over AXI-Stream.

Interview line:  “I use AXI4-Lite for control registers, AXI4 for memory bursts, and AXI-Stream for continuous data.”

Follow-up questions to expect

- Avalon-MM vs AXI? — Avalon-MM is the Intel (Altera) memory-mapped interface with fewer channels; AXI has independent read and write channels.

## Q5.8  What are AXI bursts, outstanding transactions, and IDs?   ADVANCED

D  A burst is many data beats after one address. Outstanding means more requests are issued before earlier ones finish. IDs tag transactions so responses can be matched.

P  They improve bus utilization and hide memory latency.

W  Burst length (AxLEN), size (AxSIZE), and type (FIXED, INCR, WRAP) describe the burst. With IDs, a slave can return responses out of order across different IDs, but transactions with the same ID stay in order.

E  Issue 4 reads back-to-back with different IDs, and receive data as each memory bank responds.

Interview line:  “Outstanding transactions and IDs let the master keep the bus busy while waiting for memory latency.”

Follow-up questions to expect

- What is the 4 KB rule? — A burst must not cross a 4 KB address boundary.

- What are the response codes? — OKAY, EXOKAY, SLVERR, DECERR.

# PART 6 — AVALON-ST PROJECT (MASTER → FIFO → SLAVE)

This part follows your documented project. Numbers such as 512-bit DATA, 6-bit EMPTY, 521-bit FIFO word, and the 350-cycle DONE delay come from your documents. Change them if your final RTL is different. The last group of questions (project defense) now has real model answers. Adjust them to match your own code.

## A. The Avalon-ST protocol

## Q6.1  What is Avalon-ST?   MUST KNOW

D  Avalon-ST is a streaming interface between a source and a sink, using VALID and READY.

P  It gives controlled streaming data transfer with packet support.

W  The source drives DATA, VALID, SOP, EOP, and EMPTY. The sink drives READY. A transfer happens on VALID && READY. Packet boundaries use SOP and EOP. Unused bytes on the last beat use EMPTY.

E  The Master sends packets to the Slave through the FIFO.

Interview line:  “Avalon-ST uses the same valid-ready idea as AXI-Stream. The difference is that packets use SOP and EOP, and EMPTY marks the unused bytes on the last beat.”

Follow-up questions to expect

- AXI-Stream vs Avalon-ST? — TLAST vs EOP; TKEEP vs EMPTY; Avalon-ST adds SOP.

- What is ready latency? — The number of cycles between READY and when the source may respond. In this design it is treated as zero.

## Q6.2  What are the Avalon-ST signals used in your design?

D  The main signals are DATA, VALID, READY, SOP, EOP, and EMPTY.

P  They carry the payload, the handshake, the packet boundaries, and the number of unused bytes.

W  DATA (512 bits): the payload. VALID: the source is presenting a valid beat. READY: the sink can accept.

•  SOP / EOP: first / last beat of the packet. EMPTY (6 bits): unused bytes on the final beat.

•  All signals travel from the Master to the Slave. Only READY goes backward, as backpressure.

E  A beat is accepted when VALID = 1 and READY = 1 (see the handshake question in Part 3).

Interview line:  “DATA carries the payload, VALID and READY control the transfer, SOP and EOP give packet boundaries, and EMPTY gives the byte count on the last beat.”

## Q6.3  Explain the SOP / EOP combinations.   MUST KNOW

D  SOP means Start of Packet. EOP means End of Packet. Together they identify the position of a beat.

P  They let the receiver tell first, middle, last, and single-beat conditions.

W  SOP = 1, EOP = 0 → first beat. SOP = 0, EOP = 0 → middle beat. SOP = 0, EOP = 1 → last beat. SOP = 1, EOP = 1 → single-beat packet.

E  The Slave FSM uses these to start and finish packet reception.

Interview line:  “SOP and EOP give packet boundaries, while VALID and READY control the actual transfer.”

## Q6.4  What is EMPTY and why is it needed?   MUST KNOW

D  EMPTY is the number of unused bytes on the final data beat.

P  It is needed when the packet size is not an exact multiple of the bus width.

W  It is carried with EOP. The receiver computes valid bytes on the last beat as BUS_BYTES − EMPTY. It is meaningful only on the EOP beat.

E  A 100-byte packet on a 64-byte bus: beat 1 has 64 bytes; beat 2 has EOP with EMPTY = 28, so 36 valid bytes.

Interview line:  “EMPTY tells the slave how many bytes on the last beat are not real, so the byte count is exact.”

Follow-up questions to expect

- Why is EMPTY 6 bits? — A 64-byte bus needs values 0 to 63, which is $clog2(64) = 6 bits.

These Avalon-ST questions are answered once, in earlier parts:

| Question (already answered, so not repeated) | Answered in |
| --- | --- |
| When does an Avalon-ST transfer occur? (VALID / READY combinations 00, 01, 10, 11) | Q3.1 |
| What is backpressure in your AVST design? | Q3.2 |
| What is a data beat, and what is a packet? | Q5.4 |

## B. Your project design

## Q6.5  What is the objective of your project, and how do you explain it in one minute?   MUST KNOW

D  The project is an Avalon-ST Master–FIFO–Slave communication system in Verilog HDL.

P  It generates streaming packets, buffers them through a FIFO, and receives and checks them at the Slave, with flow control.

W  The Master generates DATA, VALID, SOP, EOP, and EMPTY. The Write Wrapper packs these into a 521-bit word. The FIFO stores the stream. The Read Wrapper unpacks it for the Slave. READY provides backpressure. The Slave FSM receives packets and counts bytes and packets.

E  Path: Master → FIFO Write Wrapper → FIFO IP → FIFO Read Wrapper → Slave. Simulation checks handshake, packet boundaries, FIFO operation, and counters.

Interview line:  “I built an Avalon-ST Master–FIFO–Slave system. The master creates packets, a FIFO buffers them, and the slave checks packet and byte counts, while READY gives backpressure. I verified it with random READY and end-to-end counters.”

Follow-up questions to expect

- What was your role? — Say clearly which blocks you wrote and tested.

- What was the hardest bug? — Prepare one story (see Q13.x scenarios).

## Q6.6  What is the architecture of your design, and what are the main modules?

D  The design has five blocks: Master, FIFO Write Wrapper, FIFO IP, FIFO Read Wrapper, and Slave.

P  Each block has one job, so the design is modular and easy to debug.

W  Master: generates packets with an FSM (IDLE, SEND, DONE) and counters.

•  Write Wrapper: packs fields into the FIFO word and gives READY back to the Master.

•  FIFO IP: stores the stream (standard FIFO, one-cycle read latency).

•  Read Wrapper: reads the FIFO and unpacks the fields for the Slave.

•  Slave: FSM (IDLE, RECEIVE, DONE) that counts bytes and packets.

E  Master → Write Wrapper → FIFO → Read Wrapper → Slave.

Interview line:  “I split the design into master, write wrapper, FIFO, read wrapper, and slave, so each block can be tested alone.”

## Q6.7  Explain your Master FSM.   MUST KNOW

D  The Master FSM controls packet generation and transmission.

P  It sequences waiting, sending, and finishing.

W  IDLE: wait for READY = 1, then go to SEND.

•  SEND: send beats. If READY = 0, stay and stall. After the last beat of the last packet, go to DONE.

•  DONE: wait for the delay counter (350 in your design), reset the counters, and return to IDLE.

E  IDLE –(ready)→ SEND –(last beat of last packet)→ DONE –(delay done)→ IDLE.

Interview line:  “The master waits for ready in IDLE, streams beats in SEND while respecting backpressure, and waits a fixed delay in DONE before starting again.”

Follow-up questions to expect

- Why start only when READY is high? — It confirms the write path can accept the stream before the first beat.

- Why a delay in DONE? — To give the read side time to drain and to space the runs.

## Q6.8  How does READY affect the Master, and where does backpressure enter your design?   MUST KNOW

D  READY LOW means the receiving side cannot accept the current beat.

P  The Master must not advance as if the beat had been accepted.

W  Backpressure starts at the FIFO. When the FIFO is almost full, the Write Wrapper drives READY low. In SEND the Master keeps DATA, VALID, SOP, EOP, and EMPTY stable and does not increment the beat or packet counters. When READY returns, the same beat is transferred.

E  FIFO almost full → READY low → Master stalls in SEND.

Interview line:  “Backpressure enters at the write wrapper. The FIFO almost-full flag lowers READY, and the master holds the current beat without advancing any counter.”

## Q6.9  What are the Master counters used for?

D  The counters track beat and packet progress and the delay after a run.

P  They decide when to continue sending and when the final packet has completed.

W  A beat counter finds the last beat of a packet (and sets SOP/EOP). A packet counter finds the last packet of the run. A delay counter (up to 350) times the DONE state. They advance only on a real transfer (VALID && READY) and are cleared at the end of the run.

E  Beats per packet = ceil(packet bytes / 64) for a 512-bit bus.

Interview line:  “My counters advance only when a beat is accepted, so a stall cannot change the count.”

## Q6.10  Why do you use a FIFO in the design?

D  The FIFO is a buffer between the Master and the Slave paths.

P  It separates producer and consumer and absorbs flow-control differences.

W  The Write Wrapper stores the stream, the FIFO holds it, and the Read Wrapper reads it back out. If the Slave slows down, the FIFO fills and READY drops, so nothing is lost.

E  Master → Write Wrapper → FIFO IP → Read Wrapper → Slave.

Interview line:  “The FIFO decouples the master from the slave, so short stalls on the slave side do not stop the whole data path immediately.”

## Q6.11  Why use a FIFO IP instead of designing a FIFO manually?   ADVANCED

D  A FIFO IP is a ready, verified block from the tool vendor.

P  It saves design and verification time and maps well to the device’s block RAM.

W  IP blocks are tested, optimized for the FPGA, and offer options such as depth, width, almost-full thresholds, and FWFT mode. A manual FIFO is needed when you need special behavior, or when IP licensing or portability is a problem.

E  In the project, the IP gave a standard FIFO with one-cycle read latency, and the wrappers adapt the Avalon-ST fields to it.

Interview line:  “I used the FIFO IP because it is verified and maps directly to block RAM. I wrote the wrappers because the protocol handling is project-specific.”

Follow-up questions to expect

- Can you write a FIFO yourself? — Yes. See the tested sync and async FIFO in Part 12.

## Q6.12  What are the roles of the FIFO write wrapper and read wrapper?

D  The write wrapper prepares the stream for storage. The read wrapper retrieves the stored word and turns it back into stream signals.

P  They isolate the Avalon-ST protocol from the FIFO IP.

W  Write side: pack DATA, VALID, SOP, EOP, EMPTY into one word, write only when not full, and drive READY (low when almost full).

•  Read side: read only when not empty, handle the one-cycle read latency, unpack the fields, and hold the output when downstream READY is low.

E  The packed format is {EMPTY, EOP, SOP, VALID, DATA}.

Interview line:  “The wrapper is where the protocol meets the FIFO. It packs the beat, controls the write enable, and generates the backpressure.”

## Q6.13  Why is the FIFO word 521 bits wide, and what is the packing format?

D  The FIFO word combines the 512-bit payload with the control fields.

P  Storing them together keeps the payload and its packet metadata aligned.

W  512 (DATA) + 1 (VALID) + 1 (SOP) + 1 (EOP) + 6 (EMPTY) = 521 bits. Format: {EMPTY[5:0], EOP, SOP, VALID, DATA[511:0]}.

E  Bits [511:0] = DATA, [512] = VALID, [513] = SOP, [514] = EOP, [520:515] = EMPTY.

Interview line:  “I pack the control bits with the data so that SOP, EOP, and EMPTY always stay with the beat they belong to.”

## Q6.14  What happens when the FIFO is almost full, full, or empty?

D  Almost full means little space is left. Full means no space. Empty means no data to read.

P  These states protect the FIFO from overflow and underflow.

W  Almost full: the write side drives READY low early, so the Master stalls before an overflow can happen.

•  Full: no further write is accepted; backpressure continues.

•  Empty: the Read Wrapper does not read and does not generate a valid output until data arrives.

E  FIFO almost full → READY low → Master stalls. FIFO empty → read enable blocked.

Interview line:  “Almost-full protects the write side, and empty protects the read side. Together they keep the stream lossless.”

## Q6.15  What is the purpose of the FIFO read FSM (egress control)?   ADVANCED

D  The read-side FSM controls when stored data is read and presented to the Slave.

P  It makes sure data is sent only when valid FIFO data exists and the downstream side can accept it.

W  The FSM checks data availability (!empty) and readiness, asserts read enable, waits for the one-cycle latency, drives valid/data/SOP/EOP/EMPTY, and holds the output while downstream READY is low. States: IDLE, CHECK, READ & SEND, WAIT NEXT.

E  rd_en at cycle N → data at cycle N+1 → output valid at N+1.

Interview line:  “The read FSM handles the FIFO’s one-cycle read latency and holds the output stable whenever the slave is not ready.”

## Q6.16  Explain your Slave FSM and how it counts bytes and packets.

D  The Slave FSM controls packet reception and keeps statistics.

P  It separates waiting for a packet, receiving beats, and finishing the packet.

W  IDLE: wait for a valid first beat (VALID, READY, SOP). SOP = 1 and EOP = 0 goes to RECEIVE. SOP = 1 and EOP = 1 is a single-beat packet.

•  RECEIVE: accept middle beats. Move to DONE when a valid beat with EOP arrives.

•  DONE: update the packet counter and return to IDLE.

•  Bytes: add 64 for each full beat. On the EOP beat add 64 − EMPTY. Packets: increment on each accepted EOP beat.

E  A 100-byte packet: 64 + (64 − 28) = 100 bytes, packet count + 1.

Interview line:  “The slave counts full beats as 64 bytes, and on the last beat it subtracts EMPTY, so the byte count is exact.”

## Q6.17  Explain the end-to-end packet flow in your project.

D  A packet flows from the Master through the FIFO path to the Slave.

P  This gives streaming transfer, buffering, and controlled reception.

W  The Master sends AVST fields. The Write Wrapper packs them into a 521-bit word. The FIFO stores the word. The Read Wrapper reads and unpacks it. The Slave receives the packet and updates counters.

E  Master → Write Wrapper → FIFO IP → Read Wrapper → Slave.

Interview line:  “Beats go in through the write wrapper, wait in the FIFO if needed, come out through the read wrapper, and the slave counts them.”

## Q6.18  How do you verify the design, and which waveforms do you check?   MUST KNOW

D  I verify with a testbench that drives packets and random READY, then checks counters and waveforms.

P  I need proof that no beat is lost, duplicated, or reordered and that backpressure works.

W  Compare packets and bytes sent with packets and bytes received.

•  Toggle READY randomly, including in the middle of a packet, and check that DATA / VALID / SOP / EOP / EMPTY stay stable.

•  Fill the FIFO to almost-full and full, and drain it to empty.

•  Check SOP / EOP order and EMPTY on the last beat, for sizes smaller than, equal to, and larger than one beat.

•  Waveforms: VALID / READY, SOP / EOP / EMPTY, FIFO wr_en / rd_en / full / empty, and the byte and packet counters.

E  Assertion: if VALID and not READY, then next cycle VALID and DATA must be unchanged.

assert property (@(posedge clk) disable iff (!rst_n)

(valid && !ready) |=> (valid && $stable(data)));

Interview line:  “I check end-to-end packet and byte counts, then stress backpressure with random READY and packet sizes at the boundaries.”

## Q6.19  What are the advantages of your design, and why is it modular?

D  It is a modular FIFO-based streaming architecture.

P  It gives controlled packet transfer, buffering, scalable packet size, and reusable RTL.

W  The FIFO absorbs flow-control conditions. Wrappers separate the protocol from the storage. Separate Master and Slave FSMs keep the control simple. Each block can be reused or replaced alone.

E  To change the FIFO depth or IP, only the wrappers need attention.

Interview line:  “Modularity lets me test each block separately and replace the FIFO without touching the protocol logic.”

## Q6.20  Where are Avalon-ST streaming architectures used?

D  Avalon-ST suits streaming data paths in FPGA systems.

P  Streaming interfaces are useful when data must move continuously between processing blocks.

W  A source generates a stream and a sink consumes it with flow control. Typical uses: network packet processing, video and audio pipelines, DMA engines, signal-processing chains.

E  Ethernet MAC → packet parser → DMA → memory.

Interview line:  “Any place where data flows continuously between blocks with flow control: networking, video, DMA, and DSP.”

## C. Project defense — model answers

The questions below are the ones an interviewer asks to check that the project is really yours. The earlier version of this section had only a blank template. These are model answers built from the documented design. Replace details with the exact behavior of your RTL.

## Q6.21  Explain your AVST packet generator from input to output.   MUST KNOW

D  The packet generator is the Master block. It creates Avalon-ST beats for a packet of a given size.

P  It feeds and tests the streaming path with known packet sizes and boundaries.

W  Input: a start condition, READY from the sink, packet size and packet count settings.

•  It computes the number of beats from the packet size and bus width, and the EMPTY value for the last beat.

•  In SEND it drives DATA, VALID, SOP (first beat), EOP (last beat), and EMPTY (last beat only).

•  It advances the beat counter only when VALID && READY.

•  Output: a stream of beats with correct SOP / EOP / EMPTY. The tested generator code is in Part 12.

E  A 20-byte packet on an 8-byte bus: 3 beats; beat 1 SOP; beat 3 EOP with EMPTY = 4.

Interview line:  “Start moves the FSM to SEND. Each accepted beat advances the counter. SOP is on the first beat, EOP and EMPTY on the last.”

## Q6.22  Why did you use an FSM in the packet generator?

D  The generator has a clear sequence of phases: wait, send, finish.

P  An FSM makes the control easy to follow, verify, and change.

W  IDLE waits for start and READY. SEND holds the beat until it is accepted. DONE clears counters and waits the delay. Each phase has clear entry and exit conditions, so backpressure and end-of-packet handling are simple to add.

E  Without an FSM the same logic becomes a tangle of flags and counters.

Interview line:  “The generator is a step-by-step control problem, so an FSM gives clear states and safe transitions, and it is easy to debug in a waveform.”

These project-defense questions were already answered above or in earlier parts. They are listed here so you can find them, but they are not repeated:

| Question (already answered, so not repeated) | Answered in |
| --- | --- |
| How did you calculate the number of beats from packet size and bus width? | Q4.10 |
| How did you handle a packet size that is not an exact multiple of bus width? | Q6.4 |
| Why can a large FIFO depth help absorb burst traffic? | Q4.3 |
| What happens when TVALID is high but TREADY is low? | Q3.1 |
| How did you verify SOP, EOP, VALID, READY and EMPTY? | Q6.18 |
| Why is an asynchronous FIFO different from your normal synchronous FIFO? | Q4.2 |
| How does READY affect your packet generator / where does backpressure enter? | Q6.8 |

## Q6.23  Why might a dual-FIFO architecture be useful for separate read and write paths?   ADVANCED

D  A dual-FIFO architecture uses one FIFO for the write (ingress) path and another for the read (egress) path.

P  It keeps two traffic directions independent, so a stall in one direction does not block the other.

W  Each direction has its own buffer, depth, and backpressure. Head-of-line blocking is avoided. The cost is more memory and more control logic. Each FIFO can also sit in its own clock domain if needed (then use async FIFOs).

E  Receive packets go through one FIFO. Transmit packets go through another. A slow transmit path does not stop receive traffic.

Interview line:  “Separate FIFOs give independent flow control for each direction, at the cost of extra memory.”

Follow-up questions to expect

- Any downside? — Extra area, and two sets of flags to manage.

## Q6.24  Explain your AXI-Stream receiver or packet path.

D  The receiver accepts AXI-Stream beats, uses TREADY for flow control, and finds packet boundaries with TLAST.

P  It lets the receiving block process whole packets without losing data.

W  The receiver drives TREADY based on buffer space. On each accepted beat (TVALID && TREADY) it stores the data and sideband. On TLAST it closes the packet, updates the packet counter, and marks the packet complete. Byte count uses TKEEP on the last beat.

E  TREADY low when the input FIFO is almost full.

Interview line:  “My receiver takes a beat only on VALID and READY, stores data and sideband together, and finishes the packet on TLAST.”

## Q6.25  What lint issues did you find in the project, and how did you fix them?   MUST KNOW

D  These are typical findings when linting a streaming design like this one.

P  Lint finds coding problems before simulation and synthesis, and shows you understand code quality.

W  Width mismatch: a byte counter compared with a 32-bit constant, or EMPTY sized wrongly. Fixed by sizing counters with $clog2 and matching widths (see Q8.14).

•  Unused / undriven signals: spare sideband bits never read. Fixed by removing them or tying them off on purpose.

•  Latch risk: FSM output not assigned in every state. Fixed with default assignments (see Q8.5).

•  Positional port connections on the FIFO instance. Fixed by named connections (see Q8.23).

E  Before: if (cnt == 9'd350) with an 8-bit cnt. After: cnt widened to 9 bits.

Interview line:  “Lint found width mismatches, unused sideband signals, and a possible latch in an FSM output. I fixed each one at the root cause and re-ran lint clean. I did not waive anything without a written reason.”

Follow-up questions to expect

- Did you waive any warning? — Only with a reason, for example an intentionally unused output, and the reviewer agreed.

## Q6.26  Explain one timing issue in your design and how you would debug it.   ADVANCED

D  A typical case is a setup violation on a long path from FIFO status flags to READY, or on a wide 512-bit data path.

P  Timing closure is part of every FPGA project, and interviewers want to see a structured method.

W  Open the timing report: read the slack, startpoint, endpoint, and the path type (setup / hold).

•  Look at the data-path delay: many logic levels or a high fanout net (for example one enable driving 521 flops).

•  Fix at the root: register READY (skid buffer), duplicate the high-fanout register, pipeline the wide data path, or simplify the compare logic.

•  Re-run implementation and check that slack is positive and no new hold violation appears.

E  A full compare feeding READY and then the Master’s valid logic: register the almost-full flag first.

Interview line:  “I read the report to find the worst path, then fix it at the cause: fanout, logic depth, or a long control path. Then I confirm setup and hold again.”

Follow-up questions to expect

- See the STA method in Q9.18 and the fixes in Q9.19.

## Q6.27  Explain one CDC boundary in your design and the technique used there.   ADVANCED

D  A CDC boundary is a place where a signal crosses from one clock domain to another.

P  Interviewers check whether you can name the crossing and pick the right technique.

W  First, be honest: if the whole project runs on one clock, say so. Then explain how you *would* cross if the Master and Slave were on different clocks.

•  The stream would cross through an asynchronous FIFO (Gray-coded pointers, 2-FF synchronizers).

•  A single-bit status (for example “run done”) would use a 2-FF synchronizer; a one-cycle start command would use a toggle or handshake synchronizer.

•  The reset would be released with a reset synchronizer in each domain.

E  Master at 250 MHz, Slave at 156 MHz: replace the synchronous FIFO IP with a dual-clock (asynchronous) FIFO.

Interview line:  “My project runs on a single clock. If I split it into two clock domains, the stream crossing would use a dual-clock FIFO, and the control bits would use synchronizers.”

Follow-up questions to expect

- See the technique table in Q7.13.

# PART 7 — CLOCK DOMAIN CROSSING (CDC) AND RESET CROSSING

Pick the technique by what you are sending. Single-bit level → 2-FF synchronizer. Single pulse → toggle synchronizer. Slow control word that must not be lost → handshake. Continuous multi-bit stream → asynchronous FIFO. Learn the table in the question “How do you choose a CDC technique?”.

## Q7.1  What is CDC — Clock Domain Crossing?   MUST KNOW

D  CDC means transferring a signal or data from one clock domain to another clock domain.

P  We use CDC techniques to transfer information safely between different clocks and to avoid metastability and data corruption.

W  If the clocks are asynchronous, the receiving flip-flop may sample the signal while it is changing. This can cause metastability. For a single-bit signal we commonly use a 2-FF synchronizer. The first flip-flop may become metastable. The second flip-flop gives more time for the signal to settle before the destination logic uses it.

E  A status bit from clk_a used in clk_b is a single-bit CDC. A packet stream between unrelated clocks uses an async FIFO.

always @(posedge clk_b) begin

sync_ff1 <= signal_async;

sync_ff2 <= sync_ff1;

end

Interview line:  “For a single-bit CDC signal, I normally use a two-flop synchronizer to reduce the probability of metastability propagating into the destination domain.”

Follow-up questions to expect

- Are two clocks with the same frequency always synchronous? — No. They must come from the same source with a known, constrained phase relation.

- What CDC types exist? — Single-bit level, pulse, multi-bit data, and reset crossings.

- Do all crossings between two different clocks need CDC logic? — Yes, unless the clocks are related and the timing is constrained and verified by STA.

## Q7.2  What is metastability?   MUST KNOW

D  Metastability is an unstable output state of a flip-flop when its input changes too close to the clock edge (a setup or hold violation).

P  It matters because the uncertain result can travel into the destination logic and cause random failures.

W  The output hovers between 0 and 1 for a short time, then settles to 0 or 1, and we cannot know which. We cannot remove it. We can only give it time to resolve and reduce the chance that it reaches the next logic. The reliability is measured as MTBF.

E  A push-button signal sampled by a clock it is not related to.

Interview line:  “Metastability cannot be eliminated. A synchronizer gives the first flop time to settle, so the failure rate becomes extremely low.”

Follow-up questions to expect

- Why not use the first flop output directly? — It may be metastable, and it can fan out to several places that each see a different value.

- Why can’t we directly sample an asynchronous signal? — The sampling flop can violate setup or hold, so its output is unpredictable.

## Q7.3  What is MTBF, and what improves it?   ADVANCED

D  MTBF is Mean Time Between Failures of a synchronizer. It tells how often a metastable value would get through.

P  It lets you decide if two flops are enough or if you need three.

W  MTBF = e^(t_r / τ) / (T₀ × f_clk × f_data). t_r is the resolution time (about one clock period minus setup). τ and T₀ depend on the technology. Longer resolution time helps exponentially. Lower clock and data rates help linearly.

E  Using a slower destination clock, or adding a third flop, raises MTBF a lot. High-speed designs often use 3 flops.

Interview line:  “Resolution time is in an exponent, so one more flop or a slower clock raises MTBF dramatically.”

Follow-up questions to expect

- When do you use 3 flops? — Very high clock rate, long required life, or safety-critical use.

- Does logic between the two flops hurt? — Yes. It uses up the resolution time.

## Q7.4  What is a two-flop synchronizer?   MUST KNOW

D  It is two flip-flops in series, both clocked by the destination clock.

P  It brings a single-bit signal into a new clock domain safely.

W  Flop 1 may become metastable. It has a full clock period to settle before flop 2 samples it. Flop 2 is used by the destination logic. Rules: no logic between the two flops, no fan-out from flop 1, and the source signal should come from a flop.

E  Interrupt or done flag crossing into another clock domain.

(* ASYNC_REG = "TRUE" *) logic sync1, sync2;

always_ff @(posedge clk_dst or negedge rst_n)

if (!rst_n) {sync2, sync1} <= 2'b00;

else        {sync2, sync1} <= {sync1, sig_async};

Interview line:  “A two-flop synchronizer does not remove metastability. It gives the first stage more time to resolve, and greatly reduces the chance of it propagating.”

Follow-up questions to expect

- Should the source signal come from a flop? — Yes. A combinational source can glitch and be sampled wrongly.

- Is the reset needed on the synchronizer? — Optional. If used, keep it synchronous to the destination or asynchronous with proper release.

## Q7.5  Why do we mark synchronizer registers with ASYNC_REG?

D  ASYNC_REG is a Vivado attribute that tells the tool a register is part of a synchronizer.

P  It keeps the synchronizer flops close together and prevents optimizations that would hurt the settling time.

W  The tool places the flops in the same slice, with a short net between them. It also avoids merging, retiming, or moving logic between them. Simulation models may also propagate the metastability behavior when the attribute is set.

E  Attribute on both flops of the 2-FF synchronizer.

Interview line:  “I put ASYNC_REG on every synchronizer flop so placement keeps them together and the tool leaves them alone.”

## Q7.6  What is a single-bit level synchronizer, and can a 2-FF synchronizer carry a pulse?

D  A level synchronizer is a 2-FF synchronizer used for a signal that stays stable for several destination clocks.

P  It is the simplest and cheapest CDC for slow-changing status or enable bits.

W  The source level must stay stable for at least two destination clock periods (more in practice). A one-cycle pulse from a faster clock may be missed completely, so a 2-FF synchronizer is not safe for a short pulse. Use a toggle synchronizer, pulse stretching, or a handshake.

E  A "mode" bit changes rarely: a 2-FF synchronizer is fine. A "start" pulse: use a toggle synchronizer.

Interview line:  “A two-flop synchronizer is for levels that stay stable. For a short event I convert it into a level change first.”

## Q7.7  What is a toggle synchronizer (pulse CDC)?   MUST KNOW

D  A toggle synchronizer turns a pulse into a level change, synchronizes that change, and detects it in the destination domain.

P  It lets a short one-cycle event cross a clock domain without being missed.

W  Source domain: each pulse toggles a flop.

•  The toggle passes through a 2-FF synchronizer into the destination domain.

•  Destination: XOR of the synchronized toggle and its delayed copy gives a one-cycle pulse.

E  A "start" command from a fast domain to a slow domain. Tested code is in Part 12.

// Source domain

always_ff @(posedge clk_src) if (pulse_src) tog <= ~tog;

// Destination domain

always_ff @(posedge clk_dst) {s3, s2, s1} <= {s2, s1, tog};

assign pulse_dst = s2 ^ s3;

Interview line:  “Instead of synchronizing the pulse width, I synchronize a state change and detect the change in the destination domain.”

Follow-up questions to expect

- What is the limitation? — Events must be spaced far enough apart, otherwise two toggles can merge and one event is lost.

## Q7.8  What is the limitation of a toggle synchronizer, and what is pulse stretching?   ADVANCED

D  A toggle synchronizer needs a minimum time between events. Pulse stretching holds a pulse long enough for a slower clock to see it.

P  Both are simple pulse-CDC methods, but they only work with known timing between events.

W  If two source pulses arrive before the destination has seen the first toggle, the toggles cancel or merge. Fix by spacing the events, or add a busy / acknowledge (handshake). Pulse stretching keeps the pulse for at least two destination periods, then synchronizes it as a level. It needs a known clock ratio.

E  Fast source pulse (10 ns) stretched to 3 destination cycles at 25 MHz = 120 ns.

Interview line:  “Toggle and stretch methods assume events are spaced apart. If events can come back-to-back, I add a handshake.”

Follow-up questions to expect

- Is pulse stretching safe if the clock ratio changes? — No. It is tied to the ratio.

## Q7.9  What is a handshake synchronizer (req/ack CDC)?   MUST KNOW

D  A handshake CDC transfers data or an event using a request signal and an acknowledge signal.

P  It gives reliable transfer when the source must know that the destination received the data.

W  The source puts data on a bus and raises REQ, and holds the data stable.

•  REQ goes through a 2-FF synchronizer into the destination domain.

•  The destination captures the data and raises ACK.

•  ACK goes through a 2-FF synchronizer back to the source.

•  The source lowers REQ. The destination lowers ACK. The transfer is done.

E  Sending a configuration word to a slow domain: throughput is low, but it is safe and simple.

Interview line:  “The sender holds the request and the data stable until it sees the acknowledgement from the destination domain.”

Follow-up questions to expect

- Why is the data safe? — It does not change while REQ is high, and the destination reads it only after the synchronized REQ.

- What is the cost? — Several cycles of latency per transfer. It is not for high throughput.

## Q7.10  What is multi-bit CDC, and why is a 2-FF synchronizer not enough for a bus?   MUST KNOW

D  Multi-bit CDC is crossing a bus of several bits between clock domains.

P  All bits must belong to the same source value.

W  If each bit has its own 2-FF synchronizer, the bits can settle in different cycles. The destination sees a wrong mixed value. Safe methods: async FIFO, handshake with stable data, Gray code (for counters that change by one), or a synchronized single-bit enable that qualifies stable data.

E  A 4-bit counter changing from 0111 to 1000: separate synchronizers might show 1111 or 0000 for one cycle.

Interview line:  “I never synchronize a multi-bit binary bus bit by bit. I use a FIFO, a handshake, or Gray code when the value changes by one step.”

Follow-up questions to expect

- Give a safe multi-bit approach. — Hold the data stable in a register, synchronize a 1-bit enable, and capture the data in the destination when the synchronized enable arrives.

## Q7.11  What is a bundled-data CDC technique?   ADVANCED

D  Bundled data means a data bus travels with a control signal that arrives later than the data.

P  It allows a safe multi-bit transfer using only a single-bit synchronizer.

W  The source drives the data bus and keeps it stable. It then sends a "data valid" control through a 2-FF synchronizer. Because the data was stable before the control arrives, the destination can capture all bits together. The path from the data flops must be constrained (for example with set_max_delay -datapath_only).

E  A configuration word plus a "config_valid" toggle.

Interview line:  “The data is stable first. Only the one-bit control is synchronized, so the destination reads a coherent word.”

## Q7.12  What is a reconvergence CDC problem?   ADVANCED

D  Reconvergence is when two related signals are synchronized separately and then combined in the destination domain.

P  The two synchronized copies can arrive in different cycles, so the combined logic may see an illegal combination.

W  Because each synchronizer settles independently, a may change one cycle before b. Logic that uses both may glitch or take a wrong branch. Fix by sending both through one structure (a FIFO, a handshake, or one-hot encoding into one bus).

E  Two flags "busy" and "done" synchronized separately, then used in the destination FSM.

Interview line:  “If two signals belong together, they cross together, through one structure. Separate synchronizers can disagree for a cycle.”

## Q7.13  How do you choose a CDC technique?   MUST KNOW

D  Choose by the type of information and its rate.

P  Wrong choices cause lost data or wasted logic.

W  Ask three questions: is it one bit or many? Is it a level, a pulse, or a stream? Can the sender wait?

E  See the table below.

Interview line:  “The CDC technique must match the type and rate of the information being transferred.”

| What you send | Technique | Key point |
| --- | --- | --- |
| Slow single-bit level | 2-FF synchronizer | Signal must stay stable for a few destination clocks |
| One-cycle pulse / event | Toggle synchronizer | Turn the event into a level change; keep events spaced apart |
| Pulse, simple case | Pulse stretch + sync | Pulse must be long enough for the slower clock |
| Multi-bit control / config | Handshake (req/ack) or enable + stable data | Sender holds data until acknowledged |
| Continuous multi-bit data | Async FIFO | Gray-coded pointers |
| Counter value (one step) | Gray-coded bus + 2-FF | Only valid for +1 or -1 changes |
| Reset | Reset synchronizer | Assert async, release sync |

## Q7.14  What is the difference between pulse, toggle, and handshake synchronizers?

D  All three carry an event across domains, but with different safety and speed.

P  You should know which one fits the event rate.

W  Pulse stretch: simplest, needs a known clock ratio. Toggle: no ratio limit for a single event, but events must be spaced apart. Handshake: safest, the source knows it was received, but it is the slowest.

E  Use toggle for rare commands, handshake for critical configuration, stretch for simple fixed-ratio clocks.

Interview line:  “Stretch is simplest, toggle handles single events, and handshake gives guaranteed delivery at the cost of latency.”

## Q7.15  Why do we use an asynchronous FIFO for CDC?   MUST KNOW

D  An async FIFO is a dual-clock buffer that crosses multi-bit data safely.

P  It supports continuous streams and different clock rates, and it hides the CDC complexity behind a simple interface.

W  Data sits in dual-port memory. Only the read and write pointers cross clock domains, as Gray code through 2-FF synchronizers. Full is generated in the write domain and empty in the read domain.

E  A producer at 300 MHz sending packets to a consumer at 200 MHz. Tested code is in Part 12.

Interview line:  “The data goes through dual-port memory. Only the Gray-coded pointers cross the clock domains, each through a two-flop synchronizer.”

Follow-up questions to expect

- Why is empty compared in the read domain? — The read side knows its own pointer and gets a safe (delayed) write pointer.

## Q7.16  What is a binary pointer in a FIFO, and how are the write and read pointers used?

D  A binary pointer is a counter that points to a memory location. Async FIFOs keep one for the write side and one for the read side.

P  The binary pointer addresses memory. The Gray copy crosses the clock domain.

W  Write pointer: increments on each accepted write. Lower bits address the memory. It is converted to Gray and sent to the read domain, where it is used for empty.

•  Read pointer: increments on each accepted read. Lower bits address the memory. It is converted to Gray and sent to the write domain, where it is used for full.

E  4-bit deep FIFO with 16 words: pointers are 5 bits (4 address bits + 1 wrap bit).

Interview line:  “I use the binary pointer for memory addressing and the Gray pointer for crossing. Each side sees a delayed copy of the other side’s pointer.”

## Q7.17  What is Gray code, and why is it used for async FIFO pointers?   MUST KNOW

D  Gray code is a number code where consecutive values differ by only one bit.

P  In an async FIFO the pointers change while the other clock samples them. With Gray code only one bit changes at a time, so the sampled value is either the old or the new pointer, never a random value.

W  Keep a binary pointer for addressing. Convert to Gray for the crossing with gray = bin ^ (bin >> 1). Synchronize the Gray value into the other domain. Use the synchronized Gray pointer only for full and empty comparison.

E  Binary 3 → 4 is 011 → 100 (three bits change). Gray 3 → 4 is 010 → 110 (one bit changes).

assign gray = (bin >> 1) ^ bin;          // binary to Gray

always_comb begin                          // Gray to binary

b[W-1] = g[W-1];

for (int i = W-2; i >= 0; i--) b[i] = b[i+1] ^ g[i];

end

Interview line:  “Gray pointers change only one bit per increment, so a pointer sampled by the other clock is at worst one step old and never garbage.”

Follow-up questions to expect

- Does Gray code work for any jump? — No. Only for +1 steps, like FIFO pointers.

- Does the FIFO depth need to be a power of two? — For the standard Gray-pointer design, yes. Otherwise the Gray sequence is not cyclic.

## Q7.18  Why should we not directly synchronize a binary FIFO pointer?

D  A binary pointer can change many bits at once.

P  If the other clock samples during the change, some bits may be old and some new. The result can be a completely wrong pointer, and the FIFO may think it is full or empty when it is not.

W  Example: 0111 → 1000 changes four bits. The synchronizers may capture 1111 or 0000 for one cycle. With Gray code only one bit flips, so the captured value is 0100 or 1100 (old or new).

E  Overflow or underflow can occur if a wrong pointer is used for full or empty.

Interview line:  “Binary increments can flip many bits, and asynchronous sampling of many changing bits is unsafe. Gray removes that risk.”

## Q7.19  Why is an extra pointer bit used in an asynchronous FIFO?   MUST KNOW

D  The extra bit is one more bit than the address needs.

P  When read and write address bits are equal, the FIFO can be empty or full. The extra bit tells them apart.

W  Pointers have AW+1 bits. The lower AW bits address the memory. Empty: all bits equal. Full: address bits equal but the MSB differs (the writer has wrapped once more than the reader).

E  A depth-64 FIFO needs 6 address bits and a 7-bit pointer. Depth 8 → pointer 4 bits: wptr = 1000 and rptr = 0000 means full.

Interview line:  “Equal addresses alone cannot tell me whether the FIFO is empty or full. The extra wrap bit gives that information.”

Follow-up questions to expect

- Alternative to the extra bit? — Keep a count, but a count is hard to cross between clocks.

## Q7.20  How is FIFO empty detected in an asynchronous FIFO?

D  Empty means the read side has no unread data.

P  The read controller must prevent an invalid read.

W  In the read clock domain, compare the read Gray pointer (next value) with the synchronized write Gray pointer. If they are equal, the FIFO is empty.

E  rempty = (rgray_next == wq2_wgray);

wire rempty_v = (rgray_n == rq2_wgray);

always_ff @(posedge rclk or negedge rrst_n)

if (!rrst_n) rempty <= 1'b1;

else         rempty <= rempty_v;

Interview line:  “Empty is decided in the read clock domain using the synchronized write pointer. It resets to 1.”

## Q7.21  How is FIFO full detected in an asynchronous FIFO?

D  Full means the write side has no free entry.

P  The write controller must prevent an overflow.

W  In the write clock domain, compare the next write Gray pointer with the synchronized read Gray pointer, with the top two bits inverted. If they match, the FIFO is full.

E  wfull = (wgray_next == {~rq2_rgray[AW:AW-1], rq2_rgray[AW-2:0]});

wire wfull_v = (wgray_n == {~wq2_rgray[AW:AW-1], wq2_rgray[AW-2:0]});

always_ff @(posedge wclk or negedge wrst_n)

if (!wrst_n) wfull <= 1'b0;

else         wfull <= wfull_v;

Interview line:  “Full is decided in the write clock domain, using the synchronized read pointer and the wrap information.”

## Q7.22  Why are the upper two Gray-pointer bits inverted for full detection?   ADVANCED

D  It is the Gray-code version of "same address, different wrap bit".

P  In binary, full means only the MSB differs. We compare Gray pointers, so we need the Gray equivalent.

W  If two binary pointers differ only in the MSB, then in Gray code the MSB differs (it is copied from binary) and the second bit differs too (it equals MSB xor next bit). All the lower Gray bits are equal. So full = top two bits inverted, the rest equal.

E  Depth 8: binary 1000 vs 0000 → Gray 1100 vs 0000: top two bits differ, the rest match.

Interview line:  “When only the binary MSB differs, both of the top two Gray bits differ and the remaining bits are equal. That is why I invert the top two bits.”

## Q7.23  Why are async FIFO flags pessimistic (pointer synchronization latency)?   ADVANCED

D  Each side sees the other side’s pointer two or three clock cycles late.

P  It explains why the FIFO may show full slightly early and empty slightly late.

W  A pointer passes through a 2-FF synchronizer, so the other side sees an old value. The old write pointer makes empty stay high a little longer. The old read pointer makes full stay high a little longer. This is safe: the flags are conservative, so data is never lost or read early. It only costs a small loss of usable depth and a little latency.

E  Write a word, and the read side notices it 2–3 read clocks later.

Interview line:  “The flags are conservative: full can assert slightly early and empty can deassert slightly late. That protects the data, and only costs a little depth and latency.”

Follow-up questions to expect

- Does it lose data? — No.

- How do you compensate? — Add a little extra depth.

## Q7.24  What happens if the write clock is faster than the read clock, and the other way round?

D  The FIFO absorbs the rate difference until it fills or empties.

P  It shows that a FIFO gives elasticity, not unlimited buffering.

W  Write faster: occupancy increases. If the average write rate stays higher, FULL asserts and the writer must stop (backpressure). Read faster: occupancy decreases. EMPTY asserts and the reader waits.

E  Bursty writer at 300 MHz and steady reader at 200 MHz: the FIFO fills during bursts and drains between them.

Interview line:  “A FIFO absorbs short-term rate differences. A sustained difference will eventually fill it or empty it.”

## Q7.25  How do you size an asynchronous FIFO?   ADVANCED

D  Async FIFO depth must cover the worst-case burst, the clock ratio, and the pointer-synchronization delay.

P  A FIFO that is too shallow overflows. One that is too deep wastes memory.

W  Find the worst-case burst in write clocks and the minimum read rate during that burst.

•  depth ≥ burst × (1 − f_read / f_write) + margin, with f_read taking READY stalls into account.

•  Add margin for the pointer synchronization latency (about 3–4 cycles) and the almost-full reaction time.

•  Round up to a power of two.

E  Burst = 64 words at 300 MHz write. Read = 200 MHz (ratio 0.67). 64 × (1 − 0.67) ≈ 21 → + 8 margin = 29 → choose 32.

Interview line:  “I size depth from the burst length and clock ratio, then add margin for synchronizer latency and round up to a power of two.”

## Q7.26  What is Reset Domain Crossing (RDC), and why synchronize reset deassertion?   MUST KNOW

D  RDC concerns reset signals or reset release crossing between domains.

P  Releasing an asynchronous reset close to a clock edge can put flops into metastability, and different flops can leave reset in different cycles, so the design starts in an inconsistent state.

W  Assert asynchronously, release synchronously. A reset synchronizer in each clock domain produces a local reset that releases cleanly on that domain’s clock. Also make sure related blocks leave reset together (for example both sides of a FIFO).

E  Two clock domains each get their own reset synchronizer from the same external reset. Tested code is in Part 12.

Interview line:  “I synchronize the reset release in every clock domain, and I make sure related blocks come out of reset in a safe order.”

Follow-up questions to expect

- What if one domain is reset and its partner is not? — Partial transfers can be lost. Reset both sides, or use a handshake.

## Q7.27  What are recovery and removal checks?   ADVANCED

D  Recovery is the setup-like check for the release of an asynchronous reset. Removal is the hold-like check.

P  Reset release near a clock edge can leave flops metastable, so timing tools check it.

W  Recovery: reset must be released at least the recovery time before the next active clock edge. Removal: reset must stay asserted at least the removal time after the clock edge. A reset synchronizer meets both by releasing on a clock edge.

E  STA reports “recovery” and “removal” checks on every flop with an async reset.

Interview line:  “Reset release is a timing event. I use a reset synchronizer and check recovery and removal in the timing report.”

## Q7.28  What is CDC structural verification, and how do you verify CDC?   ADVANCED

D  Structural CDC verification is a static tool check that finds every clock crossing and tests whether it uses a safe structure.

P  CDC bugs are rare, random, and almost invisible in normal simulation.

W  Structural check (CDC tool, Vivado report_cdc): lists crossings, flags missing synchronizers, multi-bit crossings, reconvergence, and combinational logic before a synchronizer.

•  Simulation: run with different clock ratios and phase offsets, and check FIFO data and flags.

•  Constraints review: check set_clock_groups, set_max_delay -datapath_only, and false paths are used only on verified crossings.

E  Report: “unsynchronized crossing from clk_a to clk_b”. Fix with a proper synchronizer or FIFO.

Interview line:  “I check every crossing in the CDC report and confirm the right structure: synchronizer, handshake, or async FIFO.”

Follow-up questions to expect

- Does STA check CDC? — STA checks timing, not the correctness of the crossing. CDC tools check structure.

## Q7.29  How do you constrain CDC paths in Vivado?   ADVANCED

D  CDC paths need constraints that tell STA how to treat them: not as normal synchronous paths.

P  Without constraints the tool tries to time the crossing as if the clocks were related, and reports meaningless violations or misses real problems.

W  Options: set_clock_groups -asynchronous between unrelated clocks (removes all timing between them), set_false_path on a specific synchronizer input, or set_max_delay -datapath_only to keep a crossing short (used on Gray pointers and bundled data). Prefer the bounded max-delay, because it still limits skew.

E  Gray pointer path limited to one source clock period.

set_clock_groups -asynchronous -group [get_clocks clk_a] -group [get_clocks clk_b]

set_max_delay -datapath_only -from [get_cells wptr_gray_reg*] \

-to [get_cells wq1_wptr_reg*] 5.0

Common mistake:  set_clock_groups disables all checks between the groups. If a crossing is unsafe, timing tools will not warn you.

Interview line:  “For CDC I prefer a bounded max-delay constraint over a blind false path, so the crossing skew stays controlled.”

## Q7.30  How do you implement CDC in Vivado using the built-in macros?   ADVANCED

D  Vivado provides XPM (Xilinx Parameterized Macros) for CDC, so you do not write synchronizers by hand.

P  They are tested, correctly constrained (the tool adds the needed timing exceptions), and portable inside the Xilinx flow.

W  xpm_cdc_single (single bit), xpm_cdc_pulse (pulse), xpm_cdc_handshake and xpm_cdc_array_single (buses with control), xpm_cdc_gray (Gray counters), xpm_cdc_async_rst (reset), and xpm_fifo_async (async FIFO).

E  Use xpm_fifo_async for a stream crossing and xpm_cdc_single for a status bit.

Interview line:  “In Vivado I use XPM CDC macros for standard crossings, because they include the right attributes and constraints. I write my own only when I need special behavior.”

## Q7.31  What are the most common CDC mistakes?

D  These are the errors that come up again and again in reviews.

P  Knowing them helps you find bugs quickly.

W  No synchronizer on a crossing signal.

•  Logic (or fanout) between the two synchronizer flops.

•  Synchronizing a multi-bit bus bit by bit.

•  A pulse too short for the destination clock.

•  Synchronizing two related signals separately (reconvergence).

•  Using a combinational signal as the source of a crossing (it can glitch).

•  Missing reset synchronizer in a domain.

•  Blindly using false paths or clock groups on unsafe crossings.

E  A “busy” flag from a combinational expression crossing directly into another clock domain.

Interview line:  “Most CDC bugs are one of these: a missing synchronizer, a bus crossed bit by bit, a pulse that is too short, or a wrong constraint.”

## Q7.32  How would you debug a CDC issue in an interview?

D  Use a fixed, step-by-step method, not guesses.

P  A clear method shows a 3+ years mindset.

W  1. List all clocks and their relationships.

•  2. List every signal that crosses from one domain to another.

•  3. Classify each crossing: level, pulse, bus, stream, or reset.

•  4. Check that each crossing uses the right structure (see the technique table).

•  5. Check constraints, and run the CDC tool.

•  6. Reproduce with different clock ratios, and add assertions on FIFO flags and data order.

E  A random data error that appears only at some temperatures: look for a multi-bit crossing without a handshake or FIFO.

Interview line:  “I list the clocks, list the crossings, classify each one, and check the technique and the constraints.”

# PART 8 — RTL LINT

Lint rule IDs below (W336, W18, …) follow SpyGlass-style numbering used in your documents. Other tools (Verilator, Ascent Lint, HAL, Questa Lint) use different IDs for the same ideas. Learn the idea first, the number second. Each rule appears once, with its mistake and its correction together.

## A. Lint fundamentals

## Q8.1  What is RTL linting, and why do we use it?   MUST KNOW

D  Lint is a static check of RTL code that finds bad coding style and hidden bugs without running simulation.

P  It catches problems early and cheaply: latches, width mismatch, multiple drivers, unconnected ports, unsafe sensitivity lists.

W  The tool reads the RTL and applies rules. Each finding has a severity (error, warning, info). You review each one, fix real bugs, and waive only with a written reason.

E  A missing else creates a latch. Lint reports it before synthesis.

Interview line:  “Lint is my first quality gate. I do not fix warnings blindly. I understand each one, fix the real bugs, and waive only with a reason.”

Follow-up questions to expect

- What is a lint violation? — One place where the code breaks a rule. It has a rule ID, a file and line, and a message.

- What is lint severity? — Error: must fix. Warning: review and usually fix. Info: style or note.

## Q8.2  Is lint the same as simulation or synthesis?

D  No. Lint is static checking. Simulation runs the design with test vectors. Synthesis builds hardware.

P  Each tool finds different bugs, so we need all three.

W  Lint checks code structure and style for any input. Simulation checks behavior only for the stimulus you give. Synthesis may accept risky code that lint would flag (for example a latch).

E  A width mismatch can pass simulation by luck and still be wrong. Lint flags it.

Interview line:  “Lint checks the code, simulation checks behavior, and synthesis checks that hardware can be built. I use all three.”

## Q8.3  What is the typical lint workflow, and should every warning be fixed?   MUST KNOW

D  Workflow: run lint → read violations → fix real bugs → waive only with a reason → re-run until clean.

P  A clean, documented lint run gives confidence before simulation and synthesis.

W  Sort by severity and by risk. Fix latches, multiple drivers, width mismatches, and undriven signals first. A warning may be a false alarm in rare cases (an intentionally unused output). Then write a waiver with the reason and get a reviewer to approve.

E  An intentionally unconnected debug output gets a waiver note.

Interview line:  “I fix by risk, not by count. Real bugs are fixed. Anything waived has a written reason.”

## B. Rule by rule — mistake and correction

## Q8.4  W336 — Blocking assignment in a sequential block   MUST KNOW

D  It flags blocking assignments (=) used in clocked blocks.

P  Clocked state should update together at the clock edge, without misleading simulation order.

W  Use non-blocking <= for registers in clocked blocks.

E  See the mistake and corrected code below.

// MISTAKE

always @(posedge clk) q = d;      // blocking in sequential logic

// CORRECTED

always @(posedge clk) q <= d;     // non-blocking

Interview line:  “Non-blocking in clocked blocks keeps simulation equal to hardware.”

## Q8.5  W18 — Inferred latch   MUST KNOW

D  It flags a latch inferred in the design.

P  An unintended latch usually means a combinational output was not assigned on every path.

W  Assign a default value, and cover all branches with else or default.

E  See the mistake and corrected code below.

// MISTAKE

always @(*) begin

if (en) y = a;               // y not assigned when en = 0

end

// CORRECTED

always @(*) begin

y = 1'b0;                    // default

if (en) y = a;

end

Interview line:  “A latch appears when a signal is not assigned on every path. I write the default at the top of the block.”

## Q8.6  W415 — Multiple drivers   MUST KNOW

D  A signal is driven from more than one place.

P  Two drivers can conflict and give x, or hide a real bug.

W  Give each signal one clear owner.

E  See the mistake and corrected code below.

// MISTAKE

always @(posedge clk) ready <= 1'b1;

always @(posedge clk) ready <= fifo_not_full;   // second driver

// CORRECTED

always @(posedge clk) ready <= fifo_not_full;  // one owner

## Q8.7  W415a — Multiple assignments in one always block

D  The same signal is assigned several times in one block.

P  The last assignment wins, which may hide the intent.

W  Structure the block with if/else so each path assigns once, or keep a deliberate default followed by a single override.

E  See the mistake and corrected code below.

// MISTAKE

always @(posedge clk) begin

state <= IDLE;

if (start) state <= SEND;

state <= IDLE;               // accidental overwrite

end

// CORRECTED

always @(posedge clk) begin

if (start) state <= SEND;

else       state <= IDLE;

end

## Q8.8  W120 — Declared but never used   MUST KNOW

D  A signal or variable is declared and never used.

P  It is dead code, or it may point to a typo.

W  Remove it, or connect it if the intent was to use it.

E  See the mistake and corrected code below.

// MISTAKE

logic unused_flag;               // never read

// CORRECTED

// removed, or used by the logic that needs it

## Q8.9  W528 — Set but never read   MUST KNOW

D  A signal is assigned, but nothing reads it.

P  The logic does nothing. It may be a missing connection.

W  Connect the signal to where it is needed, or remove the logic.

E  See the mistake and corrected code below.

// MISTAKE

always @(posedge clk) done_q <= done;   // done_q never used

// CORRECTED

assign done_out = done_q;               // now used

## Q8.10  W123 — Read but never set

D  A signal is read but nobody drives it.

P  It is floating. Simulation shows x or z.

W  Drive the signal from the correct source.

E  See the mistake and corrected code below.

// MISTAKE

assign out = in_a & enable;      // enable is never driven

// CORRECTED

assign enable = cfg_enable;      // now driven

assign out    = in_a & enable;

## Q8.11  W240 — Input port never read

D  A module input is not used inside the module.

P  It may mean a broken interface or wasted pins.

W  Use it, remove it, or document why it is unused.

E  See the mistake and corrected code below.

// MISTAKE

module blk (input clk, input spare, output y);   // spare unused

// CORRECTED

module blk (input clk, output y);                // removed

## Q8.12  W241 — Output port not completely set

D  Some or all bits of an output are never driven.

P  Undriven output bits cause x downstream.

W  Drive every bit of every output, even constants.

E  See the mistake and corrected code below.

// MISTAKE

output [7:0] data;

assign data[3:0] = lo;           // upper bits undriven

// CORRECTED

assign data = {8'h00 | hi, lo};  // all bits driven

## Q8.13  W110 — Module port width mismatch   MUST KNOW

D  The width of a signal connected to a port is different from the port width.

P  Bits may be silently truncated or padded.

W  Make the signal and the port the same width.

E  See the mistake and corrected code below.

// MISTAKE

wire [7:0] d8;

fifo #(.DW(16)) u (.din(d8));    // 8 bits into 16-bit port

// CORRECTED

wire [15:0] d16;

fifo #(.DW(16)) u (.din(d16));

## Q8.14  W164 — Assignment width mismatch   MUST KNOW

D  The left and right side of an assignment have different widths.

P  Upper bits can be lost, or the result padded with zeros, changing the behavior.

W  Size the destination for the result, or write the truncation explicitly.

E  See the mistake and corrected code below.

// MISTAKE

logic [3:0] y;

logic [7:0] x;

assign y = x;                    // upper 4 bits lost

// CORRECTED

assign y = x[3:0];              // intent is explicit

Interview line:  “In counters and stream buses, a width mismatch is a real functional bug, so I treat it as high priority.”

## Q8.15  W362 — Unequal widths in a comparison   MUST KNOW

D  A comparison uses operands of different widths.

P  The shorter operand is extended, which can make the comparison wrong.

W  Match widths on both sides.

E  See the mistake and corrected code below.

// MISTAKE

logic [7:0] cnt;

if (cnt == 9'd256) ...           // 8-bit cnt can never equal 256

// CORRECTED

logic [8:0] cnt;

if (cnt == 9'd256) ...

## Q8.16  W484 — Possible loss of carry or borrow

D  An addition or subtraction result may not fit in the destination.

P  The carry is lost and counters wrap silently.

W  Widen the result by one bit, or prove that overflow cannot happen.

E  See the mistake and corrected code below.

// MISTAKE

logic [7:0] a, b, sum;

assign sum = a + b;              // carry lost

// CORRECTED

logic [8:0] sum;

assign sum = {1'b0, a} + {1'b0, b};

## Q8.17  W486 — Shift overflow

D  A shift may push meaningful bits out of the result width.

P  Data bits are lost.

W  Make the operand wide enough before the shift.

E  See the mistake and corrected code below.

// MISTAKE

logic [7:0] y;

assign y = x << 4;               // top bits of x are lost

// CORRECTED

logic [11:0] y;

assign y = {4'b0, x} << 4;

## Q8.18  W263 — Case item width mismatch

D  The width of a case item is different from the case selector.

P  The branch may never match.

W  Write case items with the same width as the selector.

E  See the mistake and corrected code below.

// MISTAKE

case (sel)                       // sel is 3 bits

2'b11: ...                   // 2-bit label

endcase

// CORRECTED

case (sel)

3'b011: ...

endcase

## Q8.19  W551 — Case without default (intent not complete)   MUST KNOW

D  A case statement has no default or does not cover all values.

P  Incomplete cases create latches or unsafe states.

W  Add a default, or use unique case together with a default.

E  See the mistake and corrected code below.

// MISTAKE

case (state)

IDLE: next = SEND;

SEND: next = DONE;

endcase                          // other values: undefined

// CORRECTED

case (state)

IDLE:    next = SEND;

SEND:    next = DONE;

default: next = IDLE;

endcase

## Q8.20  W122 / W456 — Sensitivity list problems

D  W122: a signal that is read is missing from the sensitivity list. W456: an unnecessary signal is in the list.

P  Simulation then behaves differently from the synthesized hardware.

W  Use always_comb or always @(*) for combinational logic, and always_ff for clocked logic.

E  See the mistake and corrected code below.

// MISTAKE

always @(a)                      // b is read but not in the list

y = a & b;

// CORRECTED

always_comb y = a & b;

## Q8.21  W210 — Unconnected module port

D  A port of an instance is not connected.

P  A missing connection can leave a block without an input or hide an output you need.

W  Connect every port, or leave an output open on purpose and say so.

E  See the mistake and corrected code below.

// MISTAKE

fifo u0 (.clk(clk), .din(din));   // dout, full, empty not connected

// CORRECTED

fifo u0 (.clk(clk), .din(din), .dout(dout), .full(full), .empty(empty));

## Q8.22  W287a — Instance input not driven

D  An input of an instance has no driver.

P  It floats. The block receives x.

W  Drive the input, or tie it to a constant when it is not used.

E  See the mistake and corrected code below.

// MISTAKE

my_blk u (.en(en_unconnected));

// CORRECTED

wire en_q = 1'b1;

my_blk u (.en(en_q));

## Q8.23  W146 — Positional port connection   MUST KNOW

D  Ports are connected by order instead of by name.

P  If the port list changes, signals connect to the wrong ports silently.

W  Use named connections.

E  See the mistake and corrected code below.

// MISTAKE

fifo u0 (clk, rst_n, wr_en, din, full);

// CORRECTED

fifo u0 (.clk(clk), .rst_n(rst_n), .wr_en(wr_en), .din(din), .full(full));

Interview line:  “I always use named port connections. It is easier to review and does not break when the port order changes.”

## Q8.24  W156 — Reverse-connected bus

D  A bus is connected with reversed bit order.

P  Bit order flips silently and breaks the data.

W  Check that the bit ranges ([7:0] vs [0:7]) match on both sides.

E  See the mistake and corrected code below.

// MISTAKE

wire [7:0] a;

wire [0:7] b;

assign b = a;                    // reversed bit order

// CORRECTED

wire [7:0] b;

assign b = a;

## Q8.25  W497 / W498 — Bus not fully set / not fully read

D  Only some bits of a bus are driven (W497) or only some bits are read (W498).

P  Undriven bits give x. Unread bits may mean wasted logic or a protocol field that is ignored by mistake.

W  Drive all bits, and read all bits you need. Document intentional unused bits.

E  See the mistake and corrected code below.

// MISTAKE

logic [7:0] bus;

assign bus[3:0] = a;             // [7:4] undriven

// CORRECTED

assign bus = {4'b0, a};

## Q8.26  W446 — Output port read inside its own module

D  The module reads one of its own output ports.

P  It is legal in SystemVerilog, but older tools and style rules dislike it, and it hides the internal signal.

W  Use an internal signal and assign the output from it.

E  See the mistake and corrected code below.

// MISTAKE

output reg valid;

always @(posedge clk) if (valid) cnt <= cnt + 1;   // reads the output

// CORRECTED

reg valid_i;

assign valid = valid_i;

always @(posedge clk) if (valid_i) cnt <= cnt + 1;

## Q8.27  W502 — Modifying a signal in the sensitivity list

D  A block assigns a signal that is also in its own sensitivity list.

P  It can create loops and unexpected re-triggering.

W  Do not drive a signal from a block that is sensitive to it. Use always_comb or always_ff.

E  See the mistake and corrected code below.

// MISTAKE

always @(a or y)                 // y is also assigned here

y = a ^ y;

// CORRECTED

always_comb y = a ^ b;

## Q8.28  Why are width and signedness issues especially important in counters and streaming buses?

D  Counters and buses carry exact counts and bytes, so every bit matters.

P  A truncated or wrapped value gives a wrong packet length, a lost beat, or a stuck FSM.

W  Check counter width against the maximum value, comparison width against the counter, EMPTY width against the bus bytes, and signed/unsigned in subtractions such as BUS_BYTES − EMPTY.

E  A beat counter wraps after 256 beats, so a long packet restarts its count.

Interview line:  “Width bugs in counters and stream buses are functional bugs, so I treat them as high priority.”

## Q8.29  Why are unused or undriven signals important in an AXI-Stream or Avalon-ST design?

D  A protocol signal that nobody drives or reads often means a broken interface.

P  Streaming protocols depend on every control signal, so a missing connection breaks the handshake or the packet boundaries.

W  Check that READY, VALID, SOP/EOP/TLAST, and EMPTY/TKEEP each have exactly one driver and one reader. Optional sideband signals should be tied off on purpose and documented.

E  An EOP that is generated but never read means the Slave never ends the packet.

Interview line:  “In a streaming design, an unused protocol signal is a red flag. I check the driver and the reader of every control signal.”

## Q8.30  How would you debug a lint violation in an interview?

D  Use a fixed method: read, locate, decide, fix.

P  It shows that you understand lint instead of silencing it.

W  1. Read the rule ID and the message.

•  2. Find the line and the signal.

•  3. Decide: real bug, style issue, or false alarm.

•  4. Fix the root cause. Waive only with a written reason.

•  5. Re-run lint and confirm the finding is gone and no new one appeared.

E  W18 on y: add a default value at the top of the always_comb block.

Interview line:  “I read the rule, find the signal, decide if it is a real bug, fix the cause, and re-run. I do not waive without a reason.”

## C. Lint-proof patterns and checklists

## Q8.31  What do lint-clean RTL patterns look like?

D  They are coding templates that avoid the common violations.

P  Following them from the start saves review time.

W  Sequential block: always_ff with non-blocking. Combinational block: defaults first, complete cases. One owner per signal. Explicit widths. Named port connections.

E  See the code below.

// Sequential

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) q <= 1'b0; else q <= d;

// Combinational with defaults

always_comb begin

next_state = state;  ready = 1'b0;

case (state)

IDLE:    if (start) next_state = SEND;

SEND:    ready = 1'b1;

default: next_state = IDLE;

endcase

end

// Explicit width for carry

logic [DATA_W:0] sum_ext;

assign sum_ext = {1'b0, a} + {1'b0, b};

// Named instance connection

my_block u_my_block (.data(data), .valid(valid), .ready(ready));

## Q8.32  What is the lint checklist for an AXI-Stream or Avalon-ST design?

D  It is a short review list for the protocol-specific lint risks.

P  It catches the bugs that general lint rules do not understand.

W  Every VALID / DATA / control relationship has a clear owner, and READY has a single driver.

•  While VALID is held under backpressure, data and sideband stay stable.

•  FIFO full / almost-full logic does not create conflicting READY drivers. FIFO empty prevents an invalid read.

•  SOP / EOP / TLAST are generated and consumed on the intended beat. EMPTY is sized correctly and used only on the final beat.

•  512-bit DATA and 521-bit packed word widths match. Pack and unpack bit positions are the same.

•  FIFO read latency is handled with no off-by-one assumption.

•  FSM states have complete next-state and output assignments. Counters have the right width and deliberate rollover.

•  No multiple drivers, no unused or undriven protocol signals, no inferred latches, and port widths match.

E  Use this list in a code review before running simulation.

Interview line:  “For streaming designs I add a protocol checklist on top of normal lint: ownership, stability, boundaries, widths, and FIFO latency.”

## D. What to master first

| Priority | Rules |
| --- | --- |
| 1 — Must know | W336 blocking vs non-blocking; W18 latch; W415 multiple drivers; W120 / W528 unused logic; W110 / W164 width mismatch; W362 comparison width; W210 / W287a connectivity. |
| 2 — Very useful | W123 read but not set; W240 unused input; W241 incomplete output; W263 case width; W122 / W456 sensitivity list; W484 carry loss; W486 shift overflow; W146 named ports. |
| 3 — Know and recognize | W156 reversed bus; W446 output read internally; W497 / W498 partial bus; W502 sensitivity list modification; W551 case intent. |

# PART 9 — STATIC TIMING ANALYSIS (STA)

Your earlier STA chapter had 60 small items, and many of them were the same idea in different words (setup time, setup slack, setup equation). Here they are grouped into one question per idea. Clock and I/O constraint topics (false path, multicycle, input/output delay, generated clocks, clock groups) are answered once in Part 10 together with their commands. Recovery and removal are answered once in Part 7.

## Q9.1  What is STA, and why do we use it?   MUST KNOW

D  Static Timing Analysis checks the timing of all constrained paths in a design without using test vectors.

P  It proves that setup, hold, and other timing rules are met across the required operating conditions. Simulation cannot cover every path and every delay condition.

W  The tool splits the design into timing paths. For each path it computes the arrival time and the required time, and reports slack. It uses clock definitions, I/O constraints, cell and wire delays, and exceptions. It checks only what is constrained.

E  A register-to-register path at 100 MHz must finish inside 10 ns.

Interview line:  “STA checks every constrained path against setup and hold using worst-case delays, so I do not need test vectors to prove timing.”

Follow-up questions to expect

- What is not covered by STA? — Unconstrained paths, and the functional correctness of CDC.

- Does STA check function? — No. Only timing.

## Q9.2  STA vs simulation vs synthesis — what is the difference?

D  Simulation checks behavior for chosen stimulus. Synthesis builds the gate-level design. STA checks the timing of that design.

P  Each answers a different question: does it work, can it be built, is it fast enough.

W  A design can pass simulation and still fail STA on a rarely used path. Synthesis estimates timing, but final timing sign-off needs STA with real placement and routing delays.

E  A functional simulation passes while STA reports negative setup slack on a rarely exercised path.

Interview line:  “Simulation proves function, synthesis builds hardware, and STA proves timing on all paths.”

## Q9.3  What is a timing path? (startpoint, endpoint, launch and capture, data path and clock path)   MUST KNOW

D  A timing path goes from a startpoint to an endpoint. The startpoint is a flop clock pin or an input port. The endpoint is a flop D pin or an output port.

P  Knowing the parts lets you read reports and find the cause of a violation.

W  A register-to-register path: launch flop → clock-to-Q delay → combinational logic → capture flop D pin.

•  The launch clock starts the data. The capture clock arrives one period later and samples it.

•  The data path is the signal path. The clock path is the clock route to the launch and capture flops.

•  Four path types: reg→reg, input→reg, reg→output, input→output.

E  Launch FF → 6 logic levels → capture FF. Delay = clock-to-Q + logic + routing.

Interview line:  “A path has a launch flop, some logic, and a capture flop. I look at the clock-to-Q, the logic delay, and the setup time.”

Follow-up questions to expect

- What is clock-to-Q delay? — The time from the clock edge until Q changes.

- What is the combinational delay? — The sum of cell delays and wire delays between the two flops.

## Q9.4  What are setup time and the setup check?   MUST KNOW

D  Setup time is the minimum time data must be stable before the capture clock edge. The setup check verifies that data arrives early enough.

P  If data arrives too late, the flop may capture a wrong value or become metastable.

W  Setup check: T_clk ≥ T_clk2q + T_logic + T_setup + T_skew + T_uncertainty.

•  Setup slack = required time − arrival time. Negative slack is a violation.

•  Setup depends on the clock period. A slower clock helps.

•  Frequency and period: f = 1 / T. A 10 ns period is 100 MHz.

E  See the numeric example in Q9.9.

Interview line:  “A setup violation is mainly a late-data problem. I fix it by shortening the path, pipelining, or slowing the clock.”

Follow-up questions to expect

- Does a slower clock fix hold? — No. Hold does not depend on the period.

## Q9.5  What are hold time and the hold check?   MUST KNOW

D  Hold time is the minimum time data must stay stable after the capture clock edge. The hold check verifies that new data does not arrive too early.

P  If new data arrives too soon, the flop captures the wrong value (the next data instead of the current one).

W  Hold check: arrival(min) ≥ capture clock arrival + T_hold.

•  It uses the fast (minimum-delay) path.

•  Hold does not depend on the clock period, because launch and capture use the same edge. Fix it by adding delay to the data path.

E  A direct flop-to-flop path with clock skew often creates a hold violation.

Interview line:  “Setup is about being fast enough. Hold is about not being too fast. Hold violations are fixed by adding delay, and the tools normally do this after routing.”

Follow-up questions to expect

- Why is hold worst at the fast corner? — Fast cells make the data path shorter.

## Q9.6  What are arrival time, required time, and slack?   MUST KNOW

D  Arrival time is when data actually reaches the endpoint. Required time is when it must be there. Slack is the margin between them.

P  Slack tells you at a glance whether a path meets timing.

W  Setup slack = required − arrival. Hold slack = arrival − required. Positive slack: the path has margin. Negative slack: the path violates timing. WNS is the worst negative slack. TNS is the sum of all negative slacks. WHS is the worst hold slack.

E  A path with −0.2 ns setup slack needs improvement.

Interview line:  “Negative slack means the timing requirement is not met for that path.”

Follow-up questions to expect

- What are WNS, TNS, and WHS? — Worst negative setup slack, total negative setup slack, and worst hold slack.

## Q9.7  What is a critical path, and what is maximum-delay versus minimum-delay analysis?

D  The critical path is the path with the worst slack. Max-delay analysis (setup) uses the slowest delays. Min-delay analysis (hold) uses the fastest delays.

P  The critical path limits the maximum clock frequency, so timing closure starts there.

W  Setup analysis uses the longest paths and the slow corner. Hold analysis uses the shortest paths and the fast corner. A long arithmetic chain often becomes the critical path.

E  A 32-bit multiply followed by an add in one stage.

Interview line:  “Timing closure often starts by understanding and optimizing the critical paths.”

## Q9.8  How do you compute Fmax from slack?   ADVANCED

D  Fmax is the highest clock frequency at which all setup checks still pass.

P  It tells you how much headroom the design has.

W  T_min = T_clk − WNS (for positive slack, T_min is smaller than T_clk). Fmax = 1 / T_min. Only valid when the critical path is a single-clock reg-to-reg path.

E  T_clk = 10 ns, WNS = +1.30 ns → T_min = 8.7 ns → Fmax ≈ 115 MHz.

Interview line:  “Fmax is one over the period minus the worst slack.”

## Q9.9  Worked example: how do you calculate setup and hold slack?   MUST KNOW

D  A numeric check of the two equations on a simple path.

P  Interviewers often ask for a quick calculation on the whiteboard.

W  Setup. Period 10 ns. Clock-to-Q 0.10. Logic + routing 8.20. Setup 0.20. Uncertainty 0.20. Zero skew.

•  Arrival = 0.10 + 8.20 = 8.30. Required = 10.00 − 0.20 − 0.20 = 9.60. Setup slack = 9.60 − 8.30 = +1.30 ns. The path passes.

•  Hold. Clock-to-Q 0.30. Logic 0.10. Hold requirement 0.20. Capture clock is 0.40 later than launch (skew).

•  Arrival (min) = 0.30 + 0.10 = 0.40. Required = 0.40 (capture clock) + 0.20 = 0.60. Hold slack = 0.40 − 0.60 = −0.20 ns. Violation.

•  Fix: add at least 0.20 ns of delay on the data path (a delay buffer). Then slack becomes ≥ 0.

E  Notice that the hold failure came from a very short path plus clock skew, while the same path easily passes setup.

Interview line:  “I write arrival and required time, subtract for slack, and compare. For hold I use the fastest delay and include the skew.”

## Q9.10  What is clock skew, and what are positive and negative skew?

D  Skew is the difference in clock arrival time between two flops.

P  It changes the time available for setup and the margin for hold.

W  If the capture clock arrives later than the launch clock (positive skew), setup gets more time but hold gets harder. If it arrives earlier (negative skew), setup gets less time but hold gets easier. Tools can use skew on purpose (“useful skew”).

E  A capture flop that gets its clock 0.3 ns later gives setup +0.3 ns and hold −0.3 ns.

Interview line:  “Skew is a trade-off between setup and hold on the same path, so I check both.”

## Q9.11  What are clock uncertainty and jitter?

D  Jitter is the small variation of the clock edge from its ideal time. Uncertainty is the margin STA subtracts to account for jitter and other effects.

P  It makes the analysis safer than the ideal-clock case.

W  Uncertainty reduces the time available for setup and tightens hold. It is set with set_clock_uncertainty or taken from the clock source characteristics.

E  0.2 ns uncertainty at 10 ns period leaves 9.8 ns for the path.

Interview line:  “Uncertainty is the safety margin for clock variation, and it takes directly from my setup budget.”

## Q9.12  What are clock latency, source latency, network latency, and propagation delay?   ADVANCED

D  Clock latency is the delay of the clock from its source to a flop clock pin. It has two parts: source latency (clock source to the clock definition point) and network latency (definition point to the flop pins). Propagation delay is the time a signal takes through a cell or a wire.

P  It is needed so STA knows when the launch and capture clock edges really arrive.

W  In FPGA implementation, the tool computes real clock network delay (propagated clocks). In early analysis you can use ideal clocks with zero latency. Total clock path delay = source latency + network latency.

E  A clock buffer tree of 1.2 ns gives network latency of 1.2 ns to every flop (and skew from differences).

Interview line:  “Latency is the delay of the clock path. Only the difference between launch and capture latency matters for the check, and that difference is skew.”

## Q9.13  What are PVT corners, and what are the slow and fast corners?   MUST KNOW

D  PVT is Process, Voltage, Temperature. A corner is one combination of the three.

P  Timing must pass in all operating conditions.

W  Slow corner (slow process, low voltage, high temperature): cells are slow, so it is worst for setup. Fast corner (fast process, high voltage, low temperature): cells are fast, so it is worst for hold.

E  Setup sign-off at the slow corner. Hold sign-off at the fast corner.

Interview line:  “I check setup at the slow corner and hold at the fast corner, because that is where each one is worst.”

## Q9.14  What are OCV and derating?   ADVANCED

D  OCV (on-chip variation) means that delays differ across the chip, even for the same cell type, because of local variation. Derating adds margin for it.

P  Real silicon is not uniform, so the analysis must not assume identical delays on every path.

W  For setup, the launch path is made slower (late) and the capture path faster (early). For hold, it is the opposite. Derating is a multiplier on delays, for example 1.05 for late paths.

E  A derate factor of 5% on the data path reduces setup slack on a long path.

Interview line:  “OCV adds pessimism on purpose, so that chip-to-chip and local variation are covered.”

## Q9.15  What are clock gating checks?   ADVANCED

D  Clock gating checks make sure the gate’s enable is stable while the gated clock is active.

P  An enable that changes while the clock is high can create glitches or runt pulses on the gated clock.

W  For an AND gate, the enable must be stable while the clock is high (setup before the rising edge, hold after the falling edge). Using an ICG cell with a latch handles this safely, and STA checks the timing at the gate.

E  The enable arrives late and overlaps the clock-high period: clock gating setup fails.

Interview line:  “I use an ICG cell and check the clock-gating setup and hold in the timing report.”

## Q9.16  What are max transition, max capacitance, and fanout?

D  They are design-rule limits: how slow a signal edge may be, how much load a driver may have, and how many loads it may drive.

P  Violations slow the path, hurt signal quality, and make timing worse.

W  A high-fanout net has a large load and a slow edge, which adds delay. Fix it by duplicating the driver register, buffering, or reducing the number of loads.

E  One enable signal driving 521 flops is a common high-fanout problem.

Interview line:  “High fanout is a common reason for a failing path. I duplicate the driver or add pipeline stages.”

## Q9.17  What are timing libraries and cell arcs?   ADVANCED

D  The timing library describes the delay and constraints of each cell. A timing arc is a delay relationship between a cell input and output.

P  STA needs this data to calculate path delays.

W  For a flop there are arcs for clock-to-Q, setup, and hold. For a gate there is an input-to-output arc. Delay depends on input slew and output load, so it is read from a table.

E  The setup and hold of a flop come from library checks.

Interview line:  “The library tells STA the delay and the setup and hold values for each cell, based on slew and load.”

## Q9.18  How do you read a timing report?   MUST KNOW

D  A timing report lists one path with each delay and the final slack.

P  You need it to find the reason for a violation quickly.

W  1. Read the slack and the check type (setup / hold).

•  2. Read the startpoint and endpoint and the clocks.

•  3. Check the period and the uncertainty.

•  4. Look at the data path delay: logic levels, fanout, net delay versus cell delay.

•  5. Look at the clock path and the skew.

•  6. Find the biggest delay item and fix that.

E  Large net delay with high fanout → duplicate the register or add a pipeline stage.

Interview line:  “I read slack first, then startpoint and endpoint, then look for the largest delay item in the data path.”

## Q9.19  How do you fix setup violations, and how do you fix hold violations?   MUST KNOW

D  Setup violations mean the path is too slow. Hold violations mean it is too fast.

P  Each needs a different fix. Wrong fixes make the other problem worse.

W  Setup: pipeline the path, reduce logic levels, restructure the logic, reduce fanout (duplicate registers), register outputs, retime, improve placement, use useful skew, or lower the clock.

•  Hold: insert delay buffers on the data path, or reduce the clock skew. Tools do this automatically after routing. Large hold violations often point to a wrong constraint or a clocking problem.

E  A 12-level path split into 6 + 6 with a register in the middle.

Interview line:  “For setup I make the path shorter. For hold I add delay. I never lower the clock to hide a hold problem, because that does not help.”

Follow-up questions to expect

- Why does adding a pipeline stage not always fix a path? — The new stage must still be balanced; and latency changes may need protocol updates.

## Q9.20  What is your STA debug checklist?   MUST KNOW

D  A fixed list of checks, in order of likelihood.

P  Most timing “problems” are constraint problems, not design problems.

W  1. Are all clocks defined? (report_clocks)

•  2. Are I/O delays correct? (check_timing, report_timing_summary)

•  3. Are there unconstrained paths or missing clocks?

•  4. Are exceptions justified (false path, multicycle)?

•  5. Are clock groups correct? (report_clock_interaction)

•  6. Then look at the worst path in detail.

•  7. Fix the cause: logic depth, fanout, placement, or clocking.

E  Negative slack of −5 ns on a path between two unrelated clocks: it is a missing clock group or CDC constraint, not a design failure.

Interview line:  “First I check that the constraints are right. Then I read the worst path. Then I fix the cause.”

Follow-up questions to expect

- What does check_timing report? — Unconstrained endpoints, missing clocks, combinational loops, and generated clocks without a master.

Related timing topics answered once in other parts:

| Question (already answered, so not repeated) | Answered in |
| --- | --- |
| What are recovery and removal checks, and what is asynchronous reset timing? | Q7.27 |
| What are false paths and multicycle paths? | Q10.7 |
| What are input and output delay? | Q10.6 |
| What are generated clocks and clock groups? | Q10.4 |

# PART 10 — XDC CONSTRAINTS (FPGA / VIVADO)

Your earlier XDC chapter had 50 items. Here, every constraint is explained once, with its concept and its command together (so there is no separate “concept” and “command” question for the same thing).

## Q10.1  What is XDC, and why do we need constraints?   MUST KNOW

D  XDC is the Xilinx Design Constraints format, based on Tcl. It tells Vivado about clocks, I/O timing, pins, and timing exceptions.

P  Without constraints the tool does not know your clock speed or pin locations, and STA cannot check unconstrained paths.

W  The file has two groups: timing constraints (clock, input/output delay, exceptions) and physical constraints (pin, I/O standard, location). Vivado uses them in synthesis and implementation. STA uses the timing part to compute slack.

E  Basic clock and pin constraints:

create_clock -name clk -period 10.000 [get_ports clk]

set_property PACKAGE_PIN W5       [get_ports clk]

set_property IOSTANDARD  LVCMOS33 [get_ports clk]

Interview line:  “XDC gives the tool the clock and I/O rules. The timing part feeds STA, and the physical part decides pins and electrical settings.”

Follow-up questions to expect

- Timing vs physical constraints? — Timing: how fast. Physical: where and how the signal connects.

- What are the first constraints you write? — The clock.

- XDC vs SDC? — XDC is Vivado’s Tcl-based format. It contains most SDC commands plus Xilinx-specific physical properties.

## Q10.2  What is the XDC file structure, and what are objects and collections?

D  An XDC file is a Tcl script. It uses commands, comments (#), and object queries to select design objects.

P  You must select the right object (port, pin, cell, net, clock) for each constraint.

W  get_ports selects top-level ports. get_pins selects pins on cells. get_cells selects instances (registers, blocks). get_nets selects nets. get_clocks selects defined clocks.

•  Each query returns a collection that you pass to a constraint command.

•  Use wildcards, for example [get_cells wptr_gray_reg*]. Object names depend on the synthesized design, so check them in Vivado.

E  Select a clock port, a register, and a clock object:

# Ports, cells, pins, nets, clocks

set p [get_ports {din[*]}]

set r [get_cells u_fifo/wptr_reg*]

set c [get_clocks clk_a]

Interview line:  “Every constraint needs the right object. I check the names in the Vivado netlist before I write them.”

## Q10.3  How does create_clock work? (period, waveform, and the missing-clock mistake)   MUST KNOW

D  create_clock defines a primary clock on a port or pin. It gives the clock a name, a period, and optionally a waveform.

P  STA needs every clock defined to compute the timing window of each path.

W  Period is in ns (10 ns = 100 MHz). The default waveform is 50% duty with the rising edge at 0. Use -waveform {rise fall} for another duty cycle. If a clock port has no create_clock, paths from it are unconstrained and not checked.

E  200 MHz clock, 50% duty. And a 10 ns clock with edges at 0 and 5 ns.

create_clock -name clk_a -period 5.0  [get_ports clk_a]            ;# 200 MHz

create_clock -name clk_b -period 10.0 -waveform {0 5} [get_ports clk_b]

Common mistake:  Forgetting create_clock on an input clock leaves every path from it unconstrained. STA reports no violations, which looks like a pass but checks nothing.

Interview line:  “I define every primary clock first, with the real period of the board clock.”

## Q10.4  What are generated clocks and clock groups?   ADVANCED

D  A generated clock is a clock derived from another clock (divider, PLL output). Clock groups tell STA which clocks are related or unrelated.

P  STA needs to know how derived clocks relate to their source, and which clock pairs it should not time against each other.

W  Use create_generated_clock for clocks created by logic dividers. In FPGAs, PLL/MMCM outputs are generated automatically by Vivado. Use set_clock_groups -asynchronous only if the domains are truly asynchronous and the CDC is handled in RTL.

E  A divided clock and an asynchronous group:

create_generated_clock -name clk_div2 -source [get_ports clk_a] \

-divide_by 2 [get_pins div_reg/Q]

set_clock_groups -asynchronous -group {clk_a} -group {clk_b}

Common mistake:  set_clock_groups -asynchronous removes all timing checks between the groups. If a crossing is unsafe, the timing tools will not warn you. In real FPGA designs, use the actual clocking resource (MMCM/PLL/BUFG) that the flow supports for generated clocks.

Interview line:  “I define generated clocks first, and I declare clock groups asynchronous only when the design really handles the crossing.”

Follow-up questions to expect

- What if a derived clock is not defined? — The tool may treat it as unconstrained or wrong.

- set_clock_groups vs set_false_path? — Clock groups act on whole clock pairs and are cleaner. False path acts on selected paths.

## Q10.5  What are set_clock_uncertainty, set_clock_latency, and clock transition?

D  set_clock_uncertainty reserves timing margin. set_clock_latency models clock delay. Clock transition is the slew of the clock edge.

P  They make the clock model closer to the real clock, or add safety margin.

W  In implementation flows, Vivado calculates real clock propagation, so you rarely set latency by hand. Uncertainty is added for jitter and margin. Use latency only when the timing methodology calls for explicit modeling.

E  Reserve 0.2 ns of margin:

set_clock_uncertainty 0.200 [get_clocks clk]

# set_clock_latency is only for explicit modeling methods

Interview line:  “I add uncertainty for margin, and I rely on the tool’s propagated clock instead of setting latency by hand.”

## Q10.6  What are set_input_delay and set_output_delay?   MUST KNOW

D  set_input_delay says how late an external signal arrives after the clock at the port. set_output_delay says how much time the external device needs after the FPGA output.

P  It makes STA check the I/O paths against the real board and the external chip.

W  Input delay is measured relative to a clock at the port. Output delay is the external setup time plus board skew. Use -max for setup analysis and -min for hold analysis. Without them, input-to-register and register-to-output paths are not checked properly.

E  External chip sends data up to 2 ns after the clock edge:

set_input_delay  -clock clk -max 2.0 [get_ports din]

set_input_delay  -clock clk -min 0.5 [get_ports din]

set_output_delay -clock clk -max 1.5 [get_ports dout]

set_output_delay -clock clk -min -0.5 [get_ports dout]

Interview line:  “Input and output delay describe what the outside world does, so STA can check the FPGA I/O paths.”

Follow-up questions to expect

- Why both -max and -min? — Setup uses the slowest arrival. Hold uses the fastest arrival.

- What is the I/O clock relationship? — The external device’s clock and the FPGA clock must be related; the constraints use the same or a virtual clock to describe it.

## Q10.7  What are false paths and multicycle paths, and how do you write them?   MUST KNOW

D  A false path is a path that never needs to meet timing. A multicycle path is a path that is allowed more than one clock cycle by design. Together with set_max_delay and set_min_delay, these are path exceptions.

P  They stop STA from reporting fake violations, but wrong use hides real bugs.

W  Use a false path only if you can prove the path is not functionally timed (a static configuration, or a crossing already protected by logic).

•  For a multicycle path, set the setup multiplier to N and the hold multiplier to N−1. Use it only if the source really changes once every N cycles (for example with an enable).

•  Never add an exception just because a path fails. First check whether the path is really required.

E  A register that loads every 4 cycles: multicycle setup 4, hold 3.

# false path (only with a proven reason)

set_false_path -from [get_clocks clk_a] -to [get_clocks clk_b]

# multicycle path: data is captured every 4 cycles

set_multicycle_path 4 -setup -from [get_cells src_reg*] -to [get_cells dst_reg*]

set_multicycle_path 3 -hold  -from [get_cells src_reg*] -to [get_cells dst_reg*]

Interview trap:  The classic wrong answer is “I add a false path when timing fails”. A false path must describe design intent, not hide a violation.

Interview line:  “I never add a false path just because a path fails. First I prove the path is not functionally timed, then I document the reason and keep the constraint as narrow as possible.”

Follow-up questions to expect

- Why is the hold multicycle N−1? — Otherwise the hold check moves N cycles away and becomes wrong.

- What is a wrong false-path example? — One that hides a real failing data path.

## Q10.8  What are set_max_delay and set_min_delay?   ADVANCED

D  set_max_delay limits how slow a path may be. set_min_delay limits how fast it may be.

P  They control paths that normal clock rules do not describe, for example CDC buses or asynchronous paths.

W  For a CDC data bus use set_max_delay -datapath_only to keep the crossing short, without an unsafe false path. The value is usually one source-clock period or less.

E  Limit an async FIFO pointer path to 5 ns:

set_max_delay -datapath_only -from [get_cells wptr_gray_reg*] \

-to [get_cells wq1_wptr_reg*] 5.0

Interview line:  “For CDC, I prefer a bounded max-delay constraint over a blind false path, so the crossing skew stays controlled.”

## Q10.9  How do I constrain pins: PACKAGE_PIN, IOSTANDARD, DRIVE, SLEW, PULLUP?   MUST KNOW

D  These properties assign the package pin and set the electrical behavior of each I/O.

P  The right pin and voltage standard make the board work and protect the device.

W  PACKAGE_PIN chooses the pin. IOSTANDARD sets the voltage level (LVCMOS33, LVDS…). DRIVE sets drive strength. SLEW sets edge speed. PULLUP / PULLDOWN sets an internal resistor. Always confirm values with the board schematic and the master XDC. Values must be valid for the chosen I/O standard.

E  An output LED with drive and slew:

set_property PACKAGE_PIN  U16       [get_ports led]

set_property IOSTANDARD   LVCMOS33  [get_ports led]

set_property DRIVE        8         [get_ports led]

set_property SLEW         SLOW      [get_ports led]

set_property PULLUP       true      [get_ports btn]

Note:  Pin names and I/O standards are board-specific. These are examples, not values to copy.

Interview line:  “Pins and I/O standards come from the board documents. I never guess them.”

## Q10.10  What are LOC and physical placement constraints?   ADVANCED

D  They place a cell at a specific location on the FPGA, or restrict it to a region.

P  Used for special cases: fixed hardware blocks, timing-critical placement, or isolating areas.

W  LOC fixes a cell to a site. PBLOCK restricts a group of cells to a region. They are powerful but easy to misuse. Use them only when the tool cannot meet timing, and base them on real device resources.

E  A synchronizer pair placed together or an IO register pinned near its pin.

set_property LOC SLICE_X10Y20 [get_cells u_sync/sync1_reg]

Common mistake:  Do not use placement constraints blindly. A wrong LOC can make timing worse and block routing.

Interview line:  “I let the tool place the design first. I use physical constraints only for a clear, specific reason.”

## Q10.11  What are common XDC mistakes?

D  These are the errors seen again and again in reviews.

P  They cause “timing met” on paper and failures on the board.

W  No clock constraint on a clock input (paths are unconstrained).

•  Missing input and output delays.

•  A false path or clock group that hides a real problem.

•  Wrong I/O standard or unassigned pins.

•  A generated clock defined without the right source.

•  Constraints written for object names that do not exist (the command fails or does nothing).

•  A constraint file that is not in the active implementation run.

E  A 100 MHz clock port that has no create_clock: everything downstream shows no slack at all.

Interview line:  “Most constraint bugs are a missing clock, a missing I/O delay, or an exception that is too wide.”

## Q10.12  What does a complete basic FPGA constraint file look like?

D  A basic XDC file has four blocks: clock, pins and electrical properties, external interface timing, and optional settings.

P  It is a template you can adapt for any small design.

W  Define the clock first, then pin and I/O properties, then I/O delays, then optional electrical settings. Replace every pin, standard, and delay with values for your own board and external device.

E  Example below. All values are examples only.

# ---------- CLOCK ----------

create_clock -name clk -period 10.000 [get_ports clk]

# ---------- PINS + ELECTRICAL ----------

set_property PACKAGE_PIN W5      [get_ports clk]

set_property IOSTANDARD  LVCMOS33 [get_ports {clk rst_n din dout}]

set_property PACKAGE_PIN V17     [get_ports rst_n]

# ---------- EXTERNAL INTERFACE TIMING ----------

set_input_delay  -clock clk -max 2.0 [get_ports din]

set_input_delay  -clock clk -min 0.5 [get_ports din]

set_output_delay -clock clk -max 1.5 [get_ports dout]

# ---------- OPTIONAL ----------

set_property DRIVE 8 [get_ports dout]

set_property SLEW  SLOW [get_ports dout]

## Q10.13  How do you debug XDC problems in Vivado, and which reports do you use?   MUST KNOW

D  Debugging XDC means checking that the constraints are loaded, applied to real objects, and giving the expected timing.

P  A structured check is much faster than guessing.

W  1. Check that the XDC file is in the project and in the active run.

•  2. Look at the log for warnings such as “no objects found”.

•  3. report_clocks — are all clocks defined with the right period?

•  4. check_timing — missing clocks, unconstrained paths, loops.

•  5. report_clock_interaction — which clock pairs are timed, and which are cut?

•  6. report_timing_summary — WNS, TNS, WHS and failing paths.

•  7. report_cdc and report_exceptions — crossings and exception coverage.

E  WNS is +4 ns but check_timing shows many unconstrained endpoints: constraints are missing.

Interview line:  “First I confirm that the constraints are really loaded. Then I use report_clocks, check_timing, and report_timing_summary to see what is checked and what is not.”

Follow-up questions to expect

- What does report_exceptions show? — All timing exceptions and how many paths each one covers.

## XDC top-10 quick table

| # | Command | One-line meaning |
| --- | --- | --- |
| 1 | create_clock | Defines a primary clock and its period |
| 2 | create_generated_clock | Defines a clock derived from another |
| 3 | set_input_delay | Arrival time of an external input |
| 4 | set_output_delay | Time needed by the external receiver |
| 5 | set_clock_groups | Cuts timing between unrelated clocks |
| 6 | set_false_path | Path is never timed (use with proof) |
| 7 | set_multicycle_path | Path gets N cycles (set hold to N−1) |
| 8 | set_max_delay -datapath_only | Bounds a CDC path without a false path |
| 9 | PACKAGE_PIN / IOSTANDARD | Pin location and voltage standard |
| 10 | report_timing_summary / check_timing | Verify what is checked and what is not |

# PART 11 — VERIFICATION, TESTBENCH, ASSERTIONS AND DEBUG

## Q11.1  What is a testbench, and what is a directed test?   MUST KNOW

D  A testbench is code that drives inputs into the design under test (DUT) and checks the outputs. A directed test checks one chosen scenario.

P  RTL is correct only if it is tested in normal and corner conditions. The testbench is not synthesized.

W  A good testbench has a clock and reset, stimulus, a checker (scoreboard or self-checking), and a pass/fail result. Directed tests cover known cases. Random tests find cases you did not think of.

E  A directed test for a FIFO: write until full, read until empty. A random test: random READY with a scoreboard.

Interview line:  “I make the testbench self-checking, so a pass or fail result needs no waveform reading.”

Follow-up questions to expect

- Directed vs random? — Directed for known cases, random for finding unexpected bugs.

- Why drive inputs away from the clock edge? — To avoid races between the testbench and the DUT.

## Q11.2  What is a scoreboard?   MUST KNOW

D  A scoreboard is a checker that compares what went into the design with what came out.

P  It proves that no data is lost, duplicated, or reordered without checking waveforms by eye.

W  Every accepted input (valid && ready on the input) is pushed into a queue. Every accepted output pops the queue and compares. At the end, the queue must be empty.

E  A FIFO or a stream path with random VALID and READY.

always @(posedge clk) begin

if (in_fire)  exp_q.push_back(in_data);

if (out_fire) begin

if (out_data !== exp_q.pop_front())

$error("Data mismatch at %0t", $time);

end

end

Interview line:  “Every accepted input goes into a queue. Every accepted output is compared with the head of the queue.”

## Q11.3  What are corner cases, and which ones matter for streaming and FIFO designs?   MUST KNOW

D  Corner cases are boundary or unusual conditions.

P  They expose bugs that normal traffic does not show.

W  Reset in the middle of a transfer, and the first transfer after reset.

•  FIFO full, almost-full, empty, and a write and read in the same cycle.

•  READY low on every beat, READY low only on the last beat, and READY low for a long time.

•  First and last beat, single-beat packet, back-to-back packets, and no gap between packets.

•  Packet sizes smaller than, equal to, and larger than one bus beat, and EMPTY = 0 and EMPTY = max.

•  Counter maximum values and rollover.

E  For a packet generator: 1 byte, 64 bytes, 65 bytes, 256 bytes on a 64-byte bus.

Interview line:  “I always test boundary conditions, because many RTL bugs occur at transitions and limits.”

## Q11.4  What are assertions (SVA)?   MUST KNOW

D  An assertion is a rule about design behavior that is checked automatically during simulation or formal verification.

P  It finds the bug at the moment it happens, not later at the output.

W  Write properties for protocol rules. For valid/ready: data stable while VALID && !READY; no x on control signals; no write when full; no read when empty. |-> checks in the same cycle. |=> checks in the next cycle.

E  Stability and no-overflow checks:

// VALID must stay high and DATA stable until READY

assert property (@(posedge clk) disable iff (!rst_n)

(valid && !ready) |=> (valid && $stable(data)));

// no unknown value on a control signal after reset

assert property (@(posedge clk) disable iff (!rst_n) !$isunknown(valid));

Interview line:  “Assertions turn protocol expectations into executable checks. I put them on interfaces and FIFO flags.”

Follow-up questions to expect

- Assertion vs checker in testbench? — Assertions can live in RTL, run in every test, and support formal.

- What is disable iff for? — It switches the assertion off during reset.

## Q11.5  What is functional coverage?

D  Functional coverage measures which scenarios your tests have actually exercised.

P  A test that passes does not prove that all cases were tested. Coverage shows what is missing.

W  Define coverage points for states, transitions, packet sizes, FIFO levels, and READY patterns. Check that every bin is hit. Missing bins show missing tests.

E  A coverpoint on packet length with bins: 1 byte, one beat, multi-beat, maximum.

Interview line:  “Coverage tells me what I did not test. Passing tests alone do not.”

Follow-up questions to expect

- Code coverage vs functional coverage? — Code coverage shows which lines ran. Functional coverage shows which scenarios ran.

## Q11.6  How do you debug an RTL problem?   MUST KNOW

D  RTL debugging means finding the difference between expected behavior and actual behavior.

P  A structured process saves time that guessing wastes.

W  1. Reproduce the failure with the smallest test.

•  2. Find the first point where behavior becomes wrong, not the final wrong output.

•  3. Trace backward to the logic that drives it.

•  4. Check clock, reset, handshake, and enable conditions at that time.

•  5. Form a hypothesis, fix, and re-run the full test and related tests.

E  EOP appears one cycle early: check the beat counter and the compare used to generate EOP.

Interview line:  “I always find the first point where behavior becomes incorrect, instead of debugging the final wrong output.”

Follow-up questions to expect

- Which signals do you open first? — Clock and reset, then VALID, READY, state, and the counters.

## Q11.7  What is a good testbench structure for a valid/ready design?   ADVANCED

D  A layered testbench separates the generator, driver, monitor, and checker.

P  It makes tests reusable and the checking independent from the stimulus.

W  Generator makes the transactions. Driver turns them into VALID/DATA, and uses a random READY model on the output side. Monitors watch both interfaces. A scoreboard compares them. Assertions check the protocol rules all the time.

E  The tests in this handbook for the FIFO and the skid buffer follow this pattern with a queue as the scoreboard.

Interview line:  “I separate stimulus, monitoring, and checking, so I can change the traffic without touching the checker.”

# PART 12 — RTL CODING PRACTICE (TESTED SOLUTIONS)

Interviewers often say: “write it on the whiteboard”. These are the most common coding questions at the 3+ years level. Every solution below was compiled and run in Icarus Verilog (-g2012) with a small self-checking testbench. The test result is written under each problem. Practice by covering the code, writing it yourself, and then comparing.

Tip:  Speak while you code: state the interface, the clock and reset, the corner cases, and the widths first. Then write. Then say how you would test it. That order is what interviewers want to hear.

## Q12.1  Warm-up: flip-flop variants, mux, decoder, counter, shift register   MUST KNOW

D  Problem: Write a D flip-flop with async reset, with sync reset, and with enable; a 4:1 mux; a 3-to-8 decoder; a loadable counter; a serial-in parallel-out shift register.

P  These small blocks check basic coding style: always_ff, non-blocking, defaults, and widths.

W  Use always_ff with non-blocking for registers. Use always_comb with a default for the decoder. Use an index for the mux.

E  Tested: compiled with no errors; the same style is exercised in the larger tests below.

module dff_variants (input logic clk, rst_n, srst, en, d, output logic q_async, q_sync, q_en);

always_ff @(posedge clk or negedge rst_n) if (!rst_n) q_async <= 1'b0; else q_async <= d;

always_ff @(posedge clk) if (srst) q_sync <= 1'b0; else q_sync <= d;

always_ff @(posedge clk) if (en) q_en <= d;

endmodule

module mux4 (input logic [3:0] d, input logic [1:0] sel, output logic y);

assign y = d[sel];

endmodule

module dec3to8 (input logic [2:0] a, input logic en, output logic [7:0] y);

always_comb begin y = '0; if (en) y[a] = 1'b1; end

endmodule

module up_counter #(parameter int W = 8) (input logic clk, rst_n, en, load, input logic [W-1:0] din, output logic [W-1:0] q);

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) q <= '0; else if (load) q <= din; else if (en) q <= q + 1'b1;

endmodule

module shift_sipo #(parameter int W = 8) (input logic clk, rst_n, en, sin, output logic [W-1:0] q);

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) q <= '0; else if (en) q <= {q[W-2:0], sin};

endmodule

Interview line:  “For small blocks I show the clock, the reset style, and the default value, so the interviewer sees the coding discipline.”

## Q12.2  Rising / falling edge detector   MUST KNOW

D  Problem: Generate a one-cycle pulse when sig goes 0→1 (and another when it goes 1→0).

P  Used on every external or asynchronous-looking control. Tests understanding of registered history.

W  Delay the signal by one flop. Rise = now & ~before. Fall = ~now & before. (The input must already be synchronous to clk.)

E  Tested: 2 rising and 2 falling edges produced exactly 2 + 2 pulses.

module edge_detect (

input  logic clk, rst_n,

input  logic sig,

output logic rise, fall

);

logic sig_d;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) sig_d <= 1'b0;

else        sig_d <= sig;

assign rise =  sig & ~sig_d;   // 0 -> 1

assign fall = ~sig &  sig_d;   // 1 -> 0

endmodule

Interview line:  “An edge detector is one flop and one gate. The input must already be in the same clock domain.”

## Q12.3  Binary to Gray and Gray to binary   MUST KNOW

D  Problem: Convert a W-bit binary number to Gray code and back.

P  Gray code is the base of async FIFO pointers.

W  Gray = b ^ (b >> 1). Binary from Gray: the MSB is the same, and each lower bit is the XOR of the previous binary bit and the current Gray bit.

E  Tested: for all 16 values: round trip is exact and neighbouring Gray values differ by exactly one bit.

module bin2gray #(parameter int W = 4) (

input  logic [W-1:0] b,

output logic [W-1:0] g

);

assign g = b ^ (b >> 1);

endmodule

module gray2bin #(parameter int W = 4) (

input  logic [W-1:0] g,

output logic [W-1:0] b

);

always_comb begin

b[W-1] = g[W-1];

for (int i = W-2; i >= 0; i--)

b[i] = b[i+1] ^ g[i];

end

endmodule

## Q12.4  Synchronous FIFO   MUST KNOW

D  Problem: Design a parameterized single-clock FIFO with full and empty flags.

P  The most common design question. It tests pointers, the extra wrap bit, and the read latency.

W  Pointers have one extra bit. Empty when pointers are equal. Full when addresses are equal and the MSB differs. Read data is registered, so it appears one cycle after rd_en.

E  Tested: random writes (60%) and reads (40%) for 3,000 cycles with a scoreboard: 3,331 words in order, no mismatch, FIFO reached full (depth 8).

module sync_fifo #(

parameter int DW = 8,

parameter int AW = 4                 // depth = 2**AW

) (

input  logic          clk, rst_n,

input  logic          wr_en,

input  logic [DW-1:0] wdata,

input  logic          rd_en,

output logic [DW-1:0] rdata,         // valid 1 cycle after rd_en

output logic          full, empty

);

logic [DW-1:0] mem [0:(1<<AW)-1];

logic [AW:0]   wptr, rptr;           // one extra bit

assign empty = (wptr == rptr);

assign full  = (wptr[AW-1:0] == rptr[AW-1:0]) && (wptr[AW] != rptr[AW]);

wire do_wr = wr_en && !full;

wire do_rd = rd_en && !empty;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin wptr <= '0; rptr <= '0; end

else begin

if (do_wr) wptr <= wptr + 1'b1;

if (do_rd) rptr <= rptr + 1'b1;

end

always_ff @(posedge clk) begin

if (do_wr) mem[wptr[AW-1:0]] <= wdata;

if (do_rd) rdata <= mem[rptr[AW-1:0]];

end

endmodule

Interview line:  “I use an extra pointer bit for full and empty, block writes when full and reads when empty, and note that read data has one cycle of latency.”

Follow-up questions to expect

- How would you make it FWFT? — Drive rdata from mem[rptr] directly (asynchronous read, which maps to LUT RAM), or add an output stage.

- How do you add almost-full? — Compare wptr − rptr with a threshold.

## Q12.5  Asynchronous FIFO (Gray pointers)   MUST KNOW

D  Problem: Design a FIFO with separate write and read clocks.

P  The standard CDC design question. Tests Gray pointers, synchronizers, and the full/empty rules.

W  Binary pointers address the memory. Gray pointers cross through two flops. Full uses the top-two-bits-inverted compare in the write domain. Empty uses a direct compare in the read domain and resets to 1.

E  Tested: write clock 7 ns, read clock 11 ns, slow-read and slow-write phases with a scoreboard: 2,400 words in order, no mismatch, both full and empty were seen.

module async_fifo #(

parameter int DW = 8,

parameter int AW = 4                       // depth = 2**AW

) (

// write side

input  logic          wclk, wrst_n, winc,

input  logic [DW-1:0] wdata,

output logic          wfull,

// read side

input  logic          rclk, rrst_n, rinc,

output logic [DW-1:0] rdata,               // first-word fall-through

output logic          rempty

);

logic [DW-1:0] mem [0:(1<<AW)-1];

logic [AW:0] wbin, wbin_n, wgray, wgray_n;

logic [AW:0] rbin, rbin_n, rgray, rgray_n;

logic [AW:0] wq1_rgray, wq2_rgray;         // read pointer synced to wclk

logic [AW:0] rq1_wgray, rq2_wgray;         // write pointer synced to rclk

// ---------------- write domain ----------------

assign wbin_n  = wbin + (winc & ~wfull);

assign wgray_n = (wbin_n >> 1) ^ wbin_n;

always_ff @(posedge wclk or negedge wrst_n)

if (!wrst_n) {wbin, wgray} <= '0;

else         {wbin, wgray} <= {wbin_n, wgray_n};

always_ff @(posedge wclk)

if (winc && !wfull) mem[wbin[AW-1:0]] <= wdata;

always_ff @(posedge wclk or negedge wrst_n)

if (!wrst_n) {wq2_rgray, wq1_rgray} <= '0;

else         {wq2_rgray, wq1_rgray} <= {wq1_rgray, rgray};

wire wfull_v = (wgray_n == {~wq2_rgray[AW:AW-1], wq2_rgray[AW-2:0]});

always_ff @(posedge wclk or negedge wrst_n)

if (!wrst_n) wfull <= 1'b0;

else         wfull <= wfull_v;

// ---------------- read domain -----------------

assign rbin_n  = rbin + (rinc & ~rempty);

assign rgray_n = (rbin_n >> 1) ^ rbin_n;

always_ff @(posedge rclk or negedge rrst_n)

if (!rrst_n) {rbin, rgray} <= '0;

else         {rbin, rgray} <= {rbin_n, rgray_n};

always_ff @(posedge rclk or negedge rrst_n)

if (!rrst_n) {rq2_wgray, rq1_wgray} <= '0;

else         {rq2_wgray, rq1_wgray} <= {rq1_wgray, wgray};

wire rempty_v = (rgray_n == rq2_wgray);

always_ff @(posedge rclk or negedge rrst_n)

if (!rrst_n) rempty <= 1'b1;

else         rempty <= rempty_v;

assign rdata = mem[rbin[AW-1:0]];

endmodule

Interview line:  “The memory is dual-port. Only Gray-coded pointers cross, through two-flop synchronizers. Full is computed in the write domain and empty in the read domain.”

Follow-up questions to expect

- Why is rempty reset to 1 and wfull to 0? — After reset the FIFO is empty and not full.

- Why register wfull and rempty? — It avoids a long combinational path into the control logic and keeps the flags glitch-free.

## Q12.6  Sequence detector (overlapping “1011”)   MUST KNOW

D  Problem: Detect the serial pattern 1011, allowing overlapped matches (for example in 1011011 two matches).

P  A classic FSM question. Tests state design and the overlap rule.

W  States: S0 (nothing), S1 (“1”), S10, S101, S1011. After a match, the last “1” is the start of a new match, so go to S1 on 1, or S10 on 0. Moore output = (state == S1011).

E  Tested: 2,000 random bits compared against a reference shift register: no mismatch.

// Overlapping "1011" detector, Moore FSM

module seq_1011 (

input  logic clk, rst_n,

input  logic din,

output logic detected

);

typedef enum logic [2:0] {S0, S1, S10, S101, S1011} state_t;

state_t state, next;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) state <= S0;

else        state <= next;

always_comb begin

next = state;

case (state)

S0:    if (din) next = S1;                    // else stay in S0

S1:    if (!din) next = S10;                  // "11" keeps a useful "1"

S10:   if (din) next = S101; else next = S0;

S101:  if (din) next = S1011; else next = S10;

S1011: if (din) next = S1;    else next = S10; // overlap handling

default: next = S0;

endcase

end

assign detected = (state == S1011);                   // Moore output

endmodule

Interview line:  “I draw the states for the prefix matched so far, and decide where each input goes. For overlap I keep the useful tail.”

Follow-up questions to expect

- Non-overlapping version? — After a match, go back to S0.

- Mealy version? — Output = (state == S101) && din, which saves one state and reacts one cycle earlier.

## Q12.7  Clock divider: even N and odd divide-by-3 with 50% duty   ADVANCED

D  Problem: Divide a clock by 3 with a 50% duty cycle. (Even division is easy.)

P  Tests clock-edge thinking. An odd divider needs both clock edges for 50% duty.

W  Count 0→1→2 on the rising edge and make a signal a that is high for two of the three cycles. Make a copy b of a on the falling edge. The output is a & b: 1.5 cycles high and 1.5 cycles low.

E  Tested: after reset, the output period is exactly 30 ns for a 10 ns input clock, high 15 ns and low 15 ns, for 180 periods.

// Divide-by-3 with 50% duty cycle

module clk_div3 (

input  logic clk, rst_n,

output logic clk_out

);

logic [1:0] cnt;

logic a, b;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin cnt <= 2'd0; a <= 1'b1; end

else begin

cnt <= (cnt == 2'd2) ? 2'd0 : cnt + 2'd1;

a   <= (cnt == 2'd2) || (cnt == 2'd0);   // high for 2 of 3 cycles

end

always_ff @(negedge clk or negedge rst_n)        // half-cycle shifted copy

if (!rst_n) b <= 1'b1;

else        b <= a;

assign clk_out = a & b;                          // 1.5 cycles high, 1.5 low

endmodule

Note:  The divide-by-N for even N is just a toggle flop that flips every N/2 input cycles. Code below (tested: 40 ns period for N = 4).

Interview line:  “An odd divider with 50% duty uses one signal on the rising edge and a half-cycle-shifted copy on the falling edge. Then I combine them.”

Follow-up questions to expect

- In FPGAs, should you build clocks from logic? — No. Use an MMCM/PLL or a clock-enable. Divided clocks from fabric flops are bad for timing and CDC.

## Q12.8  Even clock divider (divide by N, N even)

D  Problem: Divide the clock by an even number N.

P  A simple warm-up for clock questions.

W  Count N/2 input cycles, then toggle the output flop.

E  Tested: N = 4 gave a 40 ns period from a 10 ns clock.

module div_even #(parameter int N = 4) (input logic clk, rst_n, output logic clk_out);   // divide by N (even)

logic [$clog2(N)-1:0] cnt;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin cnt <= '0; clk_out <= 1'b0; end

else if (cnt == N/2-1) begin cnt <= '0; clk_out <= ~clk_out; end

else cnt <= cnt + 1'b1;

endmodule

Tip:  In real FPGA designs, use a clock-enable pulse instead of a divided clock when possible. It stays in the same clock domain.

## Q12.9  Button debouncer

D  Problem: Clean a mechanical button input that bounces.

P  Tests synchronization of an external input, plus counting to confirm stability.

W  First synchronize with 2 flops. Then count how long the synchronized input differs from the current output. Only when it stays different for N clocks, update the output.

E  Tested: a 3-cycle glitch was ignored; a stable press rose after N cycles; release fell after N cycles.

module debounce #(parameter int N = 16) (        // N = stable clocks required

input  logic clk, rst_n,

input  logic btn_in,                          // asynchronous, bouncy

output logic btn_out

);

logic s1, s2;

logic [$clog2(N+1)-1:0] cnt;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin s1 <= 1'b0; s2 <= 1'b0; end

else        begin s1 <= btn_in; s2 <= s1; end   // 2-FF synchronizer

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin cnt <= '0; btn_out <= 1'b0; end

else if (s2 == btn_out) cnt <= '0;              // no change requested

else if (cnt == N-1) begin btn_out <= s2; cnt <= '0; end

else cnt <= cnt + 1'b1;                         // input differs: count

endmodule

Interview line:  “I synchronize first, then require the new value to be stable for N clocks before I accept it.”

## Q12.10  Fixed-priority encoder

D  Problem: Given N request bits, output the index of the lowest set bit and a valid flag.

P  Tests combinational loops and defaults.

W  Loop from bit 0 upwards; take the first set bit; keep a default so no latch is created.

E  Tested: for the pattern 8'b00101000 the index was 3; with no request, valid was 0.

module prio_enc #(parameter int N = 8) (

input  logic [N-1:0]         req,

output logic                 valid,

output logic [$clog2(N)-1:0] idx             // lowest set bit wins

);

always_comb begin

valid = 1'b0;

idx   = '0;

for (int i = 0; i < N; i++)

if (req[i] && !valid) begin valid = 1'b1; idx = i[$clog2(N)-1:0]; end

end

endmodule

Interview line:  “I assign defaults first and loop through the bits, so the result is a clean combinational priority encoder.”

## Q12.11  Round-robin arbiter   MUST KNOW

D  Problem: Grant one of N requesters, rotating the priority so that no requester starves.

P  A standard design question about fairness.

W  Remember the index granted last. Next time, start searching from the following index, and wrap around. Output is one-hot or all zero.

E  Tested: with all 4 requesters active for 40 cycles each got exactly 10 grants; the grant was always one-hot; a single requester was granted; no request gave no grant.

module rr_arbiter #(parameter int N = 4) (

input  logic         clk, rst_n,

input  logic [N-1:0] req,

output logic [N-1:0] grant                    // one-hot or zero

);

localparam int IW = (N > 1) ? $clog2(N) : 1;

logic [IW-1:0] last;                          // index granted last time

integer i, idx;

always_comb begin

grant = '0;

for (i = 1; i <= N; i = i + 1) begin

idx = (last + i) % N;                 // start after 'last'

if (req[idx] && (grant == '0)) grant[idx] = 1'b1;

end

end

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) last <= N-1;                  // so index 0 has first chance

else begin

for (int k = 0; k < N; k++)

if (grant[k]) last <= k[IW-1:0];

end

endmodule

Interview line:  “The arbiter remembers who won last and starts the search after that requester, so every requester gets a fair turn.”

Follow-up questions to expect

- Fixed priority vs round robin? — Fixed priority can starve low-priority requesters. Round robin is fair.

- Weighted round robin? — Give some requesters more turns, with a credit counter per requester.

## Q12.12  Valid/ready skid buffer (register slice)   ADVANCED

D  Problem: Add a register to a valid/ready link so that READY is driven from a flop, while keeping full throughput.

P  Tests understanding of backpressure, and breaking long combinational READY paths.

W  Two storage slots: the output register and a skid register. s_ready = !skid_valid (a flop output). If the output is stalled and a beat is accepted, the beat goes into the skid register. When the output moves, the skid word goes forward.

E  Tested: random VALID (70%) and READY (50%) for 3,700 beats: order preserved, no loss, output stable while stalled; with READY always high the throughput was one beat per cycle.

// Valid/ready register slice with full throughput.

// s_ready comes from a flop, so there is no combinational path m_ready -> s_ready.

module skid_buffer #(parameter int DW = 8) (

input  logic          clk, rst_n,

input  logic          s_valid,

output logic          s_ready,

input  logic [DW-1:0] s_data,

output logic          m_valid,

input  logic          m_ready,

output logic [DW-1:0] m_data

);

logic [DW-1:0] skid_data;

logic          skid_valid;

assign s_ready = ~skid_valid;

wire in_fire  = s_valid & s_ready;

wire out_fire = m_valid & m_ready;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin

m_valid <= 1'b0;

skid_valid <= 1'b0;

end else if (skid_valid) begin

if (out_fire) begin m_data <= skid_data; skid_valid <= 1'b0; end

end else if (in_fire) begin

if (!m_valid || out_fire) begin m_data <= s_data; m_valid <= 1'b1; end

else begin skid_data <= s_data; skid_valid <= 1'b1; end

end else if (out_fire) begin

m_valid <= 1'b0;

end

endmodule

Interview line:  “A skid buffer registers READY. It needs one extra storage slot, and it keeps one beat per cycle when the output is not stalled.”

Follow-up questions to expect

- What happens on the first stalled cycle? — The accepted beat goes to the skid register, and READY drops from the next cycle.

- Does the combinational path from m_ready to s_ready exist? — No. That is the purpose.

## Q12.13  Toggle pulse synchronizer   MUST KNOW

D  Problem: Safely pass a one-cycle pulse from a fast clock to a slow clock.

P  The classic pulse-CDC question.

W  Toggle a flop on each source pulse. Synchronize the toggle with two flops in the destination domain, and XOR it with its delayed copy to make a one-cycle pulse.

E  Tested: source clock 3 ns, destination clock 13 ns, 20 pulses spaced apart: 20 pulses came out.

module pulse_sync (

input  logic src_clk, src_rst_n, pulse_in,

input  logic dst_clk, dst_rst_n,

output logic pulse_out

);

logic tog;                                     // source domain

logic s1, s2, s3;                              // destination domain

always_ff @(posedge src_clk or negedge src_rst_n)

if (!src_rst_n)   tog <= 1'b0;

else if (pulse_in) tog <= ~tog;            // each event flips the level

always_ff @(posedge dst_clk or negedge dst_rst_n)

if (!dst_rst_n) {s3, s2, s1} <= 3'b000;

else            {s3, s2, s1} <= {s2, s1, tog};

assign pulse_out = s2 ^ s3;                    // edge detect on synced level

endmodule

Interview line:  “I convert the pulse into a level change, cross the level, and detect the change in the destination domain.”

## Q12.14  Reset synchronizer   MUST KNOW

D  Problem: Assert reset asynchronously but release it synchronously to a clock.

P  Tests understanding of recovery and removal and reset domain crossing.

W  Two flops with async clear and D tied to 1. The output is the second flop.

E  Tested: reset asserted immediately (no clock needed); after release, the output went high after two clock edges.

module reset_sync (

input  logic clk, arst_n,                      // async reset in

output logic rst_n                             // async assert, sync release

);

logic q1, q2;

always_ff @(posedge clk or negedge arst_n)

if (!arst_n) {q2, q1} <= 2'b00;

else         {q2, q1} <= {q1, 1'b1};

assign rst_n = q2;

endmodule

Interview line:  “Reset asserts immediately, but it releases only on a clock edge through two flops, so recovery and removal are safe.”

## Q12.15  LFSR (pseudo-random bit generator)   ADVANCED

D  Problem: Make an 8-bit LFSR with the maximum sequence length.

P  Used for test patterns, scramblers, and CRC-like logic.

W  Shift left and feed back an XOR of the tap bits. For x⁸ + x⁶ + x⁵ + x⁴ + 1 the taps are bits 7, 5, 4, 3. Reset to a non-zero value, because all-zero is a lock-up state.

E  Tested: the sequence length was exactly 255 states before repeating.

// 8-bit maximal-length LFSR, polynomial x^8 + x^6 + x^5 + x^4 + 1

module lfsr8 (

input  logic       clk, rst_n, en,

output logic [7:0] q

);

wire fb = q[7] ^ q[5] ^ q[4] ^ q[3];

always_ff @(posedge clk or negedge rst_n)

if (!rst_n)  q <= 8'h01;                   // never all-zero

else if (en) q <= {q[6:0], fb};

endmodule

Follow-up questions to expect

- Why not reset to zero? — With XOR feedback, all-zero stays all-zero forever.

- Fibonacci vs Galois? — Different XOR placement; same sequence length, Galois has shorter paths.

## Q12.16  Parity and population count

D  Problem: Output the parity of a word, and the number of ones in it.

P  Tests reduction operators and loops with the right widths.

W  Parity is the XOR reduction ^d. Count ones with a loop that adds each bit to a counter sized with $clog2(W+1).

E  Tested: all 256 byte values: parity and ones count matched the reference.

module bitops #(parameter int W = 8) (

input  logic [W-1:0]         d,

output logic                 parity,           // 1 if odd number of ones

output logic [$clog2(W+1)-1:0] ones

);

assign parity = ^d;

always_comb begin

ones = '0;

for (int i = 0; i < W; i++) ones = ones + d[i];

end

endmodule

## Q12.17  Packet generator with SOP, EOP and EMPTY   MUST KNOW

D  Problem: Generate a packet of PKT_BYTES on a BUS_BYTES-wide stream with valid/ready, SOP, EOP, and EMPTY.

P  This is the project question. Tests beat counting, the last-beat EMPTY, and backpressure handling.

W  beats = ceil(PKT/BUS). Last-beat bytes = PKT − (beats−1)·BUS. EMPTY = BUS − last-beat bytes. The beat counter advances only on valid && ready.

E  Tested: 8-byte bus, 20-byte packet, random READY (60%), 3 packets: 9 accepted beats, 3 SOP, 3 EOP, EMPTY = 4 on every last beat, and the outputs stayed stable while READY was low.

// Avalon-ST style packet generator: SOP / EOP / EMPTY for any packet size

module pkt_gen #(

parameter int BUS_BYTES = 8,                   // 64 for a 512-bit bus

parameter int PKT_BYTES = 20

) (

input  logic                          clk, rst_n,

input  logic                          start,

output logic [8*BUS_BYTES-1:0]        data,

output logic                          valid, sop, eop,

output logic [$clog2(BUS_BYTES)-1:0]  empty,

input  logic                          ready

);

localparam int NBEATS     = (PKT_BYTES + BUS_BYTES - 1) / BUS_BYTES;

localparam int LAST_BYTES = PKT_BYTES - (NBEATS-1)*BUS_BYTES;   // 1..BUS_BYTES

localparam int EMPTY_LAST = BUS_BYTES - LAST_BYTES;             // unused bytes

localparam int CW         = $clog2(NBEATS) + 1;

typedef enum logic [1:0] {IDLE, SEND, DONE} state_t;

state_t state;

logic [CW-1:0] beat;

wire last_beat = (beat == NBEATS-1);

wire fire      = valid & ready;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin state <= IDLE; beat <= '0; end

else case (state)

IDLE: if (start) state <= SEND;

SEND: if (fire) begin

if (last_beat) state <= DONE;

else           beat  <= beat + 1'b1;

end

DONE: begin state <= IDLE; beat <= '0; end

default: state <= IDLE;

endcase

assign valid = (state == SEND);

assign sop   = valid && (beat == 0);

assign eop   = valid && last_beat;

assign empty = eop ? EMPTY_LAST[$clog2(BUS_BYTES)-1:0] : '0;

assign data  = {BUS_BYTES{beat[7:0]}};

endmodule

Interview line:  “I compute the beat count and the last-beat EMPTY at elaboration time, and I advance the beat counter only on an accepted beat.”

Follow-up questions to expect

- What changes for a 512-bit bus? — BUS_BYTES = 64, EMPTY is 6 bits.

- What if PKT_BYTES is an exact multiple of the bus? — EMPTY on the last beat is 0.

## Q12.18  Pipeline with a valid bit

D  Problem: Make a 2-stage pipeline for y = a·b + c, and carry a valid flag with the data.

P  Tests pipelining and aligning the control with the data.

W  Register the product in stage 1. Delay c by one stage so it lines up. Delay valid by the same number of stages. Reset only the valid chain.

E  Tested: 20 random inputs came out in order with correct values and a valid flag delayed by exactly two cycles.

// 2-stage pipeline: y = a*b + c, valid travels with the data

module valid_pipe #(parameter int W = 8) (

input  logic           clk, rst_n,

input  logic           in_valid,

input  logic [W-1:0]   a, b, c,

output logic           out_valid,

output logic [2*W:0]   y

);

logic [2*W-1:0] prod;

logic [W-1:0]   c_d;

logic           v1;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) begin v1 <= 1'b0; out_valid <= 1'b0; end

else        begin v1 <= in_valid; out_valid <= v1; end

always_ff @(posedge clk) begin                 // data path: no reset needed

prod <= a * b;                             // stage 1

c_d  <= c;

y    <= prod + c_d;                        // stage 2

end

endmodule

Interview line:  “Every data stage has a matching valid stage, so the control always lines up with the data.”

## Q12.19  Block RAM (simple dual-port)   ADVANCED

D  Problem: Write a simple dual-port RAM that infers block RAM.

P  Tests inference style.

W  One clocked block: write when we; read into a register every cycle (synchronous read). No reset on the array.

E  Tested: wrote 0xA5 to address 5, then read it back two cycles later.

// Simple dual-port RAM, synchronous read -> infers block RAM

module sdp_ram #(parameter int DW = 8, parameter int AW = 6) (

input  logic          clk,

input  logic          we,

input  logic [AW-1:0] waddr, raddr,

input  logic [DW-1:0] wdata,

output logic [DW-1:0] rdata

);

logic [DW-1:0] mem [0:(1<<AW)-1];

always_ff @(posedge clk) begin

if (we) mem[waddr] <= wdata;

rdata <= mem[raddr];

end

endmodule

Follow-up questions to expect

- What if the read is combinational? — The tool uses LUT RAM instead of block RAM.

- Read-during-write at the same address? — Behavior depends on the RAM mode; specify “read first”, “write first”, or “no change”.

## Q12.20  One-shot pulse stretcher

D  Problem: Stretch a one-cycle pulse so that it stays high for N clock cycles.

P  Used for LED indicators and simple pulse CDC with a known clock ratio.

W  On a pulse, load a down-counter with N. Output = counter ≠ 0. Decrement every cycle.

E  Tested: for N = 5 the output was high for exactly 5 cycles.

module oneshot_stretch #(parameter int N = 8) (input logic clk, rst_n, pulse, output logic stretched);

logic [$clog2(N+1)-1:0] cnt;

always_ff @(posedge clk or negedge rst_n)

if (!rst_n) cnt <= '0; else if (pulse) cnt <= N; else if (cnt != 0) cnt <= cnt - 1'b1;

assign stretched = (cnt != 0);

endmodule

# PART 13 — DEBUG SCENARIOS (“WHAT WOULD YOU DO IF…?”)

3+ years interviews often give a symptom and ask for your method. For each scenario: say what you suspect first, how you would check it, and how you would fix it. Each scenario appears once. The answers point back to the concept questions instead of repeating them.

## Q13.1  Scenario: data is corrupted only now and then after crossing clock domains   MUST KNOW

D  A random, rare data error that depends on temperature, voltage, or clock phase.

P  This is the typical sign of a CDC problem, not a logic bug.

W  Suspect a multi-bit bus synchronized bit by bit, a pulse that is too short, or a missing synchronizer (Q7.10, Q7.7).

•  Run the CDC tool and list every crossing. Classify each one and check the technique (Q7.13).

•  Check constraints: is a false path or clock group hiding a bad crossing?

•  Fix with a handshake, enable-plus-stable-data, or an async FIFO.

E  A 16-bit config word crossing through 16 separate 2-FF synchronizers: some bits update one cycle early.

Interview line:  “Random errors that change with conditions point to a clock domain crossing. I list the crossings and check each one.”

## Q13.2  Scenario: the FIFO overflows occasionally   MUST KNOW

D  Data is lost now and then, even though a FIFO is in the path.

P  A lossless design must never overflow, so something in the flow control is wrong.

W  Check whether READY is driven from full instead of almost-full, so in-flight beats arrive after READY drops (Q3.2).

•  Check the depth against the burst (Q4.3).

•  In an async FIFO, remember that the flags are late by a few cycles (Q7.15).

•  Add the overflow assertion and run the test with random READY (Q4.1).

E  READY depends on full, but the producer has a pipeline stage: one extra beat arrives after the FIFO is full.

Interview line:  “First I check how READY is generated. Almost-full with enough margin for the beats in flight usually solves it.”

## Q13.3  Scenario: the design works in simulation but fails on the board   MUST KNOW

D  Simulation passes, hardware misbehaves.

P  There are many possible causes, so a method is more important than a guess.

W  Check constraints first: is every clock defined? Are I/O delays set? Is timing really met? (check_timing, report_timing_summary)

•  Check CDC crossings and reset release (the board has real clocks, real jitter, real temperature).

•  Check simulation versus synthesis mismatch: latches, uninitialized registers, x assumptions, wrong sensitivity lists (Q8.1).

•  Check external interface timing and pin assignments (I/O standard and pin location).

•  Use an on-chip logic analyzer (ILA) to see the first wrong signal.

E  A register without reset starts as x in simulation (and then is “fixed” by the testbench), but powers up random on the board.

Interview line:  “I start with constraints and clocks, then CDC and reset, then simulation/synthesis mismatch, and I use an ILA to find the first wrong signal.”

## Q13.4  Scenario: timing fails at the target frequency   MUST KNOW

D  The design fails setup at, for example, 250 MHz.

P  A structured method avoids random changes.

W  Confirm that the constraints are right (Q9.19).

•  Read the worst path (Q9.18): is it logic depth, fanout, net delay, or clocking?

•  Fix the cause: pipeline, duplicate the high-fanout register, register READY with a skid buffer (Q12.12), or simplify the logic.

•  Re-run and check that hold did not get worse.

E  A 521-bit write enable driven by one almost_full compare: duplicate or register the flag.

Interview line:  “I read the worst path, find the largest delay, and fix that cause. I do not lower the clock first.”

## Q13.5  Scenario: the packet loses its last beat, or EOP never appears

D  The Slave never completes a packet, or counts one beat too few.

P  This is usually an off-by-one in counters or in FIFO read latency.

W  Check the beat counter compare: beat == NBEATS-1 (not NBEATS).

•  Check that counters advance only on valid && ready (Q3.1).

•  Check FIFO read latency: data arrives one cycle after rd_en (Q4.7).

•  Check that SOP, EOP, and EMPTY are stored in the same FIFO word as their data (Q4.11).

E  EOP appears one cycle early: the compare used the next beat count.

Interview line:  “I trace the first wrong beat in the waveform, then check the counter compare and the FIFO read latency.”

## Q13.6  Scenario: the design hangs and never moves data (deadlock)

D  VALID and READY both stay low, or an FSM never leaves a state.

P  Usually a protocol or FSM dependency problem.

W  Draw the handshake: does VALID wait for READY, while READY waits for VALID?

•  Check FSM exits: does each state have a path out? Is there a missing default?

•  Check reset: is the FSM starting from a known state, and are the FIFO pointers reset together?

•  Add a watchdog timer and an assertion “VALID is followed by READY within N cycles”.

E  The slave waits for VALID before READY, and the master waits for READY before VALID.

Interview line:  “I check the handshake dependencies first, then each FSM exit, then reset.”

## Q13.7  Scenario: hold violations appear after routing   ADVANCED

D  Setup is fine, but hold fails on many paths.

P  Hold problems usually come from clock skew or short paths.

W  Look at the failing paths: are they direct flop-to-flop paths with clock skew?

•  Check for a wrong clock constraint, a gated or derived clock, or a clock crossing treated as synchronous.

•  Let the tool add delay. If the violation is large, fix the clocking, not the data path.

•  Do not lower the clock frequency: it does not change hold (Q9.5).

E  A divided clock built from fabric logic: skew between the divided and source clock domains.

Interview line:  “Large hold violations are usually a clocking or constraint issue, so I check the clock structure before I add delay.”

## Q13.8  Scenario: outputs show X after reset

D  In simulation, control signals stay unknown after reset.

P  Unknowns spread to the outputs and hide real behavior.

W  Find the first signal that is x and trace back to its driver.

•  Check that all control registers and FSM states have a reset.

•  Check for undriven inputs and unconnected ports (W287a, W123 in Q8.1).

•  Add !$isunknown() assertions on interface signals after reset.

E  An FSM state register without reset stays x until the first assignment.

Interview line:  “I find the first x, trace it back to the driver, and reset every control register.”

## Q13.9  Scenario: throughput is lower than expected   ADVANCED

D  The stream moves less than one beat per cycle even when the sink is always ready.

P  It is usually a bubble in the control or an unnecessary stall.

W  Measure: beats accepted / total cycles (Q4.9).

•  Look for a bubble state in an FSM, such as an IDLE or DONE cycle between beats or packets.

•  Check whether READY is registered without a skid buffer, which halves throughput (Q12.12).

•  Check FIFO almost-full thresholds. Too small a margin keeps READY low.

E  A registered READY without a skid buffer gives 50% throughput.

Interview line:  “I count accepted beats against cycles, then look for bubbles in the FSMs and in the READY path.”

## Q13.10  Scenario: lint shows a multiple-driver warning but simulation passes

D  The simulation result looks correct, but lint reports W415.

P  A multiple-driver error can be hidden by the order of events in simulation.

W  Find both drivers of the signal.

•  Decide which block owns it, and remove or merge the other.

•  Remember that simulation may show the last assignment only, while synthesis can fail or create a conflict (Q8.1).

E  Two always blocks both assign ready.

Interview line:  “Passing simulation does not remove a multiple-driver problem. I merge the drivers into one owner.”

## Q13.11  Scenario: a metastability failure is suspected after a clock frequency change   ADVANCED

D  The design worked at one clock rate and fails rarely at a higher rate.

P  MTBF drops quickly when the clock and data rates go up.

W  Check all synchronizers: two flops, no logic between them, and the right attributes (Q7.4).

•  Compute MTBF for the new rate (Q7.2). Consider a third flop.

•  Check that no signal is crossing without a synchronizer, because a design may have worked by luck at a lower rate.

E  A signal that crossed without a synchronizer at 50 MHz and is now seen failing at 200 MHz.

Interview line:  “Higher clock rates reduce MTBF, so I re-check all synchronizers and add a third flop where needed.”

# PART 14 — RTL DESIGN THINKING

When an interviewer says “design a block”, do not start with code. Follow these steps out loud. This shows a 3+ years mindset.

| Step | Think about | What you say |
| --- | --- | --- |
| 1 | Interface | “What are the inputs and outputs? Is there a valid/ready handshake? What are the widths?” |
| 2 | Clock and reset | “How many clock domains? Sync or async reset? Any CDC?” |
| 3 | Behavior | “What is the function? What are the corner cases: reset, back-to-back, full, empty, stall?” |
| 4 | Rate and latency | “What throughput do I need? How many cycles of latency are allowed?” |
| 5 | Architecture | “Data path and control path. Which blocks: FSM, counters, FIFO, pipeline stages?” |
| 6 | State and storage | “What must be stored? Which registers need reset?” |
| 7 | Combinational or sequential | “A decision with no memory is combinational. Anything that needs memory or timing is sequential.” |
| 8 | Widths | “Counter width = $clog2(max+1). Check overflow and signed/unsigned.” |
| 9 | Timing risk | “Long combinational path? Add a pipeline stage. High fanout? Duplicate.” |
| 10 | Verification plan | “Directed tests, random READY, assertions, corner cases.” |

## Q14.1  How do you approach a new RTL design?   MUST KNOW

D  I first turn the requirement into interface, data-path, and control requirements.

P  This prevents coding before the architecture is understood.

W  Use the ten steps above. Identify clocks, resets, inputs, outputs, state, storage, handshake, CDC boundaries, and corner cases. Draw a block diagram and a state diagram. Then write RTL, then lint, simulate, and check synthesis and timing.

E  For a packet generator: define packet size, bus width, beat count, SOP/EOP, and ready/valid behavior before coding.

Interview line:  “I design from the requirement and the timing behavior first, then write RTL. The flow is requirement, architecture, RTL, simulation, lint, synthesis, timing, debug.”

## Q14.2  How do you decide whether logic should be combinational or sequential?

D  Combinational logic has no memory. Sequential logic remembers past values and uses a clock.

P  The wrong choice gives wrong behavior, latches, or timing trouble.

W  If the output depends only on current inputs, use combinational logic. If it must remember something, count, or wait for an event, use sequential logic. If a combinational path becomes too long, insert a register (pipeline).

E  A mux is combinational. A counter and an FSM state are sequential.

Interview line:  “If the answer needs memory or a number of cycles, it is sequential. Otherwise it is combinational.”

## Q14.3  How do you handle an ambiguous or unclear requirement?

D  An unclear requirement is one where the interface, timing, or corner behavior is not fully defined.

P  Wrong assumptions cause rework and bugs.

W  Ask short, specific questions. If you cannot ask, state your assumption clearly, choose the safe design, and make it easy to change with a parameter.

E  “I assume VALID does not depend on READY. If it does, I can add a skid buffer.”

Interview line:  “When the requirement is not clear, I state my assumptions first, then design for the safe case.”

## Q14.4  How would you answer “design a FIFO” on a whiteboard?   MUST KNOW

D  A FIFO question is really a test of how you structure a design, not just code.

P  It covers pointers, flags, latency, and CDC in one question.

W  Ask: depth, width, one clock or two, FWFT or standard, almost-full, reset type.

•  Draw: memory array, write pointer, read pointer, full and empty logic.

•  Say: extra pointer bit; block write when full and read when empty; read latency.

•  If two clocks: Gray pointers, 2-FF synchronizers, full in the write domain, empty in the read domain.

•  Mention tests: random read/write with a scoreboard, full and empty corner cases, assertions.

E  See the tested code in Q12.5 and the sync FIFO problem in Part 12.

Interview line:  “I start by asking about clocks, depth, and read behavior, because those change the whole design.”

## Q14.5  How would you design a width converter (for example 32-bit to 64-bit) with valid/ready?   ADVANCED

D  A width converter packs narrow beats into wide beats (upsizer), or splits wide beats into narrow ones (downsizer).

P  It is a common building block between blocks with different bus widths.

W  Upsizer 32→64: keep a 1-bit phase and a 32-bit holding register.

•  Phase 0: accept a beat into the holding register. Phase 1: accept the next beat and present the combined 64-bit word with VALID.

•  Hold the 64-bit word while the output READY is low, and stop taking input meanwhile (backpressure).

•  On TLAST in phase 1 (odd number of beats), present a partial word and mark the unused bytes (TKEEP/EMPTY).

E  3 input beats of 32 bits produce 2 output beats: the second one is partial with EMPTY = 4 bytes.

Interview line:  “The converter is a small FSM with a holding register. It must keep valid/ready rules, handle backpressure, and flag the partial last word.”

Follow-up questions to expect

- Downsizer? — Hold the wide word and send it in slices. READY back to the input only when the last slice is accepted.

## Q14.6  What is your RTL review checklist before you hand the code over?

D  It is a final self-review list.

P  Many bugs are visible in a calm read of the code, before any tool runs.

W  Clocks and resets: one clock per block, correct reset type and release.

•  Every always_comb has defaults. Every case has a default.

•  One driver per signal. Non-blocking in clocked blocks.

•  Widths: counters, buses, comparisons, and carry bits.

•  Handshake: stable data while VALID && !READY; counters advance only on a real transfer.

•  CDC: every crossing is identified and has the right structure.

•  Lint is clean (or every waiver has a reason). Synthesis has no unexpected latch. Timing meets, with margin.

•  Tests: random READY, corner cases, assertions, and coverage.

E  Run the list before opening a pull request.

Interview line:  “Before I hand over RTL I check clocks and resets, defaults, drivers, widths, the handshake, CDC, lint, and tests.”

# PART 15 — REVISION PAGES

This part has no new questions. It collects what you want to see the night before the interview.

## The 30-second answer template

“D: It is ___. P: We use it to ___. W: It works by ___, and the important things are ___. E: For example, in my project ___.”

Interview line:  “In my design, the master sends packets through a valid-ready handshake. A FIFO absorbs rate differences and drives backpressure through almost-full. The slave counts packets and bytes. I verified it with random READY and a scoreboard.”

## How to answer a follow-up

- Pause for two seconds. Silence is fine.

- Answer the question that was asked. Do not start a new topic.

- Give the reason (“because…”) and the risk (“otherwise…”).

- Offer one example from your project.

- If you are not sure, say what you know, and how you would check the rest.

## Say this / avoid this

| Avoid | Say instead |
| --- | --- |
| “The signal stops the sender.” | “READY goes low, so the source holds the beat. That is backpressure.” |
| “We use two flops to remove metastability.” | “Two flops reduce the chance of metastability reaching the logic. They do not remove it.” |
| “I added a false path because timing failed.” | “I proved the path is not functionally timed, then I added a narrow, documented exception.” |
| “Simulation passed, so it is fine.” | “It passed simulation. I also checked lint, synthesis warnings, CDC, and timing.” |
| “Lint was just warnings.” | “I reviewed each warning by risk, fixed the real bugs, and waived the rest with reasons.” |
| “I used a FIFO for the bus crossing.” | “I used an asynchronous FIFO with Gray-coded pointers because the clocks are asynchronous.” |
| “The design is fast.” | “It meets 250 MHz with +0.4 ns slack after pipelining the READY path.” |

## Formulas and numbers to remember

| Topic | Formula / fact |
| --- | --- |
| Frequency and period | f = 1 / T. 10 ns = 100 MHz. 5 ns = 200 MHz. |
| Counter width | $clog2(MAX + 1) bits for values 0 to MAX. |
| FIFO address bits | AW = $clog2(DEPTH). Pointer width = AW + 1. |
| FIFO full / empty | Empty: pointers equal. Full: same address, different wrap bit. |
| Async FIFO full (Gray) | wgray_next == {~rq2_rgray[AW:AW-1], rq2_rgray[AW-2:0]} |
| Binary → Gray | g = b ^ (b >> 1) |
| Bus beats | Bytes per beat = width / 8. Beats = ceil(packet bytes / bytes per beat). |
| EMPTY on the last beat | BUS_BYTES − (PKT − (beats − 1) × BUS_BYTES) |
| Peak throughput | Bus bytes × clock. 64 B × 250 MHz = 16 GB/s. |
| FIFO depth (rough) | burst × (1 − read_rate / write_rate) + margin, then round up. |
| Setup check | T ≥ t_clk2q + t_logic + t_setup + skew + uncertainty |
| Hold check | arrival(min) ≥ capture clock + t_hold. Does not depend on T. |
| Slack | Setup: required − arrival. Hold: arrival − required. |
| Fmax | 1 / (T − WNS) for a positive WNS on a reg-to-reg path. |
| Multicycle | Setup N, hold N − 1. |
| Synchronizer | 2 flops (3 at very high speed). No logic between them. |
| MTBF | e^(t_r / τ) / (T₀ · f_clk · f_data) |

## Memory map — one page

- RTL basics → combinational, sequential, register, latch, width, signedness, reset.

- Control → FSM, state, next state, Moore, Mealy, default recovery.

- Data movement → ready, valid, handshake, backpressure, skid buffer, FIFO, pipeline.

- Streaming → AXI-Stream, Avalon-ST, beat, packet, SOP, EOP, TLAST, EMPTY, TKEEP.

- CDC → metastability, synchronizer, pulse, toggle, handshake, Gray code, async FIFO, reset sync.

- Timing → clock, setup, hold, skew, jitter, slack, critical path, corners, timing closure.

- Constraints → XDC, create_clock, input/output delay, false path, multicycle, clock groups.

- Quality → lint, assertions, testbench, scoreboard, coverage, corner cases, sim/synth checks.

- Performance → latency, throughput, pipelining, Fmax, PPA.

- Design thinking → requirement → architecture → RTL → simulation → lint → synthesis → timing → debug.

## Final memory formula

Define it → Say why → Explain how (with RTL words) → Give one example → Mention the risk.

Three lines to remember: (1) A handshake moves data on VALID && READY. (2) Cross clock domains with a synchronizer for one bit, a handshake for control, and an async FIFO with Gray pointers for data. (3) Check timing with setup, hold, and slack, and keep constraints honest.

Tip:  Do not memorize every sentence. Memorize the keywords and the D → P → W → E flow, and explain in your own simple English. Study each question only once, then revise it using the checklist in the next part.

# PART 16 — PROGRESS CHECKLIST (EVERY QUESTION ONCE)

220 questions. Tick each one when you can answer it in D → P → W → E without looking. Revise only the unticked ones.

Part 2 — CORE RTL FUNDAMENTALS

☐  Q2.1  What is RTL design?

☐  Q2.2  What is combinational logic?

☐  Q2.3  What is sequential logic?

☐  Q2.4  What is a flip-flop, and what is a register?

☐  Q2.5  What is a latch, and why is an unintended latch a problem?

☐  Q2.6  Blocking versus non-blocking assignment?

☐  Q2.7  What is the difference between wire, reg, and logic, and between always, always_ff, always_comb, and always_latch?

☐  Q2.8  What are synchronous and asynchronous resets?

☐  Q2.9  What is a parameter, and what is the difference between parameter and localparam?

☐  Q2.10  What is a generate block?

☐  Q2.11  Why are signal widths important?

☐  Q2.12  Signed versus unsigned data?

☐  Q2.13  case, casez, casex, unique case, and priority case — what is the difference?

☐  Q2.14  What is synthesis?

☐  Q2.15  What is simulation versus synthesis mismatch?

☐  Q2.16  What is a race condition in RTL simulation?

☐  Q2.17  What is X propagation, and how do you handle unknown values?

☐  Q2.18  What is inference, and how do you infer RAM, DSP, and registers?

☐  Q2.19  What is a combinational loop?

☐  Q2.20  What are glitches and hazards?

☐  Q2.21  What is clock gating, and how is it done safely?

Part 3 — READY/VALID HANDSHAKE AND FSM

☐  Q3.1  What is the ready/valid handshake?

☐  Q3.2  What is backpressure?

☐  Q3.3  How does backpressure travel through a pipeline, and what is a skid buffer?

☐  Q3.4  How do you avoid deadlock in a handshake design?

☐  Q3.5  What is an FSM (Finite State Machine)?

☐  Q3.6  Moore versus Mealy FSM?

☐  Q3.7  How do you choose FSM state encoding — binary, one-hot, or Gray?

☐  Q3.8  Why use default assignments in FSM combinational logic?

☐  Q3.9  What is illegal-state recovery, and why include a default state?

☐  Q3.10  Should FSM outputs be registered?

Part 4 — FIFO, PIPELINE, PERFORMANCE

☐  Q4.1  What is a FIFO?

☐  Q4.2  What is the difference between synchronous and asynchronous FIFO?

☐  Q4.3  What is FIFO depth, and how do you calculate it?

☐  Q4.4  What is FIFO width?

☐  Q4.5  What are FIFO overflow and underflow, and how do you prevent them?

☐  Q4.6  What are almost-full and almost-empty?

☐  Q4.7  What is a FWFT FIFO versus a standard FIFO, and what is read latency?

☐  Q4.8  What is pipelining?

☐  Q4.9  Latency versus throughput?

☐  Q4.10  How do you calculate counter width and bytes per beat?

☐  Q4.11  What is packing and unpacking?

☐  Q4.12  What is PPA?

☐  Q4.13  How do you improve Fmax, reduce area, and reduce power in RTL?

Part 5 — AXI-STREAM AND AXI

☐  Q5.1  What is AXI-Stream and how does it work?

☐  Q5.2  What are the AXI-Stream signals: TDATA, TKEEP, TSTRB, TLAST, TUSER, TID, TDEST?

☐  Q5.3  Why do we need TLAST, and what does it mean when TVALID, TREADY, and TLAST are all 1?

☐  Q5.4  What is the difference between a data beat and a packet?

☐  Q5.5  Explain the complete data flow in your AXI-Stream design.

☐  Q5.6  What are the five channels of AXI4 (memory-mapped)?

☐  Q5.7  AXI4 vs AXI4-Lite vs AXI-Stream — what is the difference?

☐  Q5.8  What are AXI bursts, outstanding transactions, and IDs?

Part 6 — AVALON-ST PROJECT (MASTER → FIFO → SLAVE)

☐  Q6.1  What is Avalon-ST?

☐  Q6.2  What are the Avalon-ST signals used in your design?

☐  Q6.3  Explain the SOP / EOP combinations.

☐  Q6.4  What is EMPTY and why is it needed?

☐  Q6.5  What is the objective of your project, and how do you explain it in one minute?

☐  Q6.6  What is the architecture of your design, and what are the main modules?

☐  Q6.7  Explain your Master FSM.

☐  Q6.8  How does READY affect the Master, and where does backpressure enter your design?

☐  Q6.9  What are the Master counters used for?

☐  Q6.10  Why do you use a FIFO in the design?

☐  Q6.11  Why use a FIFO IP instead of designing a FIFO manually?

☐  Q6.12  What are the roles of the FIFO write wrapper and read wrapper?

☐  Q6.13  Why is the FIFO word 521 bits wide, and what is the packing format?

☐  Q6.14  What happens when the FIFO is almost full, full, or empty?

☐  Q6.15  What is the purpose of the FIFO read FSM (egress control)?

☐  Q6.16  Explain your Slave FSM and how it counts bytes and packets.

☐  Q6.17  Explain the end-to-end packet flow in your project.

☐  Q6.18  How do you verify the design, and which waveforms do you check?

☐  Q6.19  What are the advantages of your design, and why is it modular?

☐  Q6.20  Where are Avalon-ST streaming architectures used?

☐  Q6.21  Explain your AVST packet generator from input to output.

☐  Q6.22  Why did you use an FSM in the packet generator?

☐  Q6.23  Why might a dual-FIFO architecture be useful for separate read and write paths?

☐  Q6.24  Explain your AXI-Stream receiver or packet path.

☐  Q6.25  What lint issues did you find in the project, and how did you fix them?

☐  Q6.26  Explain one timing issue in your design and how you would debug it.

☐  Q6.27  Explain one CDC boundary in your design and the technique used there.

Part 7 — CLOCK DOMAIN CROSSING (CDC) AND RESET CROSSING

☐  Q7.1  What is CDC — Clock Domain Crossing?

☐  Q7.2  What is metastability?

☐  Q7.3  What is MTBF, and what improves it?

☐  Q7.4  What is a two-flop synchronizer?

☐  Q7.5  Why do we mark synchronizer registers with ASYNC_REG?

☐  Q7.6  What is a single-bit level synchronizer, and can a 2-FF synchronizer carry a pulse?

☐  Q7.7  What is a toggle synchronizer (pulse CDC)?

☐  Q7.8  What is the limitation of a toggle synchronizer, and what is pulse stretching?

☐  Q7.9  What is a handshake synchronizer (req/ack CDC)?

☐  Q7.10  What is multi-bit CDC, and why is a 2-FF synchronizer not enough for a bus?

☐  Q7.11  What is a bundled-data CDC technique?

☐  Q7.12  What is a reconvergence CDC problem?

☐  Q7.13  How do you choose a CDC technique?

☐  Q7.14  What is the difference between pulse, toggle, and handshake synchronizers?

☐  Q7.15  Why do we use an asynchronous FIFO for CDC?

☐  Q7.16  What is a binary pointer in a FIFO, and how are the write and read pointers used?

☐  Q7.17  What is Gray code, and why is it used for async FIFO pointers?

☐  Q7.18  Why should we not directly synchronize a binary FIFO pointer?

☐  Q7.19  Why is an extra pointer bit used in an asynchronous FIFO?

☐  Q7.20  How is FIFO empty detected in an asynchronous FIFO?

☐  Q7.21  How is FIFO full detected in an asynchronous FIFO?

☐  Q7.22  Why are the upper two Gray-pointer bits inverted for full detection?

☐  Q7.23  Why are async FIFO flags pessimistic (pointer synchronization latency)?

☐  Q7.24  What happens if the write clock is faster than the read clock, and the other way round?

☐  Q7.25  How do you size an asynchronous FIFO?

☐  Q7.26  What is Reset Domain Crossing (RDC), and why synchronize reset deassertion?

☐  Q7.27  What are recovery and removal checks?

☐  Q7.28  What is CDC structural verification, and how do you verify CDC?

☐  Q7.29  How do you constrain CDC paths in Vivado?

☐  Q7.30  How do you implement CDC in Vivado using the built-in macros?

☐  Q7.31  What are the most common CDC mistakes?

☐  Q7.32  How would you debug a CDC issue in an interview?

Part 8 — RTL LINT

☐  Q8.1  What is RTL linting, and why do we use it?

☐  Q8.2  Is lint the same as simulation or synthesis?

☐  Q8.3  What is the typical lint workflow, and should every warning be fixed?

☐  Q8.4  W336 — Blocking assignment in a sequential block

☐  Q8.5  W18 — Inferred latch

☐  Q8.6  W415 — Multiple drivers

☐  Q8.7  W415a — Multiple assignments in one always block

☐  Q8.8  W120 — Declared but never used

☐  Q8.9  W528 — Set but never read

☐  Q8.10  W123 — Read but never set

☐  Q8.11  W240 — Input port never read

☐  Q8.12  W241 — Output port not completely set

☐  Q8.13  W110 — Module port width mismatch

☐  Q8.14  W164 — Assignment width mismatch

☐  Q8.15  W362 — Unequal widths in a comparison

☐  Q8.16  W484 — Possible loss of carry or borrow

☐  Q8.17  W486 — Shift overflow

☐  Q8.18  W263 — Case item width mismatch

☐  Q8.19  W551 — Case without default (intent not complete)

☐  Q8.20  W122 / W456 — Sensitivity list problems

☐  Q8.21  W210 — Unconnected module port

☐  Q8.22  W287a — Instance input not driven

☐  Q8.23  W146 — Positional port connection

☐  Q8.24  W156 — Reverse-connected bus

☐  Q8.25  W497 / W498 — Bus not fully set / not fully read

☐  Q8.26  W446 — Output port read inside its own module

☐  Q8.27  W502 — Modifying a signal in the sensitivity list

☐  Q8.28  Why are width and signedness issues especially important in counters and streaming buses?

☐  Q8.29  Why are unused or undriven signals important in an AXI-Stream or Avalon-ST design?

☐  Q8.30  How would you debug a lint violation in an interview?

☐  Q8.31  What do lint-clean RTL patterns look like?

☐  Q8.32  What is the lint checklist for an AXI-Stream or Avalon-ST design?

Part 9 — STATIC TIMING ANALYSIS (STA)

☐  Q9.1  What is STA, and why do we use it?

☐  Q9.2  STA vs simulation vs synthesis — what is the difference?

☐  Q9.3  What is a timing path? (startpoint, endpoint, launch and capture, data path and clock path)

☐  Q9.4  What are setup time and the setup check?

☐  Q9.5  What are hold time and the hold check?

☐  Q9.6  What are arrival time, required time, and slack?

☐  Q9.7  What is a critical path, and what is maximum-delay versus minimum-delay analysis?

☐  Q9.8  How do you compute Fmax from slack?

☐  Q9.9  Worked example: how do you calculate setup and hold slack?

☐  Q9.10  What is clock skew, and what are positive and negative skew?

☐  Q9.11  What are clock uncertainty and jitter?

☐  Q9.12  What are clock latency, source latency, network latency, and propagation delay?

☐  Q9.13  What are PVT corners, and what are the slow and fast corners?

☐  Q9.14  What are OCV and derating?

☐  Q9.15  What are clock gating checks?

☐  Q9.16  What are max transition, max capacitance, and fanout?

☐  Q9.17  What are timing libraries and cell arcs?

☐  Q9.18  How do you read a timing report?

☐  Q9.19  How do you fix setup violations, and how do you fix hold violations?

☐  Q9.20  What is your STA debug checklist?

Part 10 — XDC CONSTRAINTS (FPGA / VIVADO)

☐  Q10.1  What is XDC, and why do we need constraints?

☐  Q10.2  What is the XDC file structure, and what are objects and collections?

☐  Q10.3  How does create_clock work? (period, waveform, and the missing-clock mistake)

☐  Q10.4  What are generated clocks and clock groups?

☐  Q10.5  What are set_clock_uncertainty, set_clock_latency, and clock transition?

☐  Q10.6  What are set_input_delay and set_output_delay?

☐  Q10.7  What are false paths and multicycle paths, and how do you write them?

☐  Q10.8  What are set_max_delay and set_min_delay?

☐  Q10.9  How do I constrain pins: PACKAGE_PIN, IOSTANDARD, DRIVE, SLEW, PULLUP?

☐  Q10.10  What are LOC and physical placement constraints?

☐  Q10.11  What are common XDC mistakes?

☐  Q10.12  What does a complete basic FPGA constraint file look like?

☐  Q10.13  How do you debug XDC problems in Vivado, and which reports do you use?

Part 11 — VERIFICATION, TESTBENCH, ASSERTIONS AND DEBUG

☐  Q11.1  What is a testbench, and what is a directed test?

☐  Q11.2  What is a scoreboard?

☐  Q11.3  What are corner cases, and which ones matter for streaming and FIFO designs?

☐  Q11.4  What are assertions (SVA)?

☐  Q11.5  What is functional coverage?

☐  Q11.6  How do you debug an RTL problem?

☐  Q11.7  What is a good testbench structure for a valid/ready design?

Part 12 — RTL CODING PRACTICE (TESTED SOLUTIONS)

☐  Q12.1  Warm-up: flip-flop variants, mux, decoder, counter, shift register

☐  Q12.2  Rising / falling edge detector

☐  Q12.3  Binary to Gray and Gray to binary

☐  Q12.4  Synchronous FIFO

☐  Q12.5  Asynchronous FIFO (Gray pointers)

☐  Q12.6  Sequence detector (overlapping “1011”)

☐  Q12.7  Clock divider: even N and odd divide-by-3 with 50% duty

☐  Q12.8  Even clock divider (divide by N, N even)

☐  Q12.9  Button debouncer

☐  Q12.10  Fixed-priority encoder

☐  Q12.11  Round-robin arbiter

☐  Q12.12  Valid/ready skid buffer (register slice)

☐  Q12.13  Toggle pulse synchronizer

☐  Q12.14  Reset synchronizer

☐  Q12.15  LFSR (pseudo-random bit generator)

☐  Q12.16  Parity and population count

☐  Q12.17  Packet generator with SOP, EOP and EMPTY

☐  Q12.18  Pipeline with a valid bit

☐  Q12.19  Block RAM (simple dual-port)

☐  Q12.20  One-shot pulse stretcher

Part 13 — DEBUG SCENARIOS (“WHAT WOULD YOU DO IF…?”)

☐  Q13.1  Scenario: data is corrupted only now and then after crossing clock domains

☐  Q13.2  Scenario: the FIFO overflows occasionally

☐  Q13.3  Scenario: the design works in simulation but fails on the board

☐  Q13.4  Scenario: timing fails at the target frequency

☐  Q13.5  Scenario: the packet loses its last beat, or EOP never appears

☐  Q13.6  Scenario: the design hangs and never moves data (deadlock)

☐  Q13.7  Scenario: hold violations appear after routing

☐  Q13.8  Scenario: outputs show X after reset

☐  Q13.9  Scenario: throughput is lower than expected

☐  Q13.10  Scenario: lint shows a multiple-driver warning but simulation passes

☐  Q13.11  Scenario: a metastability failure is suspected after a clock frequency change

Part 14 — RTL DESIGN THINKING

☐  Q14.1  How do you approach a new RTL design?

☐  Q14.2  How do you decide whether logic should be combinational or sequential?

☐  Q14.3  How do you handle an ambiguous or unclear requirement?

☐  Q14.4  How would you answer “design a FIFO” on a whiteboard?

☐  Q14.5  How would you design a width converter (for example 32-bit to 64-bit) with valid/ready?

☐  Q14.6  What is your RTL review checklist before you hand the code over?